STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./226/2022-23/                                                          Dated: 09.09.2022

#### TEST REPORTS

Tested for 	:      	The DGM Engg.),

NBCC  (India) Ltd.,

CIHTS Works: Sarnath,Varanasi-221007

(M/s Yes Countrywide Construction OPC Private Limited)

Ref.		:    	NBCC/AGM/IIT-BHU/2022/175  Dated:  01.07.2022

Tax Invoice No. : 	593 IIT BHU dated 01.08.2022

Details of Sample :      Concrete Mix Design.

Test Required       	 M-25 grade with admixture

______________________________________________________________________________

Sub :  Construction of Boundary wall at Baraut Camping Ground, Prayagraj, U.P.- design Mix of concrete for

M-25 Grade.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489,  Y-2022.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.  Admixture used Sikament- 3070 NS.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./226/2022-23/                                                          Dated: 09.09.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 1.92 : 1.34 : 2.01 : 0.6* 0.65 : 1 : 1.50 : 1.13 : 1.58 : 0.6* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 31.6 |
| 2 | Slump range (mm) | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 130.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 24.5 34.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Sikament- 3070 NS (Litre) | 172.3 344.7 662.5 462.5 693.8 2.068 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./226/2022-23/                                                          Dated: 09.09.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade with

admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 92.0 | 100.0 | - |
|  | 10 mm | 0.40 | 88.65 | - |
|  | 4.75 mm | 0.05 | 0.95 | 2.8 |
|  | 2.36 mm | - | 0.35 | 6.8 |
|  | 1.18 mm | - | 0.10 | 8.8 |
|  | 600 micron | - | - | 12.4 |
|  | 300 micron | - | - | 88.2 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 7.076 | 6.100 | 2.178 |
| 3. | Bulk Density (Kg./Litre) | 1.65 | 1.54 | 1.67 |
| 4. | Specific Gravity | 2.60 | 2.58 | 2.64 |
| 5. | Water absorption | 1.97 % | 2.28 % | 1.63% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./226/2022-23/                                                          Dated: 09.09.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	The DGM Engg.),

NBCC  (India) Ltd.,

CIHTS Works: Sarnath,Varanasi-221007

(M/s Yes Countrywide Construction OPC Private Limited)

Ref.		:    	NBCC/AGM/IIT-BHU/2022/175  Dated:  01.07.2022

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489,  Y-2022.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with

admixture

______________________________________________________________________________

Sub :  Construction of Boundary wall at Baraut Camping Ground, Prayagraj, U.P.- design Mix of concrete for

M-25 Grade.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 140 Minutes 350 Minutes 24.8 N/mm2 34.4 N/mm2 44.4 N/mm2 1.30 2.90 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./229/2022-23/ 	                                                  Dated: 31.08.2022

#### TEST REPORTS

Tested for 	:      	M/s  DS Enterprise

Ballia  U.P.

Ref.		:    	DS/JCS/IIT/2022-23/263  dated 22.07.2022

Tax Invoice No. : 	601  IIT BHU dated 01.08.2022

Details of Sample :      	Vetting of the Concrete Mix Design

Test Required       	M-25 grade with OPC 43-grade cement with admixture

__________________________________________________________________________

Sub: Vetting of Mix design report

Work: Construction of Jannayak Chandrasekhar State University (Phase-I) of UPPWD on  EPC basis at Ballia, Uttar

Pradesh, India

#### REPORT

With reference to your above letter regarding verification of above mentioned mix design submitted by you, 7-days and 28-days compressive strength with slump are as under:

| Sl. No. | Test | M-25 grade with OPC 43-grade cement |
| --- | --- | --- |
| 1. | Slump Achieved in the Laboratory (mm) | 80.0 |
| 2 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 27.0 33.5 |

The vetted mix designs are enclosed herewith.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./230/2022-23/ 	                                                  Dated: 31.08.2022

#### TEST REPORTS

Tested for 	:      	M/s Archon Powerinfra India Private Limited

Azamgarh, U.P.

Ref.		:    	APIIL/ AZAM/IIT.22-23/45 dated 22.07.2022

Tax Invoice No. : 	602  IIT BHU dated 01.08.2022

Details of Sample :      	Vetting of the Concrete Mix Design

Test Required       	M-25 grade with OPC 43-grade cement with admixture

__________________________________________________________________________

Sub: Vetting of Mix design report

Work: Construction of Maharaja Suhel dev State University (Phase-1) of UPPWD on EPC basis at Azamgarh, Uttar

Pradesh, India

#### REPORT

With reference to your above letter regarding verification of above mentioned mix design submitted by you, 7-days and 28-days compressive strength with slump are as under:

| Sl. No. | Test | M-25 grade with OPC 43-grade cement with admixture |
| --- | --- | --- |
| 1. | Slump Achieved in the Laboratory (mm) | 80.0 |
| 2 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 27.5 34.0 |

The vetted mix designs are enclosed herewith.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./168/2022-23/                                                           Dated: 26.08.2022

#### TEST REPORTS

Tested for 	:      	M/s Safety Controls & Devices Pvt. Ltd.

C-43/28/1, Behind Skylark Building, Newal Kishore Road,

Hazratganj, Lucknow-226 001

Ref.		:    	Nil      Dated 02.05.2022

Tax Invoice No. : 	439 IIT BHU dated 30.06.2022

Details of Sample :      Concrete Mix Design.

Test Required       	 M-25 grade with admixture

__________________________________________________________________________

Sub :  Concrete Mix Design for 25 grade

Work: Construction of Homeopathic Medical College and 100 Bed Hospital at Pindra, Varanasi

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489, W-22,  Y-2022.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.  Admixture used Auramix 200.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./168/2022-23/                                                           Dated: 26.08.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.05 : 1.42 : 2.12 : 0.4* 0.66 : 1 : 1.52 : 1.18 : 1.61 : 0.4* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 31.6 |
| 2 | Slump range (mm) | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 120.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 25.0 33.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Auramix 200 (Litre) | 172.0 344.7 705.5 487.8 731.6 1.378 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./168/2022-23/                                                           Dated: 26.08.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade with

admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 91.65 | 100.0 | - |
|  | 10 mm | 0.90 | 89.05 | - |
|  | 4.75 mm | 0.05 | 5.90 | 3.0 |
|  | 2.36 mm | - | 0.40 | 14.4 |
|  | 1.18 mm | - | 0.10 | 38.8 |
|  | 600 micron | - | - | 69.4 |
|  | 300 micron | - | - | 91.8 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.074 | 6.046 | 3.164 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.57 | 1.77 |
| 4. | Specific Gravity | 2.83 | 2.80 | 2.66 |
| 5. | Water absorption | 0.20 % | 0.45 % | 1.10% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./168/2022-23/                                                           Dated: 26.08.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	M/s Safety Controls & Devices Pvt. Ltd.

C-43/28/1, Behind Skylark Building, Newal Kishore Road,

Hazratganj, Lucknow-226 001

Ref.		:    	Nil      Dated 02.05.2022

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489, W-22,  Y-2022.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with

admixture

______________________________________________________________________________

Sub :  Concrete Mix Design for 25 grade

Work: Construction of Homeopathic Medical College and 100 Bed Hospital at Pindra, Varanasi

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 37 % 1.5 mm Expansion 125 Minutes 350 Minutes 25.1 N/mm2 34.7 N/mm2 44.8 N/mm2 1.31 2.91 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./178/2022-23/                                                      Dated: 26.08.2022

#### TEST REPORTS

Tested for 	:      	M/s Kasana Builders Pvt. Ltd.

C-23, Sector-40, Noida-201303

Distt. Gautam Budh Nagar (U.P.)

Ref.		:    	KBPL/ARS/VARN/EPC/21-22   dated 22.06.2022

Tax Invoice No. : 	1180 IIT BHU dated 13.01.2022

Details of Sample :      Concrete Mix Design.

Test Required       	  (i) M-15 grade with admixture (ii) M-25 grade with admixture

(iii) M-30 grade with admixture

__________________________________________________________________________

Sub :  Concrete Mix  Design

Work:  Construction on EPC basis of Atal Residential School at Varanasi (U.P.), India.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (Crushed sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489, W-24, M-06, Y-2022.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.  Admixture used Sika Plastiment 2001 NS.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./178/2022-23/                                                      Dated: 26.08.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for (i) M-15 grade with admixture (ii) M-25 grade with admixture and

(iii) M-30 grade with admixture

| Grade of Concrete |  | Water : Cement : Crushed : CA-(10) : CA- (20) : Admixture Sand | Remark |
| --- | --- | --- | --- |
| 1. | M-15 Grade | 0.55 : 1 : 2.58 : 1.51 : 2.26 : 0.6* 0.72 : 1 : 1.86 : 1.25 : 1.69 : 0.6* | By weight By volume |
| 2. | M-25 Grade | 0.50 : 1 : 2.25 : 1.37 : 2.06 : 0.6* 0.65 : 1 : 1.63 : 1.13 : 1.54 : 0.6* | By weight By volume |
| 3. | M-30 Grade | 0.45 : 1 : 1.94 : 1.23 : 1.85 : 0.6* 0.59 : 1 : 1.40 : 1.02 : 1.38 : 0.6* | By weight By volume |

| Sl. No. | Test | M-15 grade with admixture | M-25 grade with admixture | M-30 grade with admixture |
| --- | --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 20.78 | 31.6 | 38.25 |
| 2 | Slump range (mm) | 100-150 | 100-150 | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 115.0 | 110.0 | 115.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 16.0 22.0 | 27.5 33.0 | 29.5 40.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Crushed Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Sika Plastiment 2001 NS (Litre) | 172.3 313.3 807.3 472.7 709.1 1.879 | 172.3 344.7 775.5 473.3 710.0 2.068 | 173.3 383.0 741.6 472.0 708.0 2.298 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./178/2022-23/                                                      Dated: 26.08.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design (i) M-15 grade with

admixture (ii) M-25 grade with admixture and (iii) M-30 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (Crushed sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.4 | 100.0 | - |
|  | 10 mm | 0.30 | 85.6 | - |
|  | 4.75 mm | 0.05 | 0.40 | 2.0 |
|  | 2.36 mm | - | 0.20 | 19.6 |
|  | 1.18 mm | - | 0.10 | 45.0 |
|  | 600 micron | - | - | 67.2 |
|  | 300 micron | - | - | 83.0 |
|  | 150 micron | - | - | 97.0 |
| 2. | Fineness Modulus | 7.133 | 6.137 | 3.138 |
| 3. | Bulk Density (Kg./Litre) | 1.74 | 1.57 | 1.80 |
| 4. | Specific Gravity | 2.84 | 2.81 | 2.78 |
| 5. | Water absorption | 0.35 % | 0.45 % | 1.10% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./178/2022-23/                                                      Dated: 26.08.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	M/s Kasana Builders Pvt. Ltd.

C-23, Sector-40, Noida-201303

Distt. Gautam Budh Nagar (U.P.)

Ref.		:    	KBPL/ARS/VARN/EPC/21-22   dated 22.06.2022

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489, W-24, M-06, Y-2022.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i) M-15 grade with

admixture (ii) M-25 grade with admixture and (iii) M-30 grade with admixture

______________________________________________________________________________

Sub :  Concrete Mix  Design

Work:  Construction on EPC basis of Atal Residential School at Varanasi (U.P.), India.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 37 % 1.5 mm Expansion 120 Minutes 345 Minutes 25.2 N/mm2 35.1 N/mm2 45.2 N/mm2 1.30 2.91 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./69/2022-23/ 	                                                           Dated: 22.08.2022

#### TEST REPORTS

Tested for 	:      	Assistant Engineer (Civil)

BHU Project Varanasi -3

CPWD, Varanasi

(M/s Gupta Construction Pvt. Ltd.)

Ref.		:    		23(157) /Lk0vfHk0/ch0,Pk0;w0ifj0ea0-3/2022/128  dated 04-04-2022

Tax Invoice No. : 	 155 IIT BHU  dated 02.05.2022

Details of Sample :      Concrete Mix Design.

Test Required       	 M-25 grade with plasticizer and M-25 grade without plasticizer

__________________________________________________________________________

Sub.: Design of cement concrete mix M-25 with plasticizer and without plasticizer.

Work: Establishment of research Dairy Farm at RGSC, Barkachha, (SH: Construction of building/shed for Animal Research Facility, Poultry Shed, Milk Utility Extension Unit and Gaysadan (Package-1) at RGSC, Barkachha, Mirzapur, U.P.)

Agency:   M/s Gupta Construction Pvt. Ltd.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489,   Y-2022.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.  Admixture used Fosroc Conplast SP 430 G8.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./69/2022-23/ 	                                                           Dated: 22.08.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade with plasticizer and M-25 grade without plasticizer

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.04 : 1.42 : 2.13 : 0.6* 0.65 : 1 : 1.51 : 1.18 : 1.59 : 0.6* | By weight By volume |
| 2. | M-25 Grade | 0.50 : 1 : 1.60 : 1.12 : 1.67 : - 0.65 : 1 : 1.18 : 0.93 : 1.25 : - | By weight By volume |

| Sl. No. | Test | M-25 grade with plasticizer | M-25 grade without plasticizer |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 31.6 | 31.6 |
| 2 | Slump range (mm) | 100-150 | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 125.0 | 120.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 26.0 35.5 | 25.5 34.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Fosroc Conplast SP 430 G8 (Litre) | 172.3 344.7 702.0 488.9 733.4 2.068 | 202.7 405.5 650.0 452.7 679.1 - |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./69/2022-23/ 	                                                           Dated: 22.08.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade with

plasticizer and M-25 grade without plasticizer

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.95 | 100.0 | - |
|  | 10 mm | 0.45 | 85.75 | - |
|  | 4.75 mm | 0.05 | 0.50 | 2.8 |
|  | 2.36 mm | - | 0.20 | 10.4 |
|  | 1.18 mm | - | 0.10 | 31.2 |
|  | 600 micron | - | - | 59.4 |
|  | 300 micron | - | - | 92.2 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.126 | 6.135 | 2.950 |
| 3. | Bulk Density (Kg./Litre) | 1.74 | 1.56 | 1.76 |
| 4. | Specific Gravity | 2.84 | 2.81 | 2.65 |
| 5. | Water absorption | 0.15 % | 0.35 % | 1.15% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./69/2022-23/ 	                                                           Dated: 22.08.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Assistant Engineer (Civil)

BHU Project Varanasi -3

CPWD, Varanasi

(M/s Gupta Construction Pvt. Ltd.)

Ref.		:    		23(157) /Lk0vfHk0/ch0,Pk0;w0ifj0ea0-3/2022/128  dated 04-04-2022

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489,   Y-2022.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with

plasticizer and M-25 grade without plasticizer

______________________________________________________________________________

Sub.: Design of cement concrete mix M-25 with plasticizer and without plasticizer.

Work: Establishment of research Dairy Farm at RGSC, Barkachha, (SH: Construction of building/shed for Animal Research Facility, Poultry Shed, Milk Utility Extension Unit and Gaysadan (Package-1) at RGSC, Barkachha, Mirzapur, U.P.)

Agency:   M/s Gupta Construction Pvt. Ltd.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 36 % 1.5 mm Expansion 120 Minutes 350 Minutes 24.7 N/mm2 34.5 N/mm2 44.7 N/mm2 1.30 2.90 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./39(ii)/2022-23/                                                             Dated: 22.08.2022

#### TEST REPORTS

Tested for 	:      	AGM (FQA)

NTPC Limited, FQA, NTPC – Barh

Distt. – Patna (Bihar)-803213

(M/s BHEL )

Ref.		:    		4400/FQA/2022(Civil)/AV/1518      dated  09.02.2022

Tax Invoice No. : 	104 IIT BHU dated 20.04.2022

Details of Sample :      Concrete Mix Design.

Test Required       	 (i) M-35 grade with admixture with OPC Cement for Chimney Shell & other component

of shell

______________________________________________________________________________

Sub :  Evaluation of Materials & Concrete Mix Design for Construction of FGD Package for Stage-I & II at Barh

Super Thermal Power Project, Barh, Patna (Bihar)

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was  Ultra Tech Cement, OPC 43-grade, IS : 269, W-51, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.  Admixture used Fosroc Auramix 400.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./39(ii)/2022-23/                                                             Dated: 22.08.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-35 grade with admixture with OPC Cement for Chimney Shell &

other component  of shell

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-35 grade | 0.40 : 1 : 1.77 : 1.05 : 1.58 : 0.40* 0.56 : 1 : 1.36 : 0.94 : 1.28 : 0.40* | By weight By volume |

| Sl. No. | Test | M-35 grade with admixture for Chimney Shell &other component of shell |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 45.4 |
| 2 | Slump range (mm) | 125-175 |
| 3 | Slump Achieved in the Laboratory (mm) | 160.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 33.5 47.0 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Fosroc Auramix 400 (Litre) | 172.0 430.0 761.0 452.0 679.0 1.72 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./39(ii)/2022-23/                                                             Dated: 22.08.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-35 grade with admixture

with OPC Cement for Chimney Shell & other component  of shell

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm (Source: Koderama Bihar) | CA—10 mm (Source: Koderama Bihar) | FA (sand) (Source: Sone River Bihar) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.75 | 100.0 | - |
|  | 10 mm | 0.40 | 89.25 | - |
|  | 4.75 mm | 0.05 | 7.90 | 3.6 |
|  | 2.36 mm | - | 0.40 | 14.8 |
|  | 1.18 mm | - | 0.05 | 42.8 |
|  | 600 micron | - | - | 77.6 |
|  | 300 micron | - | - | 97.6 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 7.128 | 6.024 | 3.352 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.56 | 1.82 |
| 4. | Specific Gravity | 2.87 | 2.78 | 2.74 |
| 5. | Water absorption | 0.15 % | 0.41 % | 0.95% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./39(ii)/2022-23/                                                             Dated: 22.08.2022

#### TEST REPORTS

Tested for 	:      	AGM (FQA)

NTPC Limited, FQA, NTPC – Barh

Distt. – Patna (Bihar)-803213

(M/s BHEL )

Ref.		:    		4400/FQA/2022(Civil)/AV/1518      dated  09.02.2022

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grade, IS : 269, W-51, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-35 grade with admixture

with OPC Cement for Chimney Shell & other component  of shell

______________________________________________________________________________

Sub :  Evaluation of Materials & Concrete Mix Design for Construction of FGD Package for Stage-I & II at Barh

Super Thermal Power Project, Barh, Patna (Bihar)

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 31 % 1.0 mm Expansion 95 Minutes 280 Minutes 26.0 N/mm2 36.3 N/mm2 46.7 N/mm2 1.40 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./39(i)/2022-23/                                                             Dated: 22.08.2022

#### TEST REPORTS

Tested for 	:      	AGM (FQA)

NTPC Limited, FQA, NTPC – Barh

Distt. – Patna (Bihar)-803213

(M/s BHEL )

Ref.		:    		4400/FQA/2022(Civil)/AV/1518      dated  09.02.2022

Tax Invoice No. : 	104 IIT BHU dated 20.04.2022

Details of Sample :      Concrete Mix Design.

Test Required       	  (i) M-20 grade without admixture with PPC Cement for General Concrete

(ii) M-30 grade without admixture with PPC Cement for Piling

(iii) M-25 grade with admixture with PPC Cement for Substructure and General Concrete

(iv) M-30 grade with admixture with PPC Cement for Substructure and General Concrete

(v) M-30 grade with admixture with PPC Cement for Piling

______________________________________________________________________________

Sub :  Evaluation of Materials & Concrete Mix Design for Construction of FGD Package for Stage-I & II at Barh

Super Thermal Power Project, Barh, Patna (Bihar)

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was  Dalmia Cement, PPC, IS : 1489, Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.  Admixture used SikaPlast-350 PR.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./39(i)/2022-23/                                                             Dated: 22.08.2022

#### (Page – 2(i))

#### TEST REPORTS

Recommended Mix Proportion for (i) M-20 grade without admixture with PPC Cement for General Concrete (ii) M-30 grade without admixture with PPC Cement for Piling

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M-20 Grade | 0.55 : 1 : 2.23 : 1.13 : 1.92 0.693 : 1 : 1.54 : 0.91 : 1.40 | By weight By volume |
| 2. | M-30 Grade | 0.46 : 1 : 1.80 : 1.09 : 1.38 0.58 : 1 : 1.25 : 0.88 : 1.01 | By weight By volume |

| Sl. No. | Test | M-20 grade without admixture for General concrete | M-30 grade without admixture for Piling |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 27.6 | 39.9 |
| 2 | Slump range (mm) | 100-150 | 150-200 |
| 3 | Slump Achieved in the Laboratory (mm) | 120.0 | 170.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 20.5 28.5 | 31.5 40.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 195.25 355.0 792.0 401.0 682.0 | 195.8 426.0 767.3 464.39 585.96 |

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./39(i)/2022-23/                                                             Dated: 22.08.2022

#### (Page – 2(ii))

#### TEST REPORTS

Recommended Mix Proportion for  (iii) M-25 grade with admixture with PPC Cement for Substructure and General Concrete (iv) M-30 grade with admixture with PPC Cement for Substructure and General Concrete

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 grade | 0.50 : 1 : 2.33 : 1.27 : 1.91 : 0.40* 0.63 : 1 : 1.61 : 1.03 : 1.39 : 0.40* | By weight By volume |
| 2. | M-30 grade | 0.45 : 1 : 2.00 : 1.15 : 1.73 : 0.40* 0.567 : 1 : 1.38 : 0.93 : 1.26 : 0.40* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture for Substructure and General Concrete | M-30 grade with admixture for Substructure and General Concrete |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 |
| 2 | Slump range (mm) | 125-175 | 125-175 |
| 3 | Slump Achieved in the Laboratory (mm) | 145.0 | 135.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 24.5 34.5 | 30.5 41.0 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) SikaPlast-350 PR (Litre) | 175.0 350.0 816.0 445.0 669.0 1.40 | 175.5 390.0 780.0 449.0 675.0 1.56 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./39(i)/2022-23/                                                             Dated: 22.08.2022

#### (Page – 2(iii))

#### TEST REPORTS

Recommended Mix Proportion for (v)  M-30 grade with admixture with PPC Cement for Piling

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-30 grade | 0.45 : 1 : 1.97 : 1.21 : 1.49 : 0.45* 0.567 : 1 : 1.36 : 0.98 : 1.09 : 0.45* | By weight By volume |

| Sl. No. | Test | M-30 grade with admixture for Piling |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 39.9 |
| 2 | Slump range (mm) | 150-200 |
| 3 | Slump Achieved in the Laboratory (mm) | 180.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 31.5 41.5 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) SikaPlast-350 PR (Litre) | 180.0 400.0 788.0 484.0 596.0 1.80 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./39(i)/2022-23/                                                             Dated: 22.08.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design (i) M-20 grade without admixture with PPC Cement for General Concrete  (ii) M-30 grade without admixture with PPC Cement for Piling           (iii) M-25 grade with admixture with PPC Cement for Substructure and General Concrete (iv) M-30 grade with admixture with PPC Cement for Substructure and General Concrete (v) M-30 grade with admixture with PPC Cement for Piling

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm (Source: Koderama Bihar) | CA—10 mm (Source: Koderama Bihar) | FA (sand) (Source: Sone River Bihar) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.75 | 100.0 | - |
|  | 10 mm | 0.40 | 89.25 | - |
|  | 4.75 mm | 0.05 | 7.90 | 3.6 |
|  | 2.36 mm | - | 0.40 | 14.8 |
|  | 1.18 mm | - | 0.05 | 42.8 |
|  | 600 micron | - | - | 77.6 |
|  | 300 micron | - | - | 97.6 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 7.128 | 6.024 | 3.352 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.56 | 1.82 |
| 4. | Specific Gravity | 2.87 | 2.78 | 2.74 |
| 5. | Water absorption | 0.15 % | 0.41 % | 0.95% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./39(i)/2022-23/                                                             Dated: 22.08.2022

#### TEST REPORTS

Tested for 	:      	AGM (FQA)

NTPC Limited, FQA, NTPC – Barh

Distt. – Patna (Bihar)-803213

(M/s BHEL )

Ref.		:    		4400/FQA/2022(Civil)/AV/1518      dated  09.02.2022

Details of Sample:	Cement: Dalmia Cement, PPC, IS : 1489, Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i) M-20 grade without admixture with PPC Cement for General Concrete  (ii) M-30 grade without admixture with PPC Cement for Piling  (iii) M-25 grade with admixture with PPC Cement for Substructure and General Concrete (iv) M-30 grade with admixture with PPC Cement for Substructure and General Concrete (v) M-30 grade with admixture with PPC Cement for Piling

______________________________________________________________________________

Sub :  Evaluation of Materials & Concrete Mix Design for Construction of FGD Package for Stage-I & II at Barh

Super Thermal Power Project, Barh, Patna (Bihar)

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 8 % 37 % 1.5 mm Expansion 140 Minutes 350 Minutes 18.5 N/mm2 24.7 N/mm2 35.9 N/mm2 1.26 2.86 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./147/2022-23/                                                         Dated: 22.08.2022

#### TEST REPORTS

Tested for 	:      	DGM (Civil)

Telecommunications Consultants India Ltd.

K-703, Yamuna Apartments, Sector-4,

Gomti Nagar Extension, Lucknow, U.P. – 226010

(M/s Naresh Kumar Gupta & Co. Private Limied)

Ref.		:    	 TCIL/Mix Des/IIT BHU/2022/1   Dated 06.06.2022

Tax Invoice No. : 	376 IIT BHU dated 14.06.2022

Details of Sample :      Concrete Mix Design.

Test Required       	 M-25 grade with admixture

________________________________________________________________________

Sub :  Regarding M-25 Grade Mix Design for “Construction  of Phase-A works at JNV Sultanpur (UP).

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 43-grade, IS : 269,  Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Sika Plastiment 2001 NS.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./147/2022-23/                                                         Dated: 22.08.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 1.91 : 1.27 : 1.90 : 0.6* 0.70 : 1 : 1.53 : 1.15 : 1.55 : 0.6* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 31.6 |
| 2 | Slump range (mm) | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 130.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 26.0 33.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Sika Plastiment 2001 NS (Litre) | 182.5 364.9 697.8 462.3 693.4 2.189 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./147/2022-23/                                                         Dated: 22.08.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design M-25 grade with

admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm ( | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 87.1 | 100.0 | - |
|  | 10 mm | 0.60 | 85.9 | - |
|  | 4.75 mm | 0.05 | 0.65 | 2.6 |
|  | 2.36 mm | - | 0.45 | 7.2 |
|  | 1.18 mm | - | 0.10 | 20.8 |
|  | 600 micron | - | - | 50.2 |
|  | 300 micron | - | - | 89.8 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.123 | 6.129 | 2.696 |
| 3. | Bulk Density (Kg./Litre) | 1.72 | 1.55 | 1.75 |
| 4. | Specific Gravity | 2.63 | 2.84 | 2.67 |
| 5. | Water absorption | 1.41% | 0.20% | 1.18 % |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./147/2022-23/                                                         Dated: 22.08.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	DGM (Civil)

Telecommunications Consultants India Ltd.

K-703, Yamuna Apartments, Sector-4,

Gomti Nagar Extension, Lucknow, U.P. – 226010

(M/s Naresh Kumar Gupta & Co. Private Limied)

Ref.		:    	 TCIL/Mix Des/IIT BHU/2022/1   Dated 06.06.2022

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grade, IS : 269,  Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with admixture

______________________________________________________________________________

Sub :  Regarding M-25 Grade Mix Design for “Construction  of Phase-A works at JNV Sultanpur (UP).

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 32 % 1.0 mm Expansion 95 Minutes 265 Minutes 25.7 N/mm2 36.1 N/mm2 46.3 N/mm2 1.40 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./153/2022-23/                                                          Dated: 22.08.2022

#### TEST REPORTS

Tested for 	:      	Director

M/s  MHPL INDIA PRIVATE LIMITED

15/276, Civil Lines, Kanpur  Uttar Pradesh, 208001

Ref.		:    		MHPL/SSC/IIT-BHU/DM/220614      Dated : 14.06.2022

Tax Invoice No. : 	391 IIT BHU dated 17.06.2022

Details of Sample :      Concrete Mix Design.

Test Required       	  (i) M-25 grade with admixture for Plinth Beam, Grade slab & Deck slab

(ii) M-30 grade with admixture for Foundation Footing & Pedestal

________________________________________________________________________

Sub :  Concrete Design Mix for Development of Sigra Sports Stadium, Varanasi.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was MP Birla Multicem Cement,  PPC IS : 1489, Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Auramix 250 DIS

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./153/2022-23/                                                          Dated: 22.08.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for  (i) M-25 grade with admixture for Plinth Beam, Grade slab & Deck slab (ii) M-30 grade with admixture for Foundation Footing & Pedestal

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 grade | 0.50 : 1 : 2.05 : 1.43 : 2.15 : 0.35* 0.66 : 1 : 1.53 : 1.20 : 1.62 : 0.35* | By weight By volume |
| 2. | M-30 grade | 0.45 : 1 : 1.76 : 1.28 : 1.93 : 0.35* 0.59 : 1 : 1.41 : 1.07 : 1.45 : 0.35* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture for Plinth Beam, Grade slab & Deck slab | M-30 grade with admixture for Foundation Footing & Pedestal |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 31.6 | 38.25 |
| 2 | Slump range (mm) | 100-150 | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 135.0 | 135.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 25.5 33.0 | 31.5 40.0 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Auramix 250 DIS (Litre) | 172.3 344.7 705.5 493.7 740.5 1.21 | 172.3 383.0 673.8 492.1 738.1 1.34 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./153/2022-23/                                                          Dated: 22.08.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design for   (i) M-25 grade

with admixture for Plinth Beam, Grade slab & Deck slab (ii) M-30 grade with admixture for

Foundation Footing & Pedestal

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm ( | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 88.25 | 100.0 | - |
|  | 10 mm | 0.45 | 88.0 | - |
|  | 4.75 mm | 0.05 | 10.85 | 2.4 |
|  | 2.36 mm | - | 0.45 | 5.8 |
|  | 1.18 mm | - | 0.10 | 16.6 |
|  | 600 micron | - | - | 53.4 |
|  | 300 micron | - | - | 86.0 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.113 | 6.006 | 2.632 |
| 3. | Bulk Density (Kg./Litre) | 1.74 | 1.56 | 1.76 |
| 4. | Specific Gravity | 2.86 | 2.84 | 2.66 |
| 5. | Water absorption | 0.15 % | 0.15% | 1.21 % |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./153/2022-23/                                                          Dated: 22.08.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Director

M/s  MHPL INDIA PRIVATE LIMITED

15/276, Civil Lines, Kanpur  Uttar Pradesh, 208001

Ref.		:    		MHPL/SSC/IIT-BHU/DM/220614      Dated : 14.06.2022

Details of Sample:	Cement: MP Birla Multicem Cement,  PPC IS : 1489, Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i) M-25 grade with admixture for Plinth Beam, Grade slab & Deck slab (ii) M-30 grade with admixture for Foundation Footing & Pedestal

______________________________________________________________________________

Sub :  Concrete Design Mix for Development of Sigra Sports Stadium, Varanasi

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 36 % 1.5 mm Expansion 120 Minutes 345 Minutes 24.8 N/mm2 35.1 N/mm2 45.3 N/mm2 1.31 2.91 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./130/2022-23/                                                             Dated: 16.08.2022

#### TEST REPORTS

Tested for 	:      	Dy. Project Manager

U.P. State Bridge Corporation Ltd.

Bridge Construction Unit, Old Normal School,

Shivpur, Varanasi-221003

Ref.		:    		689 / Test Report /21-22  Dated : 21-07-20201

Tax Invoice No. : 	318 IIT BHU dated 27.05.2022

Details of Sample :      Concrete Mix Design.

Test Required       	  (i) M-30 grade with admixture

(ii) M-35 grade with admixture

(iii) M-40 grade with admixture

(iv) M-45 grade with admixture

__________________________________________________________________________

Sub :  Testing of materials for the Bridge under construction in Varanasi Unit of U.P. State Bridge Corporation

Limited.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was  Ultra Tech Cement, OPC, 43-grade IS : 269, W-10, M-03,Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.  Admixture used Auramix 350.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./130/2022-23/                                                             Dated: 16.08.2022

#### (Page – 2(i))

#### TEST REPORTS

Recommended Mix Proportion for (i) M-30 grade with admixture (ii) M-35 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-30 grade | 0.45 : 1 : 1.82 : 1.25 : 1.89 : 0.60* 0.63 : 1 : 2.39 : 1.07 : 1.50 : 0.60* | By weight By volume |
| 2. | M-35 grade | 0.40 : 1 : 1.55 : 1.06 : 1.61 : 0.70* 0.56 : 1 : 1.19 : 0.90 : 1.28 : 0.70* | By weight By volume |

| Sl. No. | Test | M-30 Grade with admixture | M-40 Grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 38.25 | 43.25 |
| 2 | Slump range (initial) (mm) | 175-225 | 175-225 |
| 3 | Slump range (after one hour ) (mm) | 100-125 | 100-125 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 190.0 | 185.0 |
| 5 | Slump Achieved in the Laboratory(after one hour) (mm) | 100.0 | 100.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 30.0 39.5 | 34.5 45.0 |
| 7 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Auramix 350 (Litre) | 175.5 390.0 710.0 487.56 736.51 2.34 | 177.8 445.0 689.63 473.50 715.27 3.115 |

Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./130/2022-23/                                                             Dated: 16.08.2022

#### (Page – 2(ii))

#### TEST REPORTS

#### TEST REPORTS

Recommended Mix Proportion for (i) M-40 grade with admixture (ii) M-45 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-40 grade | 0.35 : 1 : 1.57 : 1.08 : 1.63 : 0.80* 0.49 : 1 : 1.21 : 0.92 : 1.30 : 0.80* | By weight By volume |
| 2. | M-45 grade | 0.30 : 1 : 1.63 : 1.12 : 1.69 : 0.90* 0.42 : 1 : 1.25 : 0.96 : 1.34 : 0.90* | By weight By volume |

| Sl. No. | Test | M-40 Grade with admixture | M-45 Grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 48.25 | 53.25 |
| 2 | Slump range (initial) (mm) | 175-225 | 175-225 |
| 3 | Slump range (after one hour ) (mm) | 100-125 | 100-125 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 175.0 | 175.0 |
| 5 | Slump Achieved in the Laboratory(after one hour) (mm) | 110.0 | 125.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 40.0 51.0 | 44.5 55.5 |
| 7 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Auramix 350 (Litre) | 157.5 450.0 708.73 486.61 735.08 3.600 | 135.0 450.0 731.73 502.40 758.93 4.050 |

Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./130/2022-23/                                                             Dated: 16.08.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design (i) M-30 grade with admixture

(ii) M-35 grade with admixture (iii)  M-40 grade with admixture  (iv)  M-45 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm (Source: Dalla) | CA—10 mm (Source: Dalla) | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.05 | 100.0 | - |
|  | 10 mm | 0.30 | 88.75 | - |
|  | 4.75 mm | 0.05 | 5.55 | 5.6 |
|  | 2.36 mm | - | 0.55 | 15.8 |
|  | 1.18 mm | - | 0.05 | 40.4 |
|  | 600 micron | - | - | 76.0 |
|  | 300 micron | - | - | 96.6 |
|  | 150 micron | - | - | 99.2 |
| 2. | Fineness Modulus | 7.136 | 6.051 | 3.336 |
| 3. | Bulk Density (Kg./Litre) | 1.74 | 1.63 | 1.81 |
| 4. | Specific Gravity | 2.95 | 2.83 | 2.69 |
| 5. | Water absorption | 0.40 % | 0.45 % | 1.25% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./130/2022-23/                                                             Dated: 16.08.2022

#### TEST REPORTS

Tested for 	:      	Dy. Project Manager

U.P. State Bridge Corporation Ltd.

Bridge Construction Unit, Old Normal School,

Shivpur, Varanasi-221003

Ref.		:    		689 / Test Report /21-22  Dated : 21-07-20201

Tax Invoice No. : 	318 IIT BHU dated 27.05.2022

Details of Sample:	Cement: Ultra Tech Cement, OPC, 43-grade IS : 269, W-10, M-03,Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i) M-30 grade with admixture

(ii) M-35 grade with admixture (iii)  M-40 grade with admixture  (iv)  M-45 grade with admixture

______________________________________________________________________________

Sub :  Testing of materials for the Bridge under construction in Varanasi Unit of U.P. State Bridge Corporation

Limited.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3% 31 % 1.0 mm Expansion 85 Minutes 275 Minutes 26.7 N/mm2 36.5 N/mm2 46.9 N/mm2 1.39 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./139(i)/2022-23/                                                            Dated: 16.08.2022

#### TEST REPORTS

Tested for 	:      	M/s Santosh Kumar Singh

Vill. Gangapur, Post- Mangari,

Distt- Varanasi, U.P. 221202

Ref.			:    	Nil dated 27.05.2022

Tax Invoice No. : 	 355 IIT BHU dated 07.06.2022

Details of Sample :      	Concrete Mix Design.

Test Required       	M-25 grade

________________________________________________________________________

Sub: Testing of materials and mix of M25 RCC

Work:  Construction of G+11 Barrack for 200 persons in 48th Vahini PAC Sonebhadra

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ambuja Buildcem Cement, PPC. IS : 1489, W-17, M-04, Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./139(i)/2022-23/                                                            Dated: 16.08.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 1.68 : 1.17 : 1.75 0.64 : 1 : 1.23 : 0.96 : 1.29 | By weight By volume |

| Sl. No. | Test | M-25 grade |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 31.6 |
| 2 | Slump range (mm) | 75-125 |
| 3 | Slump Achieved in the Laboratory (mm) | 105.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 25.5 32.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 197.2 394.3 660.5 460.0 690.0 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./139(i)/2022-23/                                                            Dated: 16.08.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design for M-25 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm ( | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.9 | 100.0 | - |
|  | 10 mm | 0.30 | 86.4 | - |
|  | 4.75 mm | 0.05 | 5.90 | 3.6 |
|  | 2.36 mm | - | 0.40 | 10.8 |
|  | 1.18 mm | - | 0.05 | 29.0 |
|  | 600 micron | - | - | 61.0 |
|  | 300 micron | - | - | 91.0 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.128 | 6.073 | 2.944 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.56 | 1.75 |
| 4. | Specific Gravity | 2.84 | 2.81 | 2.65 |
| 5. | Water absorption | 0.25 % | 0.45% | 1.45% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./139(i)/2022-23/                                                            Dated: 16.08.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	M/s Santosh Kumar Singh

Vill. Gangapur, Post- Mangari,

Distt- Varanasi, U.P. 221202

Ref.			:    	Nil dated 27.05.2022

Details of Sample:	Cement:  Ambuja Buildcem Cement, PPC. IS : 1489, W-17, M-04, Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade

______________________________________________________________________________

Sub: Testing of materials and mix of M25 RCC

Work:  Construction of G+11 Barrack for 200 persons in 48th Vahini PAC Sonebhadra

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 35 % 1.5 mm Expansion 120 Minutes 350 Minutes 24.8 N/mm2 35.1 N/mm2 45.2 N/mm2 1.28 2.88 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./171/2022-23/                                                      Dated: 12.08.2022

#### TEST REPORTS

Tested for 	:      	Executive Engineer

U.P. Jal Nigam (Rural)

Division Office P.D.D.U.N.  Chandauli

(M/s Ion Exchange (India) Limited

Ref.		:    	1962 /  ty thou fe“ku /260      dated  28-05-2022

Tax Invoice No. : 	453 IIT BHU dated 05.07.2022

Details of Sample :      Concrete Mix Design.

Test Required       	  M-25 grade  with admixture and M-30 grade  with admixture

________________________________________________________________________

Sub: Testing of Construction Materials   for RCC Overhead Tank under Jal Jeevan Mission Scheme

(Phase-II) at District Chandauli

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC-43 grade. IS:269, W-21, M-05, Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Dr. FIXIT

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./171/2022-23/                                                      Dated: 12.08.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade with admixture and M-30 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.11 : 1.34 : 1.64 : 0.40* 0.71 : 1 : 1.68 : 1.21 : 1.34 : 0.40* | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 1.86 : 1.18 : 1.45 : 0.40* 0.63 : 1 : 1.48 : 1.07 : 1.18 : 0.40* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture | M-30 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 31.6 | 38.25 |
| 2 | Slump range (mm) | 50-100 | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 80.0 | 70.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 25.0 32.0 | 30.0 39.0 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Dr. FIXIT (Litre) | 187.0 374.0 790.0 499.7 615.0 1.496 | 187.0 416.0 774.7 489.8 602.0 1.664 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./171/2022-23/                                                      Dated: 12.08.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design for M-25 grade with

admixture and M-30 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm ( | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 87.3 | 100.0 | - |
|  | 10 mm | 0.40 | 88.35 | - |
|  | 4.75 mm | 0.05 | 5.40 | 2.6 |
|  | 2.36 mm | - | 0.40 | 11.4 |
|  | 1.18 mm | - | 0.05 | 39.0 |
|  | 600 micron | - | - | 74.8 |
|  | 300 micron | - | - | 95.6 |
|  | 150 micron | - | - | 99.8 |
| 2. | Fineness Modulus | 7.123 | 6.058 | 3.222 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.56 | 1.77 |
| 4. | Specific Gravity | 2.85 | 2.83 | 2.67 |
| 5. | Water absorption | 0.25 % | 0.35% | 1.38 % |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./171/2022-23/                                                      Dated: 12.08.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Executive Engineer

U.P. Jal Nigam (Rural)

Division Office P.D.D.U.N.  Chandauli

(M/s Ion Exchange (India) Limited

Ref.		:    	1962 /  ty thou fe“ku /260      dated  28-05-2022

Details of Sample:	Cement: Ultra Tech Cement, OPC-43 grade. IS:269, W-21, M-05, Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with admixture and M-30 grade with admixture

______________________________________________________________________________

Sub: Testing of Construction Materials   for RCC Overhead Tank under Jal Jeevan Mission Scheme

(Phase-II) at District Chandauli

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 31 % 1.0 mm Expansion 95 Minutes 260 Minutes 25.9 N/mm2 36.1 N/mm2 46.4 N/mm2 1.41 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./27/2022-23/                                                             Dated: 11.08.2022

#### TEST REPORTS

Tested for 	:      	AGM (FQA)

NTPC Limited

Farakka

(M/s Kanwar Enterprises Pvt. Ltd.)

Ref.			:    	 Nil    Dated 06.04.2022

Tax Invoice No. : 	 75 IIT BHU dated 12.04.2022

Details of Sample :      	Concrete Mix Design.

Test Required       	 M-25 grade

________________________________________________________________________

Sub: Mix design M-25 grade

Name of work: Construction of Toe drain and Pump house at Nischindra, Malancha and Kendua Ash Dyke, at NTPC Farakka.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC. IS : 1489, W-14, M-04, Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./27/2022-23/                                                             Dated: 11.08.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M25 Grade | 0.50 : 1 : 1.60 : 1.08 : 1.60 0.645 : 1 : 1.16 : 0.86 : 1.22 | By weight By volume |

| Test | M-25 Grade |
| --- | --- |
| Target Strength (N/mm2) | 31.6 |
| Slump range (initial) (mm) | 100-150 |
| Slump range (after one hour ) (mm) | 75-100 |
| Slump Achieved in the Laboratory (initial) (mm) | 130.0 |
| Slump Achieved in the Laboratory(after one hour) (mm) | 100.0 |
| i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 23.0 32.0 |
| Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 208.3 417.0 667.47 451.57 667.78 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./27/2022-23/                                                             Dated: 11.08.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design for M-25 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm ( | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 91.1 | 100.0 | - |
|  | 10 mm | 0.90 | 87.9 | - |
|  | 4.75 mm | 0.05 | 1.40 | 2.4 |
|  | 2.36 mm | - | 0.45 | 10.8 |
|  | 1.18 mm | - | 0.05 | 36.2 |
|  | 600 micron | - | - | 71.2 |
|  | 300 micron | - | - | 94.4 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 7.080 | 6.102 | 3.138 |
| 3. | Bulk Density (Kg./Litre) | 1.69 | 1.62 | 1.78 |
| 4. | Specific Gravity | 2.79 | 2.83 | 2.73 |
| 5. | Water absorption | 2.11 % | 1.51% | 0.63% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./27/2022-23/                                                             Dated: 11.08.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	AGM (FQA)

NTPC Limited

Farakka

(M/s Kanwar Enterprises Pvt. Ltd.)

Ref.			:    	 Nil    Dated 06.04.2022

Details of Sample:	Cement:  Ultra Tech Cement, PPC. IS : 1489, W-14, M-04, Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade

______________________________________________________________________________

Sub: Mix design M-25 grade

Name of work: Construction of Toe drain and Pump house at Nischindra, Malancha and Kendua Ash Dyke, at NTPC Farakka.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 140 Minutes 370 Minutes 23.6 N/mm2 34.0 N/mm2 44.3 N/mm2 1.29 2.89 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./114(ii)/2022-23/                                                      Dated: 03.08.2022

#### TEST REPORTS

Tested for 	:      	Executive Engineer (PD)

East central Railway,

Plant Depot, Mughalsarai

(M/s Jawed Enterprises)

Ref.		:    	PD/BT/04/XEN(PD)/21-22       dated  04/11-05-2022

Tax Invoice No. : 	263 IIT BHU dated 24.05.2022

Details of Sample :      Concrete Mix Design.

Test Required       	 M-45 grade with admixture with Granite Stone Chips

________________________________________________________________________

Sub :  Concrete Mix Design for M 45 grade for PSC bridge slabs.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Double Bull Nuvaco Cement, OPC-53 grade. IS : 269, Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Buildplast Super 2000-M.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./114(ii)/2022-23/                                                      Dated: 03.08.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-45 grade with admixture with Granite Stone Chips

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-45 Grade | 0.35 : 1 : 1.53 : 1.04 : 1.56 : 0.4* 0.49 : 1 : 1.19 : 0.94 : 1.31 : 0.4* | By weight By volume |

| Sl. No. | Test | M-45 grade with admixture with Granite Stone Chips |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 53.25 |
| 2 | Slump range (mm) | 0-50 |
| 3 | Slump Achieved in the Laboratory (mm) | 25.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 46.0 57.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Buildplast Super 2000-M (Litre) | 157.5 450.0 687.65 465.98 704.13 1.80 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./114(ii)/2022-23/                                                      Dated: 03.08.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design for M-45 grade with

admixture with Granite Stone Chips

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm (Granite tone Chips) | CA—10 mm (Granite tone Chips) | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 93.65 | 100.0 | - |
|  | 10 mm | 0.30 | 85.55 | - |
|  | 4.75 mm | 0.05 | 5.95 | 1.8 |
|  | 2.36 mm | - | 0.30 | 9.6 |
|  | 1.18 mm | - | 0.05 | 40.2 |
|  | 600 micron | - | - | 61.0 |
|  | 300 micron | - | - | 92.2 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 7.060 | 6.082 | 3.036 |
| 3. | Bulk Density (Kg./Litre) | 1.66 | 1.54 | 1.78 |
| 4. | Specific Gravity | 2.73 | 2.71 | 2.61 |
| 5. | Water absorption | 0.20 % | 0.40% | 1.79 % |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./114(ii)/2022-23/                                                      Dated: 03.08.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Executive Engineer (PD)

East central Railway,

Plant Depot, Mughalsarai

(M/s Jawed Enterprises)

Ref.		:    	PD/BT/04/XEN(PD)/21-22       dated  04/11-05-2022

Details of Sample:	Cement: Double Bull Nuvaco Cement, OPC-53 grade. IS : 269, Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design  M-45 grade with

admixture with Granite Stone Chips

______________________________________________________________________________

Sub :  Concrete Mix Design for M 45 grade for PSC bridge slabs.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 2 % 30 % 1.0 mm Expansion 80 Minutes 270 Minutes 32.4 N/mm2 39.6 N/mm2 54.7 N/mm2 1.39 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./114(i)/2022-23/                                                      Dated: 03.08.2022

#### TEST REPORTS

Tested for 	:      	Executive Engineer (PD)

East central Railway,

Plant Depot, Mughalsarai

(M/s Jawed Enterprises)

Ref.		:    	PD/BT/04/XEN(PD)/21-22       dated  04/11-05-2022

Tax Invoice No. : 	263 IIT BHU dated 24.05.2022

Details of Sample :      Concrete Mix Design.

Test Required       	 M-35 grade with admixture with Dolomite Stone Chips

________________________________________________________________________

Sub :  Concrete Mix Design for M35 grade Reinforced Cement Concrete for casting of Ballast Retainer

RCC bridge slabs.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Double Bull Nuvaco Cement, OPC-53 grade. IS : 269, Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Buildplast Super 2000-M.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./114(i)/2022-23/                                                      Dated: 03.08.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-35 grade with admixture with Dolomite Stone Chips

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-35 Grade | 0.40 : 1 : 1.64 : 1.18 : 1.77 : 0.3* 0.556 : 1 : 1.28 : 1.01 : 1.46 : 0.3* | By weight By volume |

| Sl. No. | Test | M-35 grade with admixture with Dolomite Stone Chips |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 45.395 |
| 2 | Slump range (mm) | 0-50 |
| 3 | Slump Achieved in the Laboratory (mm) | 50.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 40.0 51.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Buildplast Super 2000-M (Litre) | 167.4 418.5 687.9 491.96 740.5 1.255 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./114(i)/2022-23/                                                      Dated: 03.08.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design for M-35 grade with

admixture with Dolomite Stone Chips

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm (Dolomite Stone Chips) | CA—10 mm (Dolomite Stone Chips) | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.2 | 100.0 | - |
|  | 10 mm | 0.45 | 89.25 | - |
|  | 4.75 mm | 0.05 | 6.40 | 1.8 |
|  | 2.36 mm | - | 0.30 | 9.6 |
|  | 1.18 mm | - | 0.05 | 40.2 |
|  | 600 micron | - | - | 61.0 |
|  | 300 micron | - | - | 92.2 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 6.133 | 6.040 | 3.036 |
| 3. | Bulk Density (Kg./Litre) | 1.69 | 1.63 | 1.78 |
| 4. | Specific Gravity | 2.87 | 2.86 | 2.61 |
| 5. | Water absorption | 0.20 % | 0.40% | 1.79 % |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./114(i)/2022-23/                                                      Dated: 03.08.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Executive Engineer (PD)

East central Railway,

Plant Depot, Mughalsarai

(M/s Jawed Enterprises)

Ref.		:    	PD/BT/04/XEN(PD)/21-22       dated  04/11-05-2022

Details of Sample:	Cement: Double Bull Nuvaco Cement, OPC-53 grade. IS : 269, Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design  M-35 grade with admixture with Dolomite Stone Chips

______________________________________________________________________________

Sub :  Concrete Mix Design for M35 grade Reinforced Cement Concrete for casting of Ballast Retainer

RCC bridge slabs.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 2 % 30 % 1.0 mm Expansion 80 Minutes 270 Minutes 32.4 N/mm2 39.6 N/mm2 54.7 N/mm2 1.39 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./164/2022-23/ 	                                                  Dated: 03.08.2022

#### TEST REPORTS

Tested for 	:      	M/s Mirzapur Ghazipur STPs Private Limited

Toolika Public School

Jamuna Deva, Mau Road

Ghazipur-233001

Ref.		:    	MGSPL/2021-22/GP/059 dated 15.06.2022

Tax Invoice No. : 	433  IIT BHU dated 28.06.2022

Details of Sample :      	Review and Vetting the Concrete Mix Design

Test Required       	(i) M-25 grade with OPC 43-grade cement with admixture

(ii) M-30 grade with OPC 43-grade cement with admixture

(iii) M-30 grade with SRPC cement with admixture

__________________________________________________________________________

Sub: Review and vetting the Design Mix of M-25 and M-30 Concrete for Construction at Ghazipur site

Project: Design, Build, rehabilitation, Finance, Operate and Transfer Sewage Treatment Plants (STPs) with  associate Infrastructure, with  operation and maintenance period of 15 years under “One City One Operator”  Concept through Hybrid Annuity Based PPP Model in Mirzapur and Ghazipur, Uttar Pradesh, India; under the Namami Gange Progrmme.

#### REPORT

With reference to your above letter regarding verification of above mentioned mix design submitted by you, 7-days and 28-days compressive strength with slump are as under:

| Sl. No. | Test | M-25 grade with OPC 43-grade cement with admixture | M-30 grade with OPC 43-grade cement with admixture | M-30 grade with SRPC cement with admixture |
| --- | --- | --- | --- | --- |
| 1. | Slump Achieved in the Laboratory (mm) | 160.0 | 170.0 | 175.0 |
| 2 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 30.0 35.0 | 36.0 41.0 | 36.5 42.0 |

The vetted mix designs are enclosed herewith.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./88/2022-23/                                                          Dated: 03.08.2022

#### TEST REPORTS

Tested for 	:      	M/s Skyline Architectural Consultant

11th Floor, CP-3, Laxmi Vandan Complex,

Patrakarpuram Crossing, Gomti Nagar, Lucknow -226010

(M/s KPC Projects Limited)

Ref.		:    	399/3   dated : 05-04-2022

Tax Invoice No. : 	225 IIT BHU dated 12.05.2022

Details of Sample :      Concrete Mix Design with Prism Cement, OPC.

Test Required       	  M-30 grade with admixture and M-40 grade with admixture

________________________________________________________________________

Sub :  Design Mix & Testing of other construction materials.

Work: Construction of New Medical College at Barathi Kamrour Village in Chandauli District.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Prism Cement, OPC-43 grade. IS : 269, Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used SmartCare Maximo  Plast PC 200

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./88/2022-23/                                                          Dated: 03.08.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-30 grade with admixture and M-40 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-30 Grade | 0.45 : 1 : 1.87 : 1.37 : 2.05 : 1.0* 0.626 : 1 : 1.46 : 1.18 : 1.65 : 1.0* | By weight By volume |
| 2. | M-40 Grade | 0.38 : 1 : 1.53 : 1.13 : 1.68 : 1.0* 0.528 : 1 : 1.20 : 0.98 : 1.35 : 1.0* | By weight By volume |

| Sl. No. | Test | M-30 grade with admixture | M-40 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 38.25 | 48.25 |
| 2 | Slump range (mm) | 125-175 | 125-175 |
| 3 | Slump Achieved in the Laboratory (mm) | 155.0 | 170.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 31.5 40.5 | 38.5 51.5 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) SmartCare Maximo Plast PC 200 (Litre) | 165.0 367.0 687.0 502.0 752.0 3.67 | 165.0 434.0 665.0 487.0 728.0 4.34 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./88/2022-23/                                                          Dated: 03.08.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design for M-30 grade with

admixture and M-40 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm ( | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.55 | 100.0 | - |
|  | 10 mm | 0.40 | 87.55 | - |
|  | 4.75 mm | 0.05 | 5.45 | 2.0 |
|  | 2.36 mm | - | 0.40 | 5.8 |
|  | 1.18 mm | - | 0.05 | 19.8 |
|  | 600 micron | - | - | 57.4 |
|  | 300 micron | - | - | 92.8 |
|  | 150 micron | - | - | 99.6 |
| 2. | Fineness Modulus | 7.130 | 6.066 | 2.774 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.61 | 1.78 |
| 4. | Specific Gravity | 2.83 | 2.84 | 2.64 |
| 5. | Water absorption | 0.40 % | 0.35% | 0.98 % |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./88/2022-23/                                                          Dated: 03.08.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	M/s Skyline Architectural Consultant

11th Floor, CP-3, Laxmi Vandan Complex,

Patrakarpuram Crossing, Gomti Nagar, Lucknow -226010

(M/s KPC Projects Limited)

Ref.		:    	399/3   dated : 05-04-2022

Details of Sample:	Cement: Prism Cement, OPC-43 grade. IS : 269, Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-30 grade with

admixture and M-40 grade with admixture

______________________________________________________________________________

Sub :  Design Mix & Testing of other construction materials.

Work: Construction of New Medical College at Barathi Kamrour Village in Chandauli District.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 34 % 1.0 mm Expansion 85 Minutes 275 Minutes 26.0 N/mm2 36.5 N/mm2 46.0 N/mm2 1.39 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./164/2022-23/ 	                                                  Dated: 03.08.2022

#### TEST REPORTS

Tested for 	:      	M/s Mirzapur Ghazipur STPs Private Limited

Toolika Public School

Jamuna Deva, Mau Road

Ghazipur-233001

Ref.		:    	MGSPL/2021-22/GP/059 dated 15.06.2022

Tax Invoice No. : 	433  IIT BHU dated 28.06.2022

Details of Sample :      	Review and Vetting the Concrete Mix Design

Test Required       	(i) M-25 grade with OPC 43-grade cement with admixture

(ii) M-30 grade with OPC 43-grade cement with admixture

(iii) M-30 grade with SRPC cement with admixture

__________________________________________________________________________

Sub: Review and vetting the Design Mix of M-25 and M-30 Concrete for Construction at Ghazipur site

Project: Design, Build, rehabilitation, Finance, Operate and Transfer Sewage Treatment Plants (STPs) with  associate Infrastructure, with  operation and maintenance period of 15 years under “One City One Operator”  Concept through Hybrid Annuity Based PPP Model in Mirzapur and Ghazipur, Uttar Pradesh, India; under the Namami Gange Progrmme.

#### REPORT

With reference to your above letter regarding verification of above mentioned mix design submitted by you, 7-days and 28-days compressive strength with slump are as under:

| Sl. No. | Test | M-25 grade with OPC 43-grade cement with admixture | M-30 grade with OPC 43-grade cement with admixture | M-30 grade with SRPC cement with admixture |
| --- | --- | --- | --- | --- |
| 1. | Slump Achieved in the Laboratory (mm) | 160.0 | 170.0 | 175.0 |
| 2 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 30.0 35.0 | 36.0 41.0 | 36.5 42.0 |

The vetted mix designs are enclosed herewith.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./138/2022-23/                                                      Dated: 02.08.2022

#### TEST REPORTS

Tested for 	:      	Executive Engineer

U.P. Jal Nigam (Rural)

Division Office Ghazipur-233001

(M/s V.S.A. Infra Project Pvt. Ltd.)

Ref.		:    	879/  iz;ksx'kYkk /05    dated  19-05-2022

Tax Invoice No. : 	354 IIT BHU dated 07.06.2022

Details of Sample :      Concrete Mix Design.

Test Required       	 (i) M-20 grade with admixture, (ii) M-25 grade with admixture

(ii) M-30 grade with admixture

________________________________________________________________________

Sub:  Cement concrete mix design for Overhead Tanks under Jal Jivan Mission of Payjal Scheme.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 43-grade, IS : 269,  Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used Fosroc Conplast SP 430 G8.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./138/2022-23/                                                      Dated: 02.08.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for (i) M-20 grade with admixture,  (ii) M-25 grade with admixture                             (ii) M-30 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-20 Grade | 0.52 : 1 : 2.31 : 155 : 2.33 : 0.7* 0.73 : 1 : 1.85 : 1.39 : 1.87 : 0.7* | By weight By volume |
| 2. | M-25 Grade | 0.50 : 1 : 2.13 : 1.50 : 2.24 : 0.7* 0.70 : 1 : 1.71 : 1.35 : 1.80 : 0.7* | By weight By volume |
| 3. | M-30 Grade | 0.45 : 1 : 1.84 : 1.34 : 2.02 : 0.7* 0.63 : 1 : 1.47 : 1.20 : 1.63 : 0.7* | By weight By volume |

| Sl. No. | Test | M-20 grade with admixture | M-25 grade with admixture | M-30 grade with admixture |
| --- | --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 26.6 | 31.6 | 38.25 |
| 2 | Slump range (mm) | 75-125 | 75-125 | 75-125 |
| 3 | Slump Achieved in the Laboratory (mm) | 100.0 | 95.0 | 90.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 24.5 30.5 | 28.5 33.5 | 33.5 41.5 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Fosroc Conplast SP 430 G8 (Litre) | 167.6 320.0 738.6 496.7 745.1 2.24 | 167.6 335.2 714.7 501.4 752.1 2.35 | 167.6 372.4 683.9 500.7 751.1 2.61 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./138/2022-23/                                                      Dated: 02.08.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design for (i) M-20 grade with admixture,  (ii) M-25 grade with admixture  (ii) M-30 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 87.15 | 100.0 | - |
|  | 10 mm | 0.40 | 85.55 | - |
|  | 4.75 mm | 0.05 | 0.55 | 2.6 |
|  | 2.36 mm | - | 0.20 | 8.6 |
|  | 1.18 mm | - | 0.10 | 25.0 |
|  | 600 micron | - | - | 57.6 |
|  | 300 micron | - | - | 90.8 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.124 | 6.136 | 2.836 |
| 3. | Bulk Density (Kg./Litre) | 1.74 | 1.56 | 1.75 |
| 4. | Specific Gravity | 2.85 | 2.82 | 2.64 |
| 5. | Water absorption | 0.20 % | 0.40% | 1.60% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./138/2022-23/                                                      Dated: 02.08.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Executive Engineer

U.P. Jal Nigam (Rural)

Division Office Ghazipur-233001

(M/s V.S.A. Infra Project Pvt. Ltd.)

Ref.		:    	879/  iz;ksx'kYkk /05    dated  19-05-2022

Details of Sample:	Cement:  Ultra Tech Cement, OPC 43-grade, IS : 269,  Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i) M-20 grade with admixture,  (ii) M-25 grade with admixture  (ii) M-30 grade with admixture

______________________________________________________________________________

Sub:  Cement concrete mix design for Overhead Tanks under Jal Jivan Mission of Payjal Scheme.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 31 % 1.0 mm Expansion 90 Minutes 255 Minutes 26.7 N/mm2 36.7 N/mm2 47.2 N/mm2 1.40 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./115/2022-23/                                                             Dated: 26.07.2022

#### TEST REPORTS

Tested for 	:      	Staff Officer (Civil)

Bina Project

Northern Coalfields Limited

Post-Bina Project,  Distt. Sonebhadra U.P.-231220

(M/s Madhucon Projects Limited)

Ref.		:    		B/Civil/Madhucon-CHP/22-23/74   Dated 27-04-20224

Tax Invoice No. : 	275 IIT BHU dated 24.05.2022

Details of Sample :      Concrete Mix Design.

Test Required       	  (i) M-10 grade with admixture

(ii) M-15 grade with admixture

(iii)  M-20 grade with admixture

(iv)  M-25 grade with admixture

(v) M-30 grade with admixture

(vi) M-35 grade with admixture

__________________________________________________________________________

Sub :  Design of M10, M15, M20, M25, M30 & M35 RCC Mix Design

Name of work : Planning, design, Engineering Constructions, fabrication, Supply, Installation, Testing Trial Run and commissioning of Coal Handling Plant of 9.5 MTPA consisting of all Civil Structural, Electrical and Mechanical works and all other  accessories and facilities required to make it complete in all respects on turnkey basis and operation and maintenance of plant for Five years in Bina Kakri Amalgamation project of NCL

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was  Ultra Tech Cement, OPC, 43-grade IS : 269, W-17, M-04,Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.  Admixture used Conplast SP 430 G8.                                          .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./115/2022-23/                                                             Dated: 26.07.2022

#### (Page – 2(i))

#### TEST REPORTS

Recommended Mix Proportion for (i) M-10 grade with admixture (ii) M-15 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-10 grade | 0.65 : 1 : 2.64 : 1.91 : 2.88 : 0.70* 0.91 : 1 : 2.08 : 1.66 : 2.32 : 0.70* | By weight By volume |
| 2. | M-15 grade | 0.60 : 1 : 2.41 : 1.75 : 2.63 : 0.70* 0.84 : 1 : 1.90 : 1.52 : 2.12 : 0.70* | By weight By volume |

| Sl. No. | Test | M-10 grade with admixture | M-15 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 14.95 | 19.95 |
| 2 | Slump range (mm) | 50-100 | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 80.0 | 80.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 15.0 19.0 | 19.5 25.0 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Conplast SP 430 G8 (Litre) | 180.0 276.92 731.75 529.83 797.45 1.94 | 180.0 300.0 724.26 524.4 789.27 2.10 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./115/2022-23/                                                             Dated: 26.07.2022

#### (Page – 2(ii))

#### TEST REPORTS

Recommended Mix Proportion for (iii) M-20 grade with admixture (iv) M-25 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-20 grade | 0.55 : 1 : 2.19 : 1.58 : 2.38 : 0.70* 0.77 : 1 : 1.72 : 1.37 : 1.92 : 0.70* | By weight By volume |
| 2. | M-25 grade | 0.50 : 1 : 1.96 : 1.42 : 2.13 : 0.70* 0.70 : 1 : 1.54 : 1.24 : 1.71 : 0.70* | By weight By volume |

| Sl. No. | Test | M-20 grade with admixture | M-25 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 26.6 | 31.6 |
| 2 | Slump range (mm) | 50-100 | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 85.0 | 75.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 22.0 30.0 | 27.5 36.0 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Conplast SP 430 G8 (Litre) | 180.0 327.27 715.4 517.99 779.62 2.29 | 180.0 360.0 704.77 510.3 768.05 2.52 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./115/2022-23/                                                             Dated: 26.07.2022

#### (Page – 2(iii))

#### TEST REPORTS

Recommended Mix Proportion for (v) M-30 grade with admixture (vi) M-35 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-30 grade | 0.45 : 1 : 1.73 : 1.25 : 1.88 : 0.70* 0.63 : 1 : 1.36 : 1.09 : 1.51 : 0.70* | By weight By volume |
| 2. | M-35 grade | 0.40 : 1 : 1.50 : 1.09 : 1.64 : 0.70* 0.56 : 1 : 1.18 : 0.95 : 1.32 : 0.70* | By weight By volume |

| Sl. No. | Test | M-30 grade with admixture | M-35 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 28.25 | 43.25 |
| 2 | Slump range (mm) | 50-100 | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 75.0 | 70.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 31.5 43.0 | 35.5 46.0 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Conplast SP 430 G8 (Litre) | 180.0 400.0 691.78 500.67 753.56 2.80 | 180.0 450.0 675.52 489.12 736.18 3.15 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./115/2022-23/                                                             Dated: 26.07.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design (i) M-10 grade with admixture

(ii) M-15 grade with admixture (iii)  M-20 grade with admixture  (iv)  M-25 grade with admixture (v) M-30 grade with admixture  (vi) M-35 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 87.1 | 100.0 | - |
|  | 10 mm | 0.40 | 88.85 | - |
|  | 4.75 mm | 0.05 | 0.40 | 2.2 |
|  | 2.36 mm | - | 0.25 | 4.8 |
|  | 1.18 mm | - | 0.05 | 18.8 |
|  | 600 micron | - | - | 51.6 |
|  | 300 micron | - | - | 92.2 |
|  | 150 micron | - | - | 99.4 |
| 2. | Fineness Modulus | 7.125 | 6.105 | 2.690 |
| 3. | Bulk Density (Kg./Litre) | 1.74 | 1.61 | 1.78 |
| 4. | Specific Gravity | 2.95 | 2.94 | 2.65 |
| 5. | Water absorption | 0.40 % | 0.40 % | 1.57% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./115/2022-23/                                                             Dated: 26.07.2022

#### TEST REPORTS

Tested for 	:      	Staff Officer (Civil)

Bina Project

Northern Coalfields Limited

Post-Bina Project,  Distt. Sonebhadra U.P.-231220

(M/s Madhucon Projects Limited)

Ref.		:    		B/Civil/Madhucon-CHP/22-23/74   Dated 27-04-20224

Details of Sample:	Cement: Ultra Tech Cement, OPC, 43-grade IS : 269, W-17, M-04,Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i) M-10 grade with admixture

(ii) M-15 grade with admixture (iii)  M-20 grade with admixture  (iv)  M-25 grade with admixture (v) M-30 grade with admixture  (vi) M-35 grade with admixture

______________________________________________________________________________

Sub :  Design of M10, M15, M20, M25, M30 & M35 RCC Mix Design

Name of work : Planning, design, Engineering Constructions, fabrication, Supply, Installation, Testing Trial Run and commissioning of Coal Handling Plant of 9.5 MTPA consisting of all Civil Structural, Electrical and Mechanical works and all other  accessories and facilities required to make it complete in all respects on turnkey basis and operation and maintenance of plant for Five years in Bina Kakri Amalgamation project of NCL

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3% 31 % 1.0 mm Expansion 85 Minutes 230 Minutes 27.2 N/mm2 37.6 N/mm2 47.3 N/mm2 1.40 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./163 /2022-23/ 	                                                  Dated: 26.07.2022

#### TEST REPORTS

Tested for 	:      	General Manager (Civil)

Nigahi Project

Northern Coalfields Limited

Post-Nigahi Project, Distt. Singrauli, M.P.-486884

(M/s Madhucon Projects Limited)

Ref.		:    		NGH/Civil/22-23/93    Dated 09-05-2022

Tax Invoice No. : 	432  IIT BHU dated 28.06.2022

Details of Sample :      	Concrete Mix Design

Test Required       	 (i)  M-15 grade with admixture

(ii)  M-20 grade with admixture

(iii) M-25 grade with admixture

(iv) M-30 grade with admixture

(v)  M-35 grade with admixture

__________________________________________________________________________

Sub :  Design of  M15, M20, M25, M30 & M35 grade

Name of work : Planning, design, Engineering Constructions, fabrication, Supply, Installation, Testing Trial Run and commissioning of Coal Handling Plant of  10  MTPA consisting of all Civil Structural, Electrical and Mechanical works and all other  accessories and facilities required to make it complete in all respects on turnkey basis and operation and maintenance of plant for Five years in Nigahi Phase0IV project of NCL

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was  Ultra Tech Cement, OPC, 43-grade IS : 269, W-17, M-04,Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.  Admixture used Fosroc Conplast SP 430 G8.                                          .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./163 /2022-23/ 	                                                  Dated: 26.07.2022

#### (Page – 2(i))

#### TEST REPORTS

Recommended Mix Proportion (i)  M-15 grade with admixture, (ii)  M-20 grade with admixture, (iii) M-25 grade

with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-15 grade | 0.60 : 1 : 2.45 : 1.73 : 2.70 : 0.7* 0.84 : 1 : 1.93 : 1.52 : 2.16 : 0.7* | By weight By volume |
| 2. | M-20 grade | 0.55 : 1 : 2.22 : 1.57 : 2.45 : 0.7* 0.77 : 1 : 1.75 : 1.38 : 1.96 : 0.7* | By weight By volume |
| 3. | M-25 grade | 0.50 : 1 : 1.99 : 1.40 : 2.19 : 0.7* 0.70 : 1 : 1.57 : 1.23 : 1.75 : 0.7* | By weight By volume |

| Sl. No. | Test | M-15 grade with admixture | M-20 grade with admixture | M-25 grade with admixture |
| --- | --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 19.95 | 26.6 | 31.6 |
| 2 | Slump range (mm) | 50-100 | 50-100 | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 90.0 | 85.0 | 80.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 19.5 25.0 | 22.0 30.0 | 27.5 36.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Fosroc Conplast SP 430 G8 (Litre) | 180.0 300.0 735.19 519.05 810.68 2.10 | 180.0 327.0 726.29 512.77 800.87 2.289 | 180.0 360.0 715.41 505.08 788.87 2.52 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./163 /2022-23/ 	                                                  Dated: 26.07.2022

#### (Page – 2(ii))

#### TEST REPORTS

Recommended Mix Proportion (iv) M-30 grade with admixture, (v)  M-35 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-30 grade | 0.45 : 1 : 1.76 : 1.24 : 1.94 : 0.7* 0.63 : 1 : 1.38 : 1.09 : 1.55 : 0.7* | By weight By volume |
| 2. | M-35 grade | 0.40 : 1 : 1.52 : 1.08 : 1.68 : 0.7* 0.56 : 1 : 1.20 : 0.95 : 1.34 : 0.7* | By weight By volume |

| Sl. No. | Test | M-30 grade with admixture | M-35 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 38.25 | 43.25 |
| 2 | Slump range (mm) | 50-100 | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 75.0 | 65.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 31.5 43.0 | 35.5 46.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Fosroc Conplast SP 430 G8 (Litre) | 180.0 400.0 702.22 495.77 774.32 2.80 | 180.0 450.0 685.73 484.13 756.14 3.15 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./163 /2022-23/ 	                                                  Dated: 26.07.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design (i)  M-15 grade with admixture,

(ii)  M-20 grade with admixture, (iii) M-25 grade with admixture, (iv) M-30 grade with admixture, (v)  M-35 grade

with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 88.05 | 100.0 | - |
|  | 10 mm | 5.45 | 91.05 | - |
|  | 4.75 mm | 0.05 | 5.90 | 1.8 |
|  | 2.36 mm | - | 0.40 | 6.8 |
|  | 1.18 mm | - | 0.05 | 20.2 |
|  | 600 micron | - | - | 55.6 |
|  | 300 micron | - | - | 93.0 |
|  | 150 micron | - | - | 99.2 |
| 2. | Fineness Modulus | 7.065 | 6.026 | 2.766 |
| 3. | Bulk Density (Kg./Litre) | 1.75 | 1.59 | 1.78 |
| 4. | Specific Gravity | 3.03 | 2.91 | 2.69 |
| 5. | Water absorption | 0.25 % | 0.35 % | 1.32% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./163 /2022-23/ 	                                                  Dated: 26.07.2022

#### TEST REPORTS

Tested for 	:      	General Manager (Civil)

Nigahi Project

Northern Coalfields Limited

Post-Nigahi Project, Distt. Singrauli, M.P.-486884

(M/s Madhucon Projects Limited)

Ref.		:    		NGH/Civil/22-23/93    Dated 09-05-2022

Details of Sample:	Cement: Ultra Tech Cement, OPC, 43-grade IS : 269, W-17, M-04,Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design  (i)  M-15 grade with admixture,

(ii)  M-20 grade with admixture, (iii) M-25 grade with admixture, (iv) M-30 grade with admixture, (v)  M-35 grade with admixture

______________________________________________________________________________

Sub :  Design of  M15, M20, M25, M30 & M35 grade

Name of work : Planning, design, Engineering Constructions, fabrication, Supply, Installation, Testing Trial Run and commissioning of Coal Handling Plant of  10  MTPA consisting of all Civil Structural, Electrical and Mechanical works and all other  accessories and facilities required to make it complete in all respects on turnkey basis and operation and maintenance of plant for Five years in Nigahi Phase0IV project of NCL

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3% 31 % 1.0 mm Expansion 85 Minutes 230 Minutes 27.2 N/mm2 37.6 N/mm2 47.3 N/mm2 1.40 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./19(ii) /2022-23/ 	                                                  Dated: 26.07.2022

#### TEST REPORTS

Tested for 	:      	M/S SHREE BALA JI INFRATECH

C/o Ramji Lal S/O Puji Lal 62/6, Labour Colony

Saharanpur, U.P. - 247001

Site office: D-Plant, ATPP, Anpara

Distt. Sonbhadra-231225

Ref.		:    	Nil   Dated : 05-03-2022

Tax Invoice No. : 	43  IIT BHU dated 07.04.2022

Details of Sample :      	Concrete Mix Design

Test Required       	(i)  M-30 grade with admixture with OPC Cement

__________________________________________________________________________

Work: Design mix.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was  Ultra Tech Cement, OPC 43-grade, IS:269,  Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.  Admixture used Auramix 400.                                          .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./19(ii) /2022-23/ 	                                                  Dated: 26.07.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion M-30 grade with admixture with OPC Cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-30 grade with OPC | 0.45 : 1 : 1.90 : 1.27 : 1.91 : 0.75* 0.62 : 1 : 1.46 : 1.08 : 1.54 : 0.75* | By weight By volume |

| Sl. No. | Test | M-30 grade with admixture with OPC Cement |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 38.25 |
| 2 | Slump range (mm) | 125-175 |
| 3 | Slump Achieved in the Laboratory (mm) | 170.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2 | 31.0 42.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Auramix 400 (Litre) | 170.0 380.0 723.0 483.0 724.0 2.85 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./19(ii) /2022-23/ 	                                                  Dated: 26.07.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design (i) M-30 grade with admixture with

OPC Cement

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm (Source: Chitrangi Singrauli, MP) | CA—10 mm Source: Chitrangi Singrauli, MP) | FA (sand) (Source : Rer, Garra, Singrauli, MP) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 87.65 | 100.0 | - |
|  | 10 mm | 0.50 | 86.15 | - |
|  | 4.75 mm | 0.05 | 3.25 | 3.4 |
|  | 2.36 mm | - | 0.55 | 10.2 |
|  | 1.18 mm | - | 0.05 | 31.6 |
|  | 600 micron | - | - | 73.4 |
|  | 300 micron | - | - | 97.0 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.118 | 6.100 | 3.146 |
| 3. | Bulk Density (Kg./Litre) | 1.71 | 1.63 | 1.80 |
| 4. | Specific Gravity | 2.85 | 2.85 | 2.67 |
| 5. | Water absorption | 0.45 % | 0.40 % | 1.18% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./19(ii) /2022-23/ 	                                                  Dated: 26.07.2022

#### TEST REPORTS

Tested for 	:      	M/S SHREE BALA JI INFRATECH

C/o Ramji Lal S/O Puji Lal 62/6, Labour Colony

Saharanpur, U.P. - 247001

Site office: D-Plant, ATPP, Anpara

Distt. Sonbhadra-231225

Ref.		:    	Nil   Dated : 05-03-2022

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grade, IS:269, Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design  (i) M-30 grade with admixture with OPC Cement

______________________________________________________________________________

Work: Design mix.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3% 31 % 1.0 mm Expansion 95 Minutes 225 Minutes 27.2 N/mm2 37.6 N/mm2 46.7 N/mm2 1.38 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./19(i) /2022-23/ 	                                                  Dated: 26.07.2022

#### TEST REPORTS

Tested for 	:      	M/S SHREE BALA JI INFRATECH

C/o Ramji Lal S/O Puji Lal 62/6, Labour Colony

Saharanpur, U.P. - 247001

Site office: D-Plant, ATPP, Anpara

Distt. Sonbhadra-231225

Ref.		:    	Nil   Dated : 05-03-2022

Tax Invoice No. : 	43  IIT BHU dated 07.04.2022

Details of Sample :      	Concrete Mix Design

Test Required       	 (i) M-25 grade with admixture with PPC Cement

(ii) M-25 grade with admixture with PPC Cement for Piling

(iii) M-30 grade with admixture with PPC Cement

__________________________________________________________________________

Work: Design mix.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was  Ultra Tech Cement, PPC, IS :1489, W-12, M-03, Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used Auramix 400.                                          .

.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./19(i) /2022-23/ 	                                                  Dated: 26.07.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion (i) M-25 grade with admixture with PPC Cement (ii) M-25 grade with admixture with

PPC Cement for Piling (iii) M-30 grade with admixture with PPC Cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 grade with PPC | 0.48 : 1 : 2.11 : 1.38 : 2.07 : 0.6* 0.634 : 1 : 1.55 : 1.12 : 1.60 : 0.6* | By weight By volume |
| 2. | M-25 grade with PPC for Piling | 0.45 : 1 : 1.75 : 1.17 : 1.75 : 0.65* 0.594 : 1 : 1.28 : 0.95 : 1.35 : 0.65* | By weight By volume |
| 3. | M-30 grade with PPC | 0.45 : 1 : 1.89 : 1.27 : 1.90 : 0.7* 0.594 : 1 : 1.39 : 1.03 : 1.47 : 0.7* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture with PPC Cement | M-25 grade with admixture with PPC Cement For Piling | M-30 grade with admixture with PPC cement |
| --- | --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 31.6 | 31.6 | 38.25 |
| 2 | Slump range (mm) | 125-175 | 130-180 | 125-175 |
| 3 | Slump Achieved in the Laboratory (mm) | 170.0 | 170.0 | 170.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 25.0 34.0 | 25.0 34.5 | 30.0 41.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Auramix 400 (Litre) | 168.0 350.0 739.0 483.0 724.0 2.10 | 180.0 400.0 700.0 468.0 700.0 2.60 | 170.0 378.0 715.0 480.0 718.0 2.64 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./19(i) /2022-23/ 	                                                  Dated: 26.07.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design (i) M-25 grade with admixture with

PPC Cement (ii) M-25 grade with admixture with PPC Cement for Piling (iii) M-30 grade with admixture with PPC Cement

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm (Source: Chitrangi Singrauli, MP) | CA—10 mm Source: Chitrangi Singrauli, MP) | FA (sand) (Source : Rer, Garra, Singrauli, MP) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 87.65 | 100.0 | - |
|  | 10 mm | 0.50 | 86.15 | - |
|  | 4.75 mm | 0.05 | 3.25 | 3.4 |
|  | 2.36 mm | - | 0.55 | 10.2 |
|  | 1.18 mm | - | 0.05 | 31.6 |
|  | 600 micron | - | - | 73.4 |
|  | 300 micron | - | - | 97.0 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.118 | 6.100 | 3.146 |
| 3. | Bulk Density (Kg./Litre) | 1.71 | 1.63 | 1.80 |
| 4. | Specific Gravity | 2.85 | 2.85 | 2.67 |
| 5. | Water absorption | 0.45 % | 0.40 % | 1.18% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./19(i) /2022-23/ 	                                                  Dated: 26.07.2022

#### TEST REPORTS

Tested for 	:      	M/S SHREE BALA JI INFRATECH

C/o Ramji Lal S/O Puji Lal 62/6, Labour Colony

Saharanpur, U.P. - 247001

Site office: D-Plant, ATPP, Anpara

Distt. Sonbhadra-231225

Ref.		:    	Nil   Dated : 05-03-2022

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS :1489, W-12, M-03, Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design  (i) M-25 grade with admixture with

PPC Cement (ii) M-25 grade with admixture with PPC Cement for Piling (iii) M-30 grade with admixture with PPC Cement

______________________________________________________________________________

Work: Design mix.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6% 36 % 1.5 mm Expansion 145 Minutes 355 Minutes 26.4 N/mm2 35.9 N/mm2 44.0 N/mm2 1.32 2.92 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./133(i)/2022-23/                                                           Dated: 20.07.2022

#### TEST REPORTS

Tested for 	:      	Project Manager

M/s Vijay Construction (India) Pvt. Ltd.

407-B, Fourth Floor, Cyber Height, Vibhuti Khand,

Gomti Nagar, Lucknow-226010

Ref.		:    		VCIPL/LKO/NH-24A/2022/965   Dated 15-05-2022

Tax Invoice No. : 	327 IIT BHU dated 27.05.2022

Details of Sample :       Concrete Mix Design.

Test Required       	  (i) M-35 grade for structure with Ultra Tech OPC Cement

(ii) M-35 grade for structure with Prism OPC Cement

(iii) M-35 grade for Pile with Ultra Tech OPC Cement

(iv) M-35 grade for Pile with Prism OPC Cement

______________________________________________________________________________

Sub :  Construction of  Four Lane Elevated Viaduct from Km. 4+634 to Km 6+345 on NH-24A including retaining wall, service lane, at  grade road and improvements of junctions in the state of Uttar Pradesh on EPC Mode / Third Party testing of Aggregates, steel cement and Mix design of Concrete RCC-M35 grade for structure and RCC-M35 for Pile-reg.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was (i)Ultra Tech Cement, OPC 43-grade, IS : 269, W-17, M-04, Y-2022 (ii) Prism Cement, OPC 43-grade, IS : 269, W-18, M-05, Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used Armix HyyeCrete PC 20.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./133(i)/2022-23/                                                           Dated: 20.07.2022

#### (Page – 2(i))

#### TEST REPORTS

Recommended Mix Proportion for   (i) M-35 grade for structure with Ultra Tech OPC Cement   (ii) M-35 grade for structure with Prism OPC Cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-35 Grade with Ultra Tech | 0.40 : 1 : 1.51 : 1.05 : 1.57 : 0.5* 0.552 : 1 : 1.16 : 0.93 : 1.28 : 0.5* | By weight By volume |
| 2. | M-35 Grade with Prism | 0.40 : 1 : 1.51 : 1.05 : 1.57 : 0.5* 0.544 : 1 : 1.14 : 0.92 : 1.26 : 0.5* | By weight By volume |

| Sl. No. | Test | M-35 grade for structure with Ultra Tech OPC Cement | M-35 grade for structure with Prism OPC Cement |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 43.25 | 43.25 |
| 2 | Slump range (mm) | 100-150 | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 140.0 | 145.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 36.5 45.5 | 35.5 45.0 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Armix HyyeCrete PC 20 (Litre) | 168.0 420.0 633.0 440.0 660.0 2.10 | 168.0 420.0 633.0 440.0 660.0 2.10 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./133(i)/2022-23/                                                           Dated: 20.07.2022

#### (Page – 2(ii))

#### TEST REPORTS

Recommended Mix Proportion for   (iii) M-35 grade for Pile with Ultra Tech OPC Cement   (iv) M-35 grade for Pile with Prism OPC Cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-35 Grade with Ultra Tech | 0.40 : 1 : 1.47 : 1.02 : 1.53 : 0.5* 0.552 : 1 : 1.13 : 0.90 : 1.25 : 0.5* | By weight By volume |
| 2. | M-35 Grade with Prism | 0.40 : 1 : 1.47 : 1.02 : 1.53 : 0.5* 0.544 : 1 : 1.11 : 0.89 : 1.23 : 0.5* | By weight By volume |

| Sl. No. | Test | M-35 grade for Pile with Ultra Tech OPC Cement | M-35 grade for Pile with Prism OPC Cement |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 43.25 | 43.25 |
| 2 | Slump range (mm) | 150-200 | 150-200 |
| 3 | Slump Achieved in the Laboratory (mm) | 175.0 | 190.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 36.0 45.0 | 35.0 44.5 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Armix HyyeCrete PC 20 (Litre) | 171.0 428.0 628.0 436.0 653.0 2.14 | 171.0 428.0 628.0 436.0 653.0 2.14 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./133(i)/2022-23/                                                           Dated: 20.07.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design for (i) M-35 grade for structure with Ultra Tech OPC Cement  (ii) M-35 grade for structure with Prism OPC Cement  (iii) M-35 grade for Pile with Ultra Tech OPC Cement  (iv) M-35 grade for Pile with Prism OPC Cement

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.3 | 100.0 | - |
|  | 10 mm | 0.40 | 87.85 | - |
|  | 4.75 mm | 0.05 | 7.40 | 4.2 |
|  | 2.36 mm | - | 0.30 | 16.0 |
|  | 1.18 mm | - | 0.05 | 40.0 |
|  | 600 micron | - | - | 71.2 |
|  | 300 micron | - | - | 95.0 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 7.133 | 6.044 | 3.252 |
| 3. | Bulk Density (Kg./Litre) | 1.69 | 1.56 | 1.80 |
| 4. | Specific Gravity | 2.78 | 2.76 | 2.63 |
| 5. | Water absorption | 0.20 % | 0.30% | 1.39% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./133(i)/2022-23/                                                           Dated: 20.07.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Project Manager

M/s Vijay Construction (India) Pvt. Ltd.

407-B, Fourth Floor, Cyber Height, Vibhuti Khand,

Gomti Nagar, Lucknow-226010

Ref.		:    		VCIPL/LKO/NH-24A/2022/965   Dated 15-05-2022

Tax Invoice No. : 	327 IIT BHU dated 27.05.2022

Details of Sample:	Cement:  (i) Ultra Tech Cement, OPC 43-grade, IS : 269, W-17, M-04, Y-2022

(ii) Prism Cement, OPC 43-grade, IS : 269, W-18, M-05, Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i) M-35 grade for structure with Ultra Tech OPC Cement  (ii) M-35 grade for structure with Prism OPC Cement  (iii) M-35 grade for Pile with Ultra Tech OPC Cement  (iv) M-35 grade for Pile with Prism OPC Cement

_____________________________________________________________________________

Sub :  Construction of  Four Lane Elevated Viaduct from Km. 4+634 to Km 6+345 on NH-24A including retaining wall, service lane, at  grade road and improvements of junctions in the state of Uttar Pradesh on EPC Mode / Third Party testing of Aggregates, steel cement and Mix design of Concrete RCC-M35 grade for structure and RCC-M35 for Pile-reg.

| Sl.No. | Tests | Results |  |
| --- | --- | --- | --- |
|  |  | Ultra Tech Cement, OPC 43-grade,IS: 269, W-17, M-04, Y-2022 | Prism Cement, OPC 43-grade, IS : 269, W-18, M-05, Y-2022 |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 2 % 31 % 1.0 mm Expansion 90 Minutes 275 Minutes 26.4 N/mm2 36.1 N/mm2 46.5 N/mm2 1.38 3.10 | 3 % 34 % 1.0 mm Expansion 95 Minutes 295 Minutes 25.7 N/mm2 35.6 N/mm2 45.9 N/mm2 1.36 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./40/2022-23/                                                              Dated: 15.07.2022

#### TEST REPORTS

Tested for 	:      	Executive Engineer

Construction Division

U.P. Jal Nigam (Rural)

Prahaladpur Kasganj

(M/s Power Mech Projects Ltd.)

Ref.		:    		174/ VsfLVxa ,Vk /03   Dated: 21.03.2022

Tax Invoice No. : 	106 IIT BHU dated 20.04.2022

Details of Sample :      Concrete Mix Design.

Test Required       	  (i) M-25 grade with admixture

(ii) M-30 grade with admixture

________________________________________________________________________

Sub :  Testing of construction material  under Jal Jeevan Mission of Agreement No. 112/ED/2021-22 dated 17.08.2021

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 43-grade, IS : 269, W-18, Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used Conplast SP 430.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./40/2022-23/                                                              Dated: 15.07.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for (i) M-25 grade with admixture (ii) M-30 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.48 : 1 : 2.32 : 1.69 : 2.56 : 1.0* 0.677 : 1 : 1.86 : 1.47 : 2.09 : 1.0* | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 2.20 : 1.58 : 2.42 : 1.0* 0.635 : 1 : 1.76 : 1.38 : 1.97 : 1.0* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture | M-30 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 31.6 | 38.25 |
| 2 | Slump range (mm) | 50-100 | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 70.0 | 65.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) i) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 25.5 32.0 | 32.5 40.0 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Conplast SP 430 (Litre) | 154.0 321.0 745.0 542.9 822.0 3.21 | 154.0 342.0 753.2 540.0 828.9 3.42 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./40/2022-23/                                                              Dated: 15.07.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design for (i) M-25 grade with

admixture (ii) M-30 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 92.0 | 100.0 | - |
|  | 10 mm | 0.80 | 88.7 | - |
|  | 4.75 mm | 0.05 | 1.30 | 3.2 |
|  | 2.36 mm | - | 0.35 | 13.0 |
|  | 1.18 mm | - | 0.05 | 39.0 |
|  | 600 micron | - | - | 74.8 |
|  | 300 micron | - | - | 96.4 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 7.072 | 6.096 | 3.252 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.62 | 1.76 |
| 4. | Specific Gravity | 3.07 | 3.00 | 2.69 |
| 5. | Water absorption | 0.20 % | 0.35% | 1.85% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./40/2022-23/                                                              Dated: 15.07.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Executive Engineer

Construction Division

U.P. Jal Nigam (Rural)

Prahaladpur Kasganj

(M/s Power Mech Projects Ltd.)

Ref.		:    		174/ VsfLVxa ,Vk /03   Dated: 21.03.2022

Details of Sample:	Cement:  Ultra Tech Cement, OPC 43-grade, IS : 269, W-18, Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i) M-25 grade with

admixture (ii) M-30 grade with admixture

______________________________________________________________________________

Sub :  Testing of construction material  under Jal Jeevan Mission of Agreement No. 112/ED/2021-22 dated 17.08.2021

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 31 % 1.0 mm Expansion 85 Minutes 285 Minutes 26.7 N/mm2 36.4 N/mm2 46.7 N/mm2 1.41 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./70/2022-23/                                                              Dated: 08.07.2022

#### TEST REPORTS

Tested for 	:      	AGM (FQA & Safety)

Kanti Bijlee Utpadan Nigam Limited

P.O. Kanti thermal,  Distt.- Muzaffapur-843130 Bihar

(M/s VASU CHEMICALS LLP)

Ref.		:    		KBUNL/FQA/2022/51   Dated : 31.03.2022

Tax Invoice No. : 	156 IIT BHU dated 02.05.2022

Details of Sample :      Concrete Mix Design.

Test Required       	  (i) M-25 grade with admixture

(ii) M-30 grade with admixture for Pile

________________________________________________________________________

Sub :  Mix Design.

Work:   Supply cum Installation of Chlorine DI-Oxide (CLO2) Plant, Civil Works Package for Muzaffarpur Thermal Power Project.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC. IS : 1489, W-09, M-03, Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used SikaCim.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./70/2022-23/                                                               Dated: 08.07.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for (i) M-25 grade with admixture (ii) M-30 grade with admixture for

Pile

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 1.77 : 1.38 : 2.06 : 0.2* 0.63 : 1 : 1.25 : 1.09 : 1.55 : 0.2* | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 1.57 : 1.37 : 1.67 : 1.0* 0.567 : 1 : 1.11 : 1.09 : 1.25 : 1.0* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture | M-30 grade with admixture for Pile |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 |
| 2 | Slump range (mm) | 50-100 | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 80.0 | 140.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) i) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 25.0 34.5 | 30.0 41.5 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) SikaCim (Litre) | 178.0 356.0 631.6 490.9 733.8 0.712 | 180.0 400.0 628.0 549.0 668.8 4.0 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./70/2022-23/                                                       Dated: 08.07.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design for (i) M-25 grade with

admixture (ii) M-30 grade with admixture for Pile

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 94.05 | 100.0 | - |
|  | 10 mm | 0.40 | 75.25 | - |
|  | 4.75 mm | 0.05 | 0.95 | 2.2 |
|  | 2.36 mm | - | 0.30 | 5.2 |
|  | 1.18 mm | - | 0.05 | 13.8 |
|  | 600 micron | - | - | 36.4 |
|  | 300 micron | - | - | 80.8 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 7.055 | 6.235 | 2.372 |
| 3. | Bulk Density (Kg./Litre) | 1.68 | 1.59 | 1.78 |
| 4. | Specific Gravity | 2.81 | 2.82 | 2.58 |
| 5. | Water absorption | 1.73 % | 1.83% | 1.73% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./70/2022-23/                                                            Dated: 08.07.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	AGM (FQA & Safety)

Kanti Bijlee Utpadan Nigam Limited

P.O. Kanti thermal,  Distt.- Muzaffapur-843130 Bihar

(M/s VASU CHEMICALS LLP)

Ref.		:    		KBUNL/FQA/2022/51   Dated : 31.03.2022

Details of Sample:	Cement:  Ultra Tech Cement, PPC. IS : 1489, W-09, M-03, Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i) M-25 grade with

admixture (ii) M-30 grade with admixture for Pile

______________________________________________________________________________

Sub :  Mix Design.

Work:   Supply cum Installation of Chlorine DI-Oxide (CLO2) Plant, Civil Works Package for Muzaffarpur Thermal Power Project.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 145 Minutes 360 Minutes 23.6 N/mm2 33.6 N/mm2 43.6 N/mm2 1.26 2.86 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./143/2022-23/                                                      Dated: 06.07.2022

#### TEST REPORTS

Tested for 	:      	Executive Engineer

Varanasi Vikas Pradhikaran

Varanasi

(M/s  K K Construction and Builders)

Ref.			:    	 52 / fo0izk0 / fu0 /2022-23    Dated 18.04.2022

Tax Invoice No. : 	 363 IIT BHU dated 10.06.2022

Details of Sample :      	Concrete Mix Design.

Test Required       	 M-25 grade

________________________________________________________________________

Sub: Mix design M-25 grade  for Sarnath Buddhist Circuit Project under Vishwa Bank Funded U.P. Pro-Puar Development Project.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC. IS : 1489, W-14, M-04, Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./143/2022-23/                                                      Dated: 06.07.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M25 Grade | 0.50 : 1 : 1.74 : 1.06 : 1.63 0.645 : 1 : 1.25 : 0.91 : 1.34 | By weight By volume |

| Sl. No. | Test | M-25 grade |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (mm) | 100-125 |
| 3 | Slump Achieved in the Laboratory (mm) | 105.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 25.5 35.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 198.0 396.0 689.0 418.0 646.0 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./143/2022-23/                                                      Dated: 06.07.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design for M-25 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm ( | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 91.75 | 100.0 | - |
|  | 10 mm | 0.55 | 87.4 | - |
|  | 4.75 mm | 0.05 | 0.90 | 2.8 |
|  | 2.36 mm | - | 0.45 | 9.0 |
|  | 1.18 mm | - | 0.05 | 22.8 |
|  | 600 micron | - | - | 50.0 |
|  | 300 micron | - | - | 88.4 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.077 | 6.112 | 2.720 |
| 3. | Bulk Density (Kg./Litre) | 1.57 | 1.50 | 1.80 |
| 4. | Specific Gravity | 2.63 | 2.55 | 2.76 |
| 5. | Water absorption | 1.36 % | 1.75% | 1.22% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./143/2022-23/                                                      Dated: 06.07.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Executive Engineer

Varanasi Vikas Pradhikaran

Varanasi

(M/s  K K Construction and Builders)

Ref.			:    	 52 / fo0izk0 / fu0 /2022-23    Dated 18.04.2022

Details of Sample:	Cement:  Ultra Tech Cement, PPC. IS : 1489, W-14, M-04, Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade

______________________________________________________________________________

Sub: Mix design M-25 grade  for Sarnath Buddhist Circuit Project under Vishwa Bank Funded U.P. Pro-Puar Development Project.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 145 Minutes 365Minutes 24.3 N/mm2 34.1 N/mm2 44.4 N/mm2 1.29 2.89 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./66 /2022-23/ 	                                                  Dated: 05.07.2022

#### TEST REPORTS

Tested for 	:      	Upper Project Manager

U.P.R.N. N. Ltd.,

Unit-19, Plot No. 4, Sector-7

Gomti Nagar Extension, Lucknow-226002

Ref.		:    		291 /  vi0 ifj0izCk0/bdkÃ-19 /U;wjks lkbUl / jkfufu / 22  Dated:11-04-2022

Tax Invoice No. : 	152  IIT BHU dated 02.05.2022

Details of Sample :      	Concrete Mix Design

Test Required       	 M-25 grade with Plasticizer

________________________________________________________________________

Sub.: Design of Cement Concrete Mix M-25 with Plasticizer.

Name of work: Construction of Advance Neuron Sciences Centre at Dr. Ram Manohar Lohia Aayurvigyan Sansthan, Lucknow

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Birla Cement, PPC, IS : 1489,  Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used Fosroc Conplast SP 430 G8.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./66 /2022-23/ 	                                                  Dated: 05.07.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion M-25 grade with Plasticizer

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 grade | 0.57 : 1 : 1.98 : 1.23 : 2.19 : 0.80* 0.741 : 1 : 1.45 : 0.99 : 1.63 : 0.80* | By weight By volume |

| Sl. No. | Test | M-25 grade with Plasticizer |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (mm) | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 140.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 24.5 34.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Fosroc Conplast SP 430 G8 (Litre) | 197.0 345.6 684.3 425.1 756.9 2.77 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./66 /2022-23/ 	                                                  Dated: 05.07.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade with

Plasticizer

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.4 | 100.0 | - |
|  | 10 mm | 0.30 | 86.65 | - |
|  | 4.75 mm | 0.05 | 5.40 | 1.8 |
|  | 2.36 mm | - | 0.45 | 5.8 |
|  | 1.18 mm | - | 0.05 | 16.2 |
|  | 600 micron | - | - | 42.8 |
|  | 300 micron | - | - | 92.0 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 7.133 | 6.075 | 2.574 |
| 3. | Bulk Density (Kg./Litre) | 1.75 | 1.61 | 1.78 |
| 4. | Specific Gravity | 2.78 | 2.85 | 2.65 |
| 5. | Water absorption | 0.25 % | 0.20 % | 1.59% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING  LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./66 /2022-23/ 	                                                  Dated: 05.07.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Upper Project Manager

U.P.R.N. N. Ltd.,

Unit-19, Plot No. 4, Sector-7

Gomti Nagar Extension, Lucknow-226002

Ref.		:    		291 /  vi0 ifj0izCk0/bdkÃ-19 /U;wjks lkbUl / jkfufu / 22  Dated:11-04-2022

Details of Sample:	Cement: Birla Cement, PPC, IS : 1489,  Y-2022.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with Plasticizer

______________________________________________________________________________

Sub.: Design of Cement Concrete Mix M-25 with Plasticizer.

Name of work: Construction of Advance Neuron Sciences Centre at Dr. Ram Manohar Lohia Aayurvigyan Sansthan, Lucknow

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 36 % 1.5 mm Expansion 140 Minutes 360 Minutes 24.1 N/mm2 34.7 N/mm2 44.1 N/mm2 1.30 2.90 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. 										Dated:

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design

| S.No | Test | R E S U L T |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) | Cement |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |  |
|  | 40 mm |  |  |  |  |
|  | 20 mm |  |  |  |  |
|  | 10 mm |  |  |  |  |
|  | 4.75 mm |  |  |  |  |
|  | 2.36 mm |  |  |  |  |
|  | 1.18 mm |  |  |  |  |
|  | 600 micron |  |  |  |  |
|  | 300 micron |  |  |  |  |
|  | 150 micron |  |  |  |  |
| 2. | Fineness Modulus |  |  |  |  |
| 3. | Bulk Density (Kg./Litre) |  |  |  |  |
| 4. | Specific Gravity |  |  |  |  |
| 5. | Water absorption |  |  |  |  |

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./73/2022-23/                                                             Dated: 01.07.2022

#### TEST REPORTS

Tested for 	:      	Assistant  Engineer

Lucknow Subdivision-IV

CPWD, Lucknow

(M/s Tempcon Engineers LLP )

Ref.		:    	23(CAT)/ y0miea0-IV/77  Dated : 21-04-2022

Tax Invoice No. : 	181 IIT BHU dated 05.05.2022

Details of Sample :       Concrete Mix Design.

Test Required       	  M-25 grade with admixture

________________________________________________________________________

Sub :  Design Mix of M-25 grade  pumpable concrete

Work: Construction of  office building for Central administrative Tribunal (CAT), Lucknow, bench including Internal electrical installation, fire  fighting system, fire  alarm VRV/VRF system, EPBAX system, DG set, Sub Station, lifts water supply and horticulture work at Gomti Nagar Luknow

Agency:  M/s Tempcon Engineers LLP

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489,  Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used Fosroc Conplast SP 430 G8.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./73/2022-23/                                                             Dated: 01.07.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.03 : 1.36 : 2.04 : 1.0* 0.65 : 1 : 1.47 : 1.14 : 1.62 : 1.0* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.25 |
| 2 | Slump range (mm) | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 130.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 25.0 35.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Fosroc Conplast SP 430 G8 (Litre) | 172.0 344.0 697.0 468.5 703.0 3.44 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./73/2022-23/                                                             Dated: 01.07.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade with

admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 85.8 | 100.0 | - |
|  | 10 mm | 0.10 | 86.2 | - |
|  | 4.75 mm | 0.05 | 0.40 | 4.2 |
|  | 2.36 mm | - | 0.20 | 18.0 |
|  | 1.18 mm | - | 0.10 | 39.2 |
|  | 600 micron | - | - | 65.2 |
|  | 300 micron | - | - | 92.4 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.141 | 6.131 | 3.180 |
| 3. | Bulk Density (Kg./Litre) | 1.64 | 1.55 | 1.79 |
| 4. | Specific Gravity | 2.71 | 2.66 | 2.63 |
| 5. | Water absorption | 0.35 % | 0.50 % | 1.80% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING  LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./73/2022-23/                                                             Dated: 01.07.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Assistant  Engineer

Lucknow Subdivision-IV

CPWD, Lucknow

(M/s Tempcon Engineers LLP )

Ref.		:    	23(CAT)/ y0miea0-IV/77  Dated : 21-04-2022

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489,  Y-2022.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with

admixture

______________________________________________________________________________

Sub :  Design Mix of M-25 grade  pumpable concrete

Work: Construction of  office building for Central administrative Tribunal (CAT), Lucknow, bench including Internal electrical installation, fire  fighting system, fire  alarm VRV/VRF system, EPBAX system, DG set, Sub Station, lifts water supply and horticulture work at Gomti Nagar Luknow

Agency:  M/s Tempcon Engineers LLP

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 145 Minutes 340 Minutes 24.4 N/mm2 34.4 N/mm2 44.0 N/mm2 1.30 2.90 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./607/2022-23/                                                      Dated: 20.05.2022

#### TEST REPORTS

Tested for 	:      	Executive Engineer

Mirzapur Nahar Prakhand

Mirzapur

Ref.			:    	194 /  eh0u0iz0eh0  Dated: 19.01.2022

Tax Invoice No. : 	 1448 IIT BHU dated 09.03.2022

Details of Sample :      	Concrete Mix Design.

Test Required       	 M-20 grade and M-25 grade

________________________________________________________________________

Sub: M20 and M25 grade concrete Mix Design

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC-43 grade. IS : 269,  Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./607/2022-23/                                                      Dated: 20.05.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-20 grade and M-25 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M20 Grade | 0.55 : 1 : 2.11 : 1.31 : 1.96 0.759 : 1 : 1.63 : 1.10 : 1.56 | By weight By volume |
| 2. | M25 Grade | 0.50 : 1 : 1.84 : 1.19 : 1.78 0.69 : 1 : 1.42 : 1.00 : 1.42 | By weight By volume |

| Sl. No. | Test | M-20 grade | M-25 grade |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 27.6 | 33.7 |
| 2 | Slump range (mm) | 75-125 | 75-125 |
| 3 | Slump Achieved in the Laboratory (mm) | 115.0 | 110.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 24.0 33.0 | 30.0 37.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 192.5 350.0 739.0 459.0 686.0 | 192.5 385.0 708.0 458.0 685.0 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./607/2022-23/                                                      Dated: 20.05.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design for M-20 grade and       M-25 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm ( | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 87.05 | 100.0 | - |
|  | 10 mm | 0.40 | 86.05 | - |
|  | 4.75 mm | 0.03 | 6.90 | 2.2 |
|  | 2.36 mm | - | 0.40 | 4.8 |
|  | 1.18 mm | - | 0.30 | 15.4 |
|  | 600 micron | - | - | 52.8 |
|  | 300 micron | - | - | 93.0 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 7.125 | 6.064 | 2.670 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.65 | 1.79 |
| 4. | Specific Gravity | 2.85 | 2.82 | 2.63 |
| 5. | Water absorption | 0.20 % | 0.40% | 1.53% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./607/2022-23/                                                      Dated: 20.05.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Executive Engineer

Mirzapur Nahar Prakhand

Mirzapur

Ref.			:    	194 /  eh0u0iz0eh0  Dated: 19.01.2022

Details of Sample:	Cement:  Ultra Tech Cement, OPC-43 grade. IS : 269,  Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-20 grade and M-25 grade

______________________________________________________________________________

Sub: M20 and M25 grade concrete Mix Design

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 32 % 1.0 mm Expansion 90 Minutes 280 Minutes 26.4 N/mm2 36.4 N/mm2 46.7 N/mm2 1.38 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./529/2022-23/                                                      Dated: 20.05.2022

#### TEST REPORTS

Tested for 	:      	Mr. U. Ravindra Prasad

AGM (Projects)-Ayodhya

M/s Vishal Infrastructure Limited

18, Sant Nagar Phase-1, Dayal Bagh, Agra, U.P.- 282005

Ref.		:    	Email dated  31.01.2022

Tax Invoice No. : 	1307 IIT BHU dated 10.02.2022

Details of Sample :      Concrete Mix Design.

Test Required       	  M-25 grade with admixture

________________________________________________________________________

Sub :  Design Mix of M-25 grade

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC-43 grade. IS : 269, W-04, M-01, Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Sika Plastiment 2001 NS.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./529/2022-23/                                                      Dated: 20.05.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.34 : 1.18 : 2.19 : 0.5* 0.695 : 1 : 1.81 : 0.98 : 1.89 : 0.5* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (mm) | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 70.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 27.5 35.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Sika Plastiment 2001 NS (Litre) | 167.5 335.0 785.0 395.0 734.0 1.675 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./529/2022-23/                                                      Dated: 20.05.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design for M-25 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm ( | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 97.1 | 100.0 | - |
|  | 10 mm | 0.45 | 90.8 | - |
|  | 4.75 mm | 0.05 | 2.60 | 4.8 |
|  | 2.36 mm | - | 0.90 | 24.2 |
|  | 1.18 mm | - | 0.05 | 55.2 |
|  | 600 micron | - | - | 87.2 |
|  | 300 micron | - | - | 98.4 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.024 | 7.057 | 3.688 |
| 3. | Bulk Density (Kg./Litre) | 1.61 | 1.68 | 1.80 |
| 4. | Specific Gravity | 2.64 | 2.72 | 2.68 |
| 5. | Water absorption | 0.30 % | 0.20% | 1.12 % |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./529/2022-23/                                                      Dated: 20.05.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Mr. U. Ravindra Prasad

AGM (Projects)-Ayodhya

M/s Vishal Infrastructure Limited

18, Sant Nagar Phase-1, Dayal Bagh, Agra, U.P.- 282005

Ref.		:    	Email dated  31.01.2022

Details of Sample:	Cement:  Ultra Tech Cement, OPC-43 grade. IS : 269, W-04, M-01, Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with admixture

______________________________________________________________________________

Sub :  Design Mix of M-25 grade

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 31 % 1.0 mm Expansion 120 Minutes 250 Minutes 26.4 N/mm2 37.2 N/mm2 47.2 N/mm2 1.39 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./676/2022-23/                                                          Dated: 17.05.2022

#### TEST REPORTS

Tested for 	:      	M/s Skyline Architectural Consultant

11th Floor, CP-3, Laxmi Vandan Complex,

Patrakarpuram Crossing, Gomti Nagar, Lucknow -226010

(M/s KPC Projects Limited)

Ref.		:    	SAC/01/2022      Dated : 23-02-2022

Tax Invoice No. : 	1571 IIT BHU dated 26.03.2022

Details of Sample :      Concrete Mix Design.

Test Required       	  M-30 grade with admixture and M-40 grade with admixture

________________________________________________________________________

Sub :  Design Mix of M-30 grade & M 40 grade

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was M.P. Birla Concrecem Cement, OPC-43 grade. IS : 269, Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used SmartCare Maximo  Plast PC 200

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./676/2022-23/                                                          Dated: 17.05.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-30 grade with admixture and M-40 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-30 Grade | 0.45 : 1 : 1.92 : 1.25 : 1.87 : 0.5* 0.63 : 1 : 1.50 : 1.08 : 1.50 : 0.5* | By weight By volume |
| 2. | M-40 Grade | 0.38 : 1 : 1.53 : 1.04 : 1.55 : 0.5* 0.532 : 1 : 1.20 : 0.90 : 1.25 : 0.5* | By weight By volume |

| Sl. No. | Test | M-30 grade with admixture | M-40 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 39.9 | 50.9 |
| 2 | Slump range (mm) | 125-175 | 125-175 |
| 3 | Slump Achieved in the Laboratory (mm) | 135.0 | 130.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 33.0 44.0 | 43.5 54.5 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) SmartCare Maximo Plast PC 200 (Litre) | 171.0 380.0 730.0 475.0 711.0 1.90 | 171.0 450.0 689.0 468.0 698.0 2.25 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./676/2022-23/                                                          Dated: 17.05.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design for M-30 grade with

admixture and M-40 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm ( | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 87.15 | 100.0 | - |
|  | 10 mm | 0.65 | 88.4 | - |
|  | 4.75 mm | 0.05 | 6.9 | 2.8 |
|  | 2.36 mm | - | 0.45 | 5.8 |
|  | 1.18 mm | - | 0.05 | 15.8 |
|  | 600 micron | - | - | 54.4 |
|  | 300 micron | - | - | 94.4 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.122 | 6.042 | 2.722 |
| 3. | Bulk Density (Kg./Litre) | 1.74 | 1.62 | 1.79 |
| 4. | Specific Gravity | 2.84 | 2.83 | 2.62 |
| 5. | Water absorption | 0.30 % | 0.40% | 1.46 % |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./676/2022-23/                                                          Dated: 17.05.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	M/s Skyline Architectural Consultant

11th Floor, CP-3, Laxmi Vandan Complex,

Patrakarpuram Crossing, Gomti Nagar, Lucknow -226010

(M/s KPC Projects Limited)

Ref.		:    	SAC/01/2022      Dated : 23-02-2022

Details of Sample:	Cement: M.P. Birla Concrecem Cement, OPC-43 grade. IS : 269, Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-30 grade with

admixture and M-40 grade with admixture

______________________________________________________________________________

Sub :  Design Mix of M-30 grade & M 40 grade

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 30 % 1.0 mm Expansion 115 Minutes 250 Minutes 26.1 N/mm2 27.2 N/mm2 47.2 N/mm2 1.40 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./20/2022-23/                                                          Dated: 04.05.2022

#### TEST REPORTS

Tested for 	:      	Jt. Gen. Manager Engg. (P)

Engineer-In-Charge

Airports Authority of India

Ayodhya Airport

Ref.		:    	AAI/AYO/Jt. GM(P)/PEB/2021-22/83 Dated : 09-03-2022

Tax Invoice No. : 	44 IIT BHU dated 07.04.2022

Details of Sample :      Concrete Mix Design.

Test Required       	  (i)   M-25 grade with admixture with Ultra Tech Cement, OPC

(ii)  M-30 grade with admixture with Ultra Tech Cement, OPC

(iii)  M-40 grade with admixture with Ultra Tech Cement, OPC

(iv)  M-25 grade with admixture with SHREE Cement, OPC

(v)   M-30 grade with admixture with SHREE Cement, OPC

(vi)   M-40 grade with admixture with SHREE Cement, OPC

________________________________________________________________________

Name of work:   Development of Ayodhya airport, Uttar Pradesh-Construction of Pre-Engineering Buildings (PEB)

and other associated works on design & Build Basis (EPC)

SH: Design Mix for M25, M-30 and M-40 grade concrete.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was (i) Ultra Tech Cement, OPC, 43-grade, IS : 269, W-09, M-03, Y-2022

(ii)  SHREE Cement, OPC 43-grade, IS : 269, W-08, M-02, Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Auramix  400

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./20/2022-23/                                                          Dated: 04.05.2022

#### (Page – 2(i))

#### TEST REPORTS

Recommended Mix Proportion for   (i)   M-25 grade with admixture with Ultra Tech Cement, OPC ii)  M-30 grade

with admixture with Ultra Tech Cement, OPC (iii)  M-40 grade with admixture with Ultra Tech Cement, OPC

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.46 : 1.19 : 2.20 : 0.4* 0.695 : 1 : 1.90 : 1.08 : 1.90 : 0.4* | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 2.12 : 1.06 : 1.98 : 0.4* 0.626 : 1 : 1.64 : 0.96 : 1.71 : 0.4* | By weight By volume |
| 3. | M-40 Grade | 0.38 : 1 : 1.69 : 0.89 : 1.64 : 0.45* 0.528 : 1 : 1.31 : 0.81 : 1.42 : 0.45* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture with Ultra Tech Cement, OPC | M-30 grade with admixture with Ultra Tech Cement, OPC | M-40 grade with admixture with Ultra Tech Cement, OPC |
| --- | --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 | 50.9 |
| 2 | Slump range (mm) | 50-100 | 50-100 | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 80.0 | 90.0 | 90.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 30.0 37.5 | 35.0 45.0 | 44.5 55.5 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Auramix 400 (Litre) | 165.0 330.0 812.0 393.0 726.0 1.32 | 165.15 367.0 778.0 389.0 727.0 1.648 | 165.3 435.0 735.0 387.0 713.0 1.958 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./20/2022-23/                                                          Dated: 04.05.2022

#### (Page – 2(ii))

#### TEST REPORTS

Recommended Mix Proportion for   (iv)   M-25 grade with admixture with SHREE Cement, OPC(v)  M-30 grade

with admixture with SHREE Cement, OPC (vi)  M-40 grade with admixture with SHREE Cement, OPC

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.46 : 1.19 : 2.20 : 0.5* 0.675 : 1 : 1.85 : 1.05 : 1.84 : 0.5* | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 2.12 : 1.06 : 1.98 : 0.55* 0.608 : 1 : 1.59 : 0.94 : 1.66 : 0.55* | By weight By volume |
| 3. | M-40 Grade | 0.38 : 1 : 1.69 : 0.89 : 1.64 : 0.6* 0.513 : 1 : 1.27 : 0.79 : 1.38 : 0.6* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture with SHREE Cement, OPC | M-30 grade with admixture with SHREE Cement, OPC | M-40 grade with admixture with SHREE Cement, OPC |
| --- | --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 | 50.9 |
| 2 | Slump range (mm) | 50-100 | 50-100 | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 55.0 | 70.0 | 60.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 29.5 37.0 | 34.0 44.5 | 43.0 55.0 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Auramix 400 (Litre) | 165.0 330.0 812.0 393.0 726.0 1.65 | 165.15 367.0 778.0 389.0 727.0 2.019 | 165.3 435.0 735.0 387.0 713.0 2.61 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./20/2022-23/                                                          Dated: 04.05.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design for   (i)   M-25 grade with admixture with

Ultra Tech Cement, OPC (ii)  M-30 grade with admixture with Ultra Tech Cement, OPC (iii)  M-40 grade with

admixture with Ultra Tech Cement, OPC (iv)   M-25 grade with admixture with SHREE Cement, OPC(v)  M-30 grade

with admixture with SHREE Cement, OPC (vi)  M-40 grade with admixture with SHREE Cement, OPC

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 91.3 | 100.0 | - |
|  | 10 mm | 0.40 | 87.8 | - |
|  | 4.75 mm | 0.05 | 4.4 | 7.0 |
|  | 2.36 mm | - | 0.55 | 21.8 |
|  | 1.18 mm | - | 0.05 | 47.8 |
|  | 600 micron | - | - | 78.2 |
|  | 300 micron | - | - | 97.2 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.083 | 6.072 | 3.510 |
| 3. | Bulk Density (Kg./Litre) | 1.61 | 1.53 | 1.80 |
| 4. | Specific Gravity | 2.69 | 2.67 | 2.69 |
| 5. | Water absorption | 0.20 % | 0.30% | 1.41 % |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./20/2022-23/                                                          Dated: 04.05.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Jt. Gen. Manager Engg. (P)

Engineer-In-Charge

Airports Authority of India

Ayodhya Airport

Ref.		:    	AAI/AYO/Jt. GM(P)/PEB/2021-22/83 Dated : 09-03-2022

Details of Sample:	Cement: (i)   Ultra Tech Cement, OPC, 43-grade, IS : 269, W-09, M-03, Y-2022

(ii)  SHREE Cement, OPC 43-grade, IS : 269, W-08, M-02, Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i)   M-25 grade with admixture with

Ultra Tech Cement, OPC (ii)  M-30 grade with admixture with Ultra Tech Cement, OPC (iii)  M-40 grade with admixture with Ultra Tech Cement, OPC (iv)   M-25 grade with admixture with SHREE Cement, OPC(v)  M-30 grade with admixture with SHREE Cement, OPC (vi)  M-40 grade with admixture with SHREE Cement, OPC

______________________________________________________________________________

Name of work:   Development of Ayodhya airport, Uttar Pradesh-Construction of Pre-Engineering Buildings (PEB)

and other associated works on design & Build Basis (EPC)

SH: Design Mix for M25, M-30 and M-40 grade concrete.

| Sl.No. | Tests | Results |  |
| --- | --- | --- | --- |
|  |  | Ultra Tech Cement, OPC, 43-grade, IS : 269, W-09, M-03, Y-2022 | SHREE Cement, OPC 43-grade, IS : 269, W-08, M-02, Y-2022 |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 31 % 1.0 mm Expansion 110 Minutes 320 Minutes 29.1 N/mm2 36.3 N/mm2 46.4 N/mm2 1.39 3.10 | 4 % 33 % 1.0 mm Expansion 115 Minutes 330 Minutes 28.1 N/mm2 34.8 N/mm2 45.6 N/mm2 1.35 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./669/2022-23/                                                      Dated: 04.05.2022

#### TEST REPORTS

Tested for 	:      	M/s  Angelic Infracon Private Limited

534, MalGodam Road,

Sultanpur-228001

Ref.		:    	181/AIPL/20-21     Dated : 17-02-2022

Tax Invoice No. : 	1562 IIT BHU dated 26.03.2022

Details of Sample :      Concrete Mix Design.

Test Required       	  M-30 grade with admixture

________________________________________________________________________

Sub :  Design Mix of M-30 grade

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489, W-06, M-02, Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Auramix 350 DIS

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./669/2022-23/                                                      Dated: 04.05.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-30 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-30 Grade | 0.45 : 1 : 1.94 : 1.49 : 1.49 : 0.6* 0.581 : 1 : 1.39 : 1.18 : 1.10 : 0.6* | By weight By volume |

| Sl. No. | Test | M-30 Grade with admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 39.9 |
| 2 | Slump range (initial) (mm) | 150-200 |
| 3 | Slump range (after one hour ) (mm) | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 185.0 |
| 5 | Slump Achieved in the Laboratory(after one hour) (mm) | 120.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 31.0 41.5 |
| 7 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Auramix 350 DIS (Litre) | 173.25 385.0 747.0 574.0 574.0 2.31 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./669/2022-23/                                                        Dated: 04.05.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design for  M-30 grade with

Admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm ( | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 87.9 | 100.0 | - |
|  | 10 mm | 0.40 | 86.25 | - |
|  | 4.75 mm | 0.05 | 5.90 | 2.8 |
|  | 2.36 mm | - | 0.45 | 8.0 |
|  | 1.18 mm | - | 0.05 | 24.6 |
|  | 600 micron | - | - | 56.6 |
|  | 300 micron | - | - | 92.4 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.117 | 6.074 | 2.834 |
| 3. | Bulk Density (Kg./Litre) | 1.75 | 1.63 | 1.80 |
| 4. | Specific Gravity | 2.86 | 2.82 | 2.67 |
| 5. | Water absorption | 0.20 % | 0.55% | 1.43 % |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./669/2022-23/                                                      Dated: 04.05.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	M/s  Angelic Infracon Private Limited

534, MalGodam Road,

Sultanpur-228001

Ref.		:    	181/AIPL/20-21     Dated : 17-02-2022

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489, W-06, M-02, Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-30 grade with

Admixture

______________________________________________________________________________

Sub :  Design Mix of M-30 grade

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 145 Minutes 365 Minutes 24.7 N/mm2 34.7 N/mm2 44.4 N/mm2 1.29 2.89 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./605/2022-23/                                                                Dated: 04.05.2022

#### TEST REPORTS

Tested for 	:      	Upper Project Manager (Civil)

U.P.R.N.N. Ltd.

Vindhyachal Unit, Mirzapur

Camp Office: Plot No. 4, Sector-7, Gomtinagar Extension

(Near Saheedpath), Lucknow

Ref.		:    	250/ ifj0izc0foU/;k0b0feTkkZ0/ foU/; dk0 jkfufu /21-22   Dated : 18-01-2022

Tax Invoice No. : 	1435 IIT BHU dated 09.03.2022

Details of Sample :      Concrete Mix Design.

Test Required       	  M-30 grade with admixture for Slump 75-125 mm

__________________________________________________________________________

Sub :  Mix Design M-30 grade for Vindhya Corridor at district Mirzapur

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489,  Y-2022.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used  Conplast SP 430.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./605/2022-23/                                                                Dated: 04.05.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-30 grade with admixture for slump 75-125 mm

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-30 Grade | 0.45 : 1 : 1.77 : 1.13 : 1.70 : 0.5* 0.585 : 1 : 1.29 : 0.96 : 1.33 : 0.5* | By weight By volume |

| Sl. No. | Test | M-30 Grade with admixture for slump 75-125 mm |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 39.9 |
| 2 | Slump range (mm) | 75-125 |
| 3 | Slump Achieved in the Laboratory (mm) | 110.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 28.0 42.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Conplast SP 430 (Litre) | 175.5 390.0 690.0 441.0 663.0 1.95 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./605/2022-23/                                                                Dated: 04.05.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design M-30 grade with admixture for slump 75-125 mm

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm . | FA ( sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 87.9 | 100.0 | - |
|  | 10 mm | 0.40 | 86.4 | - |
|  | 4.75 mm | 0.05 | 7.35 | 1.8 |
|  | 2.36 mm | - | 0.45 | 18.2 |
|  | 1.18 mm | - | 0.05 | 34.4 |
|  | 600 micron | - | - | 49.8 |
|  | 300 micron | - | - | 90.4 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 7.117 | 6.058 | 2.934 |
| 3. | Bulk Density (Kg./Litre) | 1.66 | 1.53 | 1.79 |
| 4. | Specific Gravity | 2.64 | 2.60 | 2.67 |
| 5. | Water absorption | 0.35% | 0.45% | 1.65% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./605/2022-23/                                                                Dated: 04.05.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Upper Project Manager (Civil)

U.P.R.N.N. Ltd.

Vindhyachal Unit, Mirzapur

Camp Office: Plot No. 4, Sector-7, Gomtinagar Extension

(Near Saheedpath), Lucknow

Ref.		:    	250/ ifj0izc0foU/;k0b0feTkkZ0/ foU/; dk0 jkfufu /21-22   Dated : 18-01-2022

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489,  Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-30 grade with admixture for slump 75-125 mm

______________________________________________________________________________

Sub :  Mix Design M-30 grade for Vindhya Corridor at district Mirzapur

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 145 Minutes 350 Minutes 24.3 N/mm2 34.4 N/mm2 44.1 N/mm2 1.30 2.90 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./602/2022-23/                                                                Dated: 04.05.2022

#### TEST REPORTS

Tested for 	:      	Unit Incharge

U.P.R.N.N. Ltd.

P.G.I. Unit-1

S.G.P.G.I. Campus near Jheel

Raibarelly Road, Lucknow-226014

(M/s ISC INFRA DEVELOPERS)

Ref.			:    	651/ ih0th0vkÃ0/ bdkÃ-1/irzk-Mk;fCkVhd LksUVj/jkfufu /22   Dated : 16-02-2022

Tax Invoice No. : 	 1426 IIT BHU dated 03.03.2022

Details of Sample :      	Concrete Mix Design

Test Required       	 M-25 grade with admixture

__________________________________________________________________________

Sub: M-25 grade mix design for construction of Advance Diabetic Centre under S.G.P.G.I.M.S. Campus.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489, W-07, M-02, Y-2022.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used  CHYSO-OPTIMA-5662.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./602/2022-23/                                                                Dated: 04.05.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.16 : 1.32 : 1.98 : 0.4* 0.65 : 1 : 1.56 : 1.08 : 1.48 : 0.4* | By weight By volume |

| Sl. No. | Test | M-25 Grade with admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (mm) | 125-175 |
| 3 | Slump Achieved in the Laboratory (mm) | 160.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 25.5 35.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) CHYSO-OPTIMA-5662 (Litre) | 175.0 350.0 756.0 462.0 693.0 1.40 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./602/2022-23/                                                                Dated: 04.05.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design M-25 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm . | FA ( sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.05 | 100.0 | - |
|  | 10 mm | 0.40 | 87.75 | - |
|  | 4.75 mm | 0.05 | 5.90 | 2.4 |
|  | 2.36 mm | - | 0.30 | 8.8 |
|  | 1.18 mm | - | 0.05 | 27.2 |
|  | 600 micron | - | - | 63.4 |
|  | 300 micron | - | - | 93.8 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.135 | 6.060 | 2.946 |
| 3. | Bulk Density (Kg./Litre) | 1.74 | 1.59 | 1.80 |
| 4. | Specific Gravity | 2.81 | 2.83 | 2.66 |
| 5. | Water absorption | 0.10% | 0.71% | 1.10% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./602/2022-23/                                                                Dated: 04.05.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Unit Incharge

U.P.R.N.N. Ltd.

P.G.I. Unit-1

S.G.P.G.I. Campus near Jheel

Raibarelly Road, Lucknow-226014

(M/s ISC INFRA DEVELOPERS)

Ref.			:    	651/ ih0th0vkÃ0/ bdkÃ-1/irzk-Mk;fCkVhd LksUVj/jkfufu /22   Dated : 16-02-2022

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489, W-07, M-02, Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with admixture

______________________________________________________________________________

Sub: M-25 grade mix design for construction of Advance Diabetic Centre under S.G.P.G.I.M.S. Campus.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 36 % 1.5 mm Expansion 145 Minutes 380 Minutes 24.7 N/mm2 34.5 N/mm2 44.4 N/mm2 1.30 2.90 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./21/2022-23/ 	                                                               Dated: 04.05.2022

#### TEST REPORTS

Tested for 	:      	RCM L & T Buxar Project Site

Larsen and Toubro Limited,

2x660 MW,  Buxar Thermal Power Project

Ref.		:     	 L & T POWER/SJVN/BUXAR/EPC/21-22/38 Dated 08-12-2021

Tax Invoice No. : 	52 IIT BHU dated 12.04.2022

Details of Sample :      	Concrete Mix Design with MP Birla Multicem Cement, PPC, chhatarpur source of aggregate,  Sone River sand and admixture CICO PLAST SUPER C711

Test Required       	 (i) M-20 grade (ii) M-25 grade (iii)  M-30 grade

__________________________________________________________________________

Work: Concrete Mix Design

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (River sand) are given in Appendix –‘A’.

- The Cement supplied was M.P. Birla Multicem Cement, PPC, IS : 1489, W-47, M-11, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used CICO PLAST SUPER C711.                                .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./21/2022-23/ 	                                                               Dated: 04.05.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion (i) M-20 grade  (ii) M-25 grade (iii)  M-30 grade

| Grade of Concrete |  | Water : Cement : River : CA-(10) : CA- (20) : Admixture Sand | Remark |
| --- | --- | --- | --- |
| 1. | M-20 grade | 0.55 : 1 : 2.39 : 1.34 : 2.01 : 0.6* 0.710 : 1 : 1.75 : 1.09 : 1.53 : 0.6* | By weight By volume |
| 2. | M-25 grade for civil work | 0.50 : 1 : 2.07 : 1.21 : 1.81 : 0.6* 0.645 : 1 : 1.52 : 0.99 : 1.38 : 0.6* | By weight By volume |
| 3. | M-30 grade | 0.45 : 1 : 1.78 : 1.09 : 1.63 : 0.6* 0.581 : 1 : 1.30 : 0.89 : 1.24 : 0.6* | By weight By volume |

| Sl. No. | Test | M-20 grade | M-25 grade for civil work | M-30grade |
| --- | --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 27.6 | 33.7 | 39.9 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 | 150-200 |
| 3 | Slump range (after one hour ) (mm) | 100-150 | 100-150 | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 190.0 | 185.0 | 185.0 |
| 5 | Slump Achieved in the Laboratory(after one hour)(mm) | 140.0 | 135.0 | 135.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 18.5 28.5 | 23.0 34.5 | 28.5 41.5 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) River Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) CICO PLAST SUPER C711 (Litre) | 181.5 330.0 789.0 442.0 660.0 1.98 | 182.5 365.0 756.0 442.0 661.0 2.19 | 182.25 405.0 721.0 441.0 660.0 2.43 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./21/2022-23/ 	                                                               Dated: 04.05.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design (i) M-20 grade (ii)

M-25 grade (iii)  M-30 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm (Source: Chhatarpur) | CA—10 mm (Source: Chhatarpur) | River sand (Source : Sone river) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 99.5 | 100.0 | - |
|  | 10 mm | 3.25 | 90.15 | - |
|  | 4.75 mm | 0.05 | 9.75 | 1.8 |
|  | 2.36 mm | - | 0.40 | 6.8 |
|  | 1.18 mm | - | 0.05 | 22.8 |
|  | 600 micron | - | - | 58.8 |
|  | 300 micron | - | - | 94.2 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 6.972 | 5.997 | 2.832 |
| 3. | Bulk Density (Kg./Litre) | 1.69 | 1.58 | 1.76 |
| 4. | Specific Gravity | 2.70 | 2.78 | 2.70 |
| 5. | Water absorption | 0.50 % | 0.55 % | 1.08% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./21/2022-23/ 	                                                               Dated: 04.05.2022

#### TEST REPORTS

Tested for 	:      	RCM L & T Buxar Project Site

Larsen and Toubro Limited,

2x660 MW,  Buxar Thermal Power Project

Ref.		:     	 L & T POWER/SJVN/BUXAR/EPC/21-22/38 Dated 08-12-2021

Details of Sample:	Cement:  M.P. Birla Multicem Cement, PPC, IS : 1489, W-47, M-11, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design  Design (i) M-20 grade  (ii) M-25 grade (iii)  M-30 grade

______________________________________________________________________________

Work: Concrete Mix Design

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 5 % 28 % 1.5 mm Expansion 165 Minutes 250 Minutes 19.5 N/mm2 33.1 N/mm2 45.1 N/mm2 1.29 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./614/2022-23/                                                                Dated: 20.04.2022

#### TEST REPORTS

Tested for 	:      	M/s Doosan Power System India Pvt. Ltd.

DPSI site Office

Obra C Extension thermal Power Station

U.P. Rajya Vidyut Utpadan Nigam Ltd.

Obra, District: Sonebhadra, U.P. – 231219

(M/s Shiv Shakti Associates, Vill. Piparwan, Badhkul, Chitrangi, Singrauli)

Ref.			:    	OBRA C-DPSI-SITEUP-0079    Dated : 04-03-2022

Tax Invoice No. : 	 1521 IIT BHU dated 11.03.2022

Details of Sample :      	Concrete Mix Design with Ultra Tech Cement, OPC 43-grade., Coarse

Aggregate (Source: Vill. Piparwan, Badhkul, Chitrangi, Dsitt. Singrauli, M.P.)  & Sand (Source: Chhattisgarh )

Test Required       	 M-25 grade with admixture

__________________________________________________________________________

Sub: M-25 mix design with Ultratech Cement and Chitrangi source of aggregate

Ref.: EPC Package for Obra “C” 2x660 MW Thermal Power Extension Project

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 43-grade, IS: 269,  Y-2022.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used SikaPlast-5061 NS.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./614/2022-23/                                                                Dated: 20.04.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.48 : 1 : 2.11 : 1.45 : 1.78 : 0.6* 0.677 : 1 : 1.69 : 1.29 : 1.44 : 0.6* | By weight By volume |

| Sl. No. | Test | M-25 Grade with admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (mm) | 125-175 |
| 3 | Slump Achieved in the Laboratory (mm) | 165.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 27.0 35.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) SikaPlast-5061 NS (Litre) | 175.2 365.0 770.0 529.0 650.0 2.19 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./614/2022-23/                                                                Dated: 20.04.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design M-25 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm Source: Vill. Piparwan, Badhkul, Chitrangi, Dsitt. Singrauli, M.P. | CA—10 mm Source: Vill. Piparwan, Badhkul, Chitrangi, Dsitt. Singrauli, M.P. | FA ( sand) Source: Chhattisgarh |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 93.65 | 100.0 | - |
|  | 10 mm | 0.75 | 91.75 | - |
|  | 4.75 mm | 0.05 | 6.00 | 2.6 |
|  | 2.36 mm | - | 0.45 | 6.0 |
|  | 1.18 mm | - | 0.20 | 22.4 |
|  | 600 micron | - | 0.10 | 58.6 |
|  | 300 micron | - | - | 93.0 |
|  | 150 micron | - | - | 99.2 |
| 2. | Fineness Modulus | 7.056 | 6.015 | 2.818 |
| 3. | Bulk Density (Kg./Litre) | 1.74 | 1.59 | 1.76 |
| 4. | Specific Gravity | 2.86 | 2.87 | 2.69 |
| 5. | Water absorption | 0.45% | 0.30% | 0.69% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./614/2022-23/                                                                Dated: 20.04.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	M/s Doosan Power System India Pvt. Ltd.

DPSI site Office

Obra C Extension thermal Power Station

U.P. Rajya Vidyut Utpadan Nigam Ltd.

Obra, District: Sonebhadra, U.P. – 231219

(M/s Shiv Shakti Associates, Vill. Piparwan, Badhkul, Chitrangi, Singrauli)

Ref.		:    		OBRA C-DPSI-SITEUP-0079    Dated : 04-03-2022

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grade,IS : 269,  Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with admixture

______________________________________________________________________________

Sub: M-25 mix design with Ultratech Cement and Chitrangi source of aggregate

Ref.: EPC Package for Obra “C” 2x660 MW Thermal Power Extension Project

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 31 % 1.0 mm Expansion 110 Minutes 250 Minutes 26.4 N/mm2 37.7 N/mm2 47.2 N/mm2 1.41 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./579(i)/2022-23/                                                          Dated: 08.04.2022

#### TEST REPORTS

Tested for 	:      	Sr. Mgr. (FES & FQA)

Bhartiya Rail Bijlee Company Ltd.

NTPC, Nabinagar, Aurangabad

(M/s Shakti Construction)

Ref.		:    	BRBCL/FQA/2022/SHAKTI-Design Mix x   Dated : 01-02-2022

Tax Invoice No. : 	1381 IIT BHU dated 21.02.2022

Details of Sample :      Concrete Mix Design.

Test Required       	    (i) M-10 grade without admixture and M-20 grade with admixture

(ii) M-25 grade with admixture and M-30 grade with admixture

__________________________________________________________________________

Sub: Evaluation of materials and concrete mix design.

Work: Construction of Raw Water Reservoir at BRBCL “Package for BRBCL Nabinagar TPP(4x250 MW)

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Double Bull Cement,  PPC IS : 1489, W-08, M-02, Y-2022.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used SikaPlast-5202 NS.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./579(i)/2022-23/                                                          Dated: 08.04.2022

#### (Page – 2(i))

#### TEST REPORTS

Recommended Mix Proportion for   (i) M-10 grade without admixture and M-20 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-10 Grade | 0.70 : 1 : 3.05 : 1.45 : 2.69 : - 0.896 : 1 : 2.19 : 1.21 : 2.07 : - | By weight By volume |
| 2. | M-20 Grade | 0.55 : 1 : 2.54 : 1.37 : 2.54 : 0.4* 0.704 : 1 : 1.83 : 1.15 : 1.96 : 0.4* | By weight By volume |

| Sl. No. | Test | M-10 grade without admixture | M-20 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 13.8 | 27.6 |
| 2 | Slump range (mm) | 50-100 | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 80.0 | 95.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 11.5 18.0 | 18.0 28.0 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) SikaPlast-5202 NS (Litre) | 189.0 270.0 824.0 392.0 726.0 - | 167.75 305.0 775.0 418.0 775.0 1.22 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./579(i)/2022-23/                                                          Dated: 08.04.2022

#### (Page – 2(ii))

#### TEST REPORTS

Recommended Mix Proportion for (ii) M-25 grade with admixture and M-30 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.23 : 1.25 : 2.32 : 0.4* 0.64 : 1 : 1.60 : 1.05 : 1.79 : 0.4* | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 1.90 : 1.11 : 2.06 : 0.4* 0.576 : 1 : 1.37 : 0.93 : 1.59 : 0.4* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture | M-30 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 |
| 2 | Slump range (mm) | 50-100 | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 90.0 | 90.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 23.5 35.0 | 29.0 41.5 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) SikaPlast-5202 NS (Litre) | 167.5 335.0 747.0 419.0 777.0 1.34 | 168.75 375.0 713.0 416.0 773.0 1.50 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./579(i)/2022-23/                                                          Dated: 08.04.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design  (i) M-10 grade without

admixture and M-20 grade with admixture (ii) M-25 grade with admixture and M-30 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm (Source: Chhaterpur) | CA—10 mm (Source: Chhaterpur) | FA ( sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.55 | 100.0 | - |
|  | 10 mm | 0.15 | 90.1 | - |
|  | 4.75 mm | 0.05 | 5.65 | 2.4 |
|  | 2.36 mm | - | 0.40 | 6.2 |
|  | 1.18 mm | - | 0.05 | 18.0 |
|  | 600 micron | - | - | 47.6 |
|  | 300 micron | - | - | 87.6 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 7.133 | 6.038 | 2.606 |
| 3. | Bulk Density (Kg./Litre) | 1.66 | 1.53 | 1.78 |
| 4. | Specific Gravity | 2.75 | 2.79 | 2.71 |
| 5. | Water absorption | 0.35 % | 0.60% | 1.50% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./579(i)/2022-23/                                                          Dated: 08.04.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Sr. Mgr. (FES & FQA)

Bhartiya Rail Bijlee Company Ltd.

NTPC, Nabinagar, Aurangabad

(M/s Shakti Construction)

Ref.		:    	BRBCL/FQA/2022/SHAKTI-Design Mix x   Dated : 01-02-2022

Details of Sample:	Cement: Double Bull Cement,  PPC IS : 1489, W-08, M-02, Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design Design  (i) M-10 grade without

admixture and M-20 grade with admixture (ii) M-25 grade with admixture and M-30 grade with admixture

______________________________________________________________________________

Sub: Evaluation of materials and concrete mix design.

Work: Construction of Raw Water Reservoir at BRBCL “Package for BRBCL Nabinagar TPP(4x250 MW)

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 33 % 1.5 mm Expansion 145 Minutes 375 Minutes 23.5 N/mm2 32.3 N/mm2 41.3 N/mm2 1.28 2.88 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./536/2021-22/                                                         Dated: 29.03.2022

#### TEST REPORTS

Tested for 	:      	Sr. Mgr. (FES & FQA)

Bhartiya Rail Bijlee Company Ltd.

NTPC, Nabinagar, Aurangabad

(M/s  A  K  Verma)

Ref.			:    	BRBCL/FQA/2022/A.K. VERMA-Design Mix   Dated : 25-01-2022

Tax Invoice No. : 	 1319 IIT BHU dated 14.02.2022

Details of Sample :      	Concrete Mix Design.

Test Required       	 M-25 grade

___________________________________________________________________________

Sub: Evaluation of materials and concrete mix design.

Work: Construction of Administrative building  at NTPP, Package for BRBCL Nabinagar TPP(4x250 MW)

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes   (i) 20 mm. (ii) 10 mm

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Double Bull Cement,  PPC IS : 1489, W-48, M-12, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./536/2021-22/                                                         Dated: 29.03.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M25 Grade | 0.50 : 1 : 1.82 : 1.05 : 1.96 0.64 : 1 : 1.31 : 0.86 : 1.52 | By weight By volume |

| Sl. No. | Test | M-25 grade |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (mm) | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 85.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 24.0 34.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 190.0 380.0 692.0 399.0 745.0 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./536/2021-22/                                                         Dated: 29.03.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA—20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 87.55 | 100.0 | - |
|  | 10 mm | 1.20 | 86.05 | - |
|  | 4.75 mm | 0.05 | 0.75 | 1.6 |
|  | 2.36 mm | - | 0.30 | 4.4 |
|  | 1.18 mm | - | 0.05 | 16.4 |
|  | 600 micron | - | - | 45.8 |
|  | 300 micron | - | - | 88.4 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 7.122 | 6.129 | 2.554 |
| 3. | Bulk Density (Kg./Litre) | 1.65 | 1.56 | 1.78 |
| 4. | Specific Gravity | 2.77 | 2.75 | 2.72 |
| 5. | Water absorption | 0.30% | 0.60 % | 1.50% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./536/2021-22/                                                         Dated: 29.03.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Sr. Mgr. (FES & FQA)

Bhartiya Rail Bijlee Company Ltd.

NTPC, Nabinagar, Aurangabad

(M/s  A  K  Verma)

Ref.			:    	BRBCL/FQA/2022/A.K. VERMA-Design Mix   Dated : 25-01-2022

Details of Sample:	Cement: Double Bull Cement,  PPC IS : 1489, W-48,, M-12, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade

______________________________________________________________________________

Sub: Evaluation of materials and concrete mix design.

Work: Construction of Administrative building  at NTPP, Package for BRBCL Nabinagar TPP(4x250 MW)

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 33 % 1.5 mm Expansion 150 Minutes 380 Minutes 23.2 N/mm2 31.9 N/mm2 41.1 N/mm2 1.28 2.88 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./525/2021-22/                                                         Dated: 17.03.2022

#### TEST REPORTS

Tested for 	:      	Project Manager (Varanasi Tube Well Scheme)

M/s L & T  Constructions

RWS Empanelment Tubewell Scheme

L-34 Ashok Vihar Colony Phase-1,Varanasi, U.P.-221007

Ref.		:    		Ref. No. L & T-WET/RWS/VNS-TWS/188  Dated 11-01-2022

Tax Invoice No. : 	1290  IIT BHU dated 07.02.2022

Details of Sample :      Concrete Mix Design.

Test Required       	  (i)   M-25 grade with admixture with Ultra Tech Cement, OPC

(ii)  M-30 grade with admixture with Ultra Tech Cement, OPC

(iii)  M-40 grade with admixture with Ultra Tech Cement, OPC for 425 Kg.

Cement

________________________________________________________________________

Sub :  Concrete Mix design- Grades M25, M30 & M40

Projects: VARANASI TUBE WELL SCHEME-Package 1.

.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 43-grde, IS : 269,  Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used for M-25 & M-30 grade : KUNAPLAST PC-30VNS and

for M-40 grade : KUNAPLAST PC-70HES.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./525/2021-22/                                                         Dated: 17.03.2022

#### (Page – 2(i))

#### TEST REPORTS

Recommended Mix Proportion   (i)   M-25 grade with admixture with Ultra Tech Cement, OPC(ii)  M-30 grade

with admixture with  Ultra Tech Cement, OPC

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.45 : 1.45 : 1.77 : 0.55* 0.70 : 1 : 1.93 : 1.24 : 1.48 : 0.55* | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 2.12 : 1.31 : 1.60 : 0.55* 0.63 : 1 : 1.67 : 1.12 : 1.33 : 0.55* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture | M-30 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 |
| 3 | Slump range (after one hour ) (mm) | 100-150 | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 170.0 | 180.0 |
| 5 | Slump Achieved in the Laboratory(after one hour) (mm) | 120.0 | 130.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 27.5 36.0 | 31.5 42.0 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) KUNAPLAST PC-30VNS (Litre) | 167.5 335.0 821.0 486.0 593.0 1.85 | 167.4 372.0 789.0 487.0 595.0 2.05 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./525/2021-22/                                                         Dated: 17.03.2022

#### (Page – 2(ii))

#### TEST REPORTS

Recommended Mix Proportion  (iii)  M-40 grade with admixture with Ultra Tech Cement, OPC for 425

Kg. Cement.

Cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-40 Grade for 425 Kg Cement | 0.34 : 1 : 1.73 : 1.08 : 1.61 : 0.4* 0.476 : 1 : 1.36 : 0.92 : 1.34 : 0.4* | By weight By volume |

| Sl. No. | Test | M-40 grade with admixture for 425 Kg. Cement |
| --- | --- | --- |
| 1. | Target Strength (N/mm2) | 50.9 |
| 2. | Slump range (initial) (mm) | 100-150 |
| 3. | Slump range (after one hour ) (mm) | 50-100 |
| 4. | Slump Achieved in the Laboratory (initial) (mm) | 140.0 |
| 5. | Slump Achieved in the Laboratory(after one hour) (mm) | 80.0 |
| 6. | i) After 24 hours Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) iii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 16.0 39.0 53.5 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) KUNAPLAST PC-70HES (Litre) | 144.5 425.0 735.0 459.0 685.0 1.70 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./525/2021-22/                                                                  Dated: 17.03.2022 Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design for (i)   M-25 grade   with

admixture with Ultra Tech Cement, OPC (ii)  M-30 grade with admixture with Ultra Tech Cement, OPC

(iii)  M-40 grade with admixture with Ultra Tech Cement, OPC for 425 Kg. Cement

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm Source: Hanumana Belt (Chitrangi)-MP | CA—10 mm (Source: Hanumana Belt (Chitrangi)-MP | FA (sand) (Source: Dehri-Bihar) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 94.25 | 100.0 | - |
|  | 10 mm | 0.70 | 86.75 | - |
|  | 4.75 mm | 0.05 | 0.80 | 2.4 |
|  | 2.36 mm | - | 0.30 | 6.4 |
|  | 1.18 mm | - | 0.05 | 20.8 |
|  | 600 micron | - | - | 57.4 |
|  | 300 micron | - | - | 95.8 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 7.050 | 6.121 | 2.816 |
| 3. | Bulk Density (Kg./Litre) | 1.68 | 1.64 | 1.78 |
| 4. | Specific Gravity | 2.65 | 2.65 | 2.67 |
| 5. | Water absorption | 0.50 % | 0.50 % | 1.08 % |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./525/2021-22/                                                                 Dated: 17.03.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Project Manager (Varanasi Tube Well Scheme)

M/s L & T  Constructions

RWS Empanelment Tubewell Scheme

L-34 Ashok Vihar Colony Phase-1,Varanasi, U.P.-221007

Ref.		:    		Ref. No. L & T-WET/RWS/VNS-TWS/188  Dated 11-01-2022

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grde, IS : 269,  Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i)   M-25 grade   with

admixture with Ultra Tech Cement, OPC (ii)  M-30 grade with admixture with Ultra Tech Cement, OPC (iii)  M-40 grade with admixture with Ultra Tech Cement, OPC for 425 Kg. Cement

______________________________________________________________________________

Sub :  Concrete Mix design- Grades M25, M30 & M40

Projects: VARANASI TUBE WELL SCHEME-Package 1.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 2 % 30 % 1.0 mm Expansion 85 Minutes 285 Minutes 26.4 N/mm2 37.7 N/mm2 47.2 N/mm2 1.40 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./524/2021-22/                                                         Dated: 17.03.2022

#### TEST REPORTS

Tested for 	:      	Project Manager (Varanasi Tube Well Scheme)

M/s L & T  Constructions

RWS Empanelment Tubewell Scheme

L-34 Ashok Vihar Colony Phase-1,Varanasi, U.P.-221007

Ref.		:    		Ref. No. L & T-WET/RWS/VNS-TWS/189  Dated 11-01-2022

Tax Invoice No. : 	1289  IIT BHU dated 07.02.2022

Details of Sample :      Concrete Mix Design.

Test Required       	  (i)   M-25 grade with admixture with MP Birla Concrecem Cement, OPC

(ii)  M-30 grade with admixture with MP Birla Concrecem Cement, OPC

(iii)  M-40 grade with admixture with MP Birla Concrecem Cement, OPC for 425 Kg.

Cement

________________________________________________________________________

Sub :  Concrete Mix design- Grades M25, M30 & M40

Projects: VARANASI TUBE WELL SCHEME-Package 1.

.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was MP Birla Concrecem Cement, OPC 43-grde, IS : 269,  Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used for M-25 & M-30 grade : KUNAPLAST PC-30VNS and

for M-40 grade : KUNAPLAST PC-70HES.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./524/2021-22/                                                         Dated: 17.03.2022

#### (Page – 2(i))

#### TEST REPORTS

Recommended Mix Proportion   (i)   M-25 grade with admixture with MP Birla Concrecem Cement, OPC

(ii)  M-30 grade with admixture with MP Birla Concrecem Cement, OPC

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.45 : 1.45 : 1.77 : 0.55* 0.705 : 1 : 1.94 : 1.25 : 1.49 : 0.55* | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 2.12 : 1.31 : 1.60 : 0.55* 0.635 : 1 : 1.68 : 1.13 : 1.34 : 0.55* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture | M-30 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 |
| 3 | Slump range (after one hour ) (mm) | 100-150 | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 165.0 | 185.0 |
| 5 | Slump Achieved in the Laboratory(after one hour) (mm) | 140.0 | 150.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 27.0 35.0 | 31.5 41.5 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) KUNAPLAST PC-30VNS (Litre) | 167.5 335.0 821.0 486.0 593.0 1.85 | 167.4 372.0 789.0 487.0 595.0 2.05 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./524/2021-22/                                                         Dated: 17.03.2022

#### (Page – 2(ii))

#### TEST REPORTS

Recommended Mix Proportion  (iii)  M-40 grade with admixture with MP Birla Concrecem Cement,

OPC for 425 Kg. Cement.

Cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-40 Grade for 425 Kg Cement | 0.34 : 1 : 1.73 : 1.08 : 1.61 : 0.4* 0.479 : 1 : 1.37 : 0.93 : 1.35 : 0.4* | By weight By volume |

| Sl. No. | Test | M-40 grade with admixture for 425 Kg. Cement |
| --- | --- | --- |
| 1. | Target Strength (N/mm2) | 50.9 |
| 2. | Slump range (initial) (mm) | 100-150 |
| 3. | Slump range (after one hour ) (mm) | 50-100 |
| 4. | Slump Achieved in the Laboratory (initial) (mm) | 145.0 |
| 5. | Slump Achieved in the Laboratory(after one hour) (mm) | 90.0 |
| 6. | i) After 24 hours Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) iii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 15.3 38.5 53.0 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) KUNAPLAST PC-70HES (Litre) | 144.5 425.0 735.0 459.0 685.0 1.70 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./524/2021-22/                                                                  Dated: 17.03.2022 Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design for (i)   M-25 grade   with

admixture with MP Birla Concrecem Cement, OPC  (ii)  M-30 grade with admixture with MP Birla Concrecem

Cement, OPC (iii)  M-40 grade with admixture with MP Birla Concrecem Cement, OPC for 425 Kg. Cement

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm Source: Hanumana Belt (Chitrangi)-MP | CA—10 mm (Source: Hanumana Belt (Chitrangi)-MP | FA (sand) (Source: Dehri-Bihar) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 94.25 | 100.0 | - |
|  | 10 mm | 0.70 | 86.75 | - |
|  | 4.75 mm | 0.05 | 0.80 | 2.4 |
|  | 2.36 mm | - | 0.30 | 6.4 |
|  | 1.18 mm | - | 0.05 | 20.8 |
|  | 600 micron | - | - | 57.4 |
|  | 300 micron | - | - | 95.8 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 7.050 | 6.121 | 2.816 |
| 3. | Bulk Density (Kg./Litre) | 1.68 | 1.64 | 1.78 |
| 4. | Specific Gravity | 2.65 | 2.65 | 2.67 |
| 5. | Water absorption | 0.50 % | 0.50 % | 1.08 % |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./524/2021-22/                                                                 Dated: 17.03.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Project Manager (Varanasi Tube Well Scheme)

M/s L & T  Constructions

RWS Empanelment Tubewell Scheme

L-34 Ashok Vihar Colony Phase-1,Varanasi, U.P.-221007

Ref.		:    		Ref. No. L & T-WET/RWS/VNS-TWS/189  Dated 11-01-2022

Details of Sample:	Cement: MP Birla Concrecem Cement, OPC 43-grde, IS : 269,  Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i)   M-25 grade   with

admixture with MP Birla Concrecem Cement, OPC  (ii)  M-30 grade with admixture with MP Birla Concrecem Cement, OPC (iii)  M-40 grade with admixture with MP Birla Concrecem Cement, OPC for 425 Kg. Cement

______________________________________________________________________________

Sub :  Concrete Mix design- Grades M25, M30 & M40

Projects: VARANASI TUBE WELL SCHEME-Package 1.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 26 % 1.0 mm Expansion 95 Minutes 295 Minutes 25.2 N/mm2 36.9 N/mm2 46.0 N/mm2 1.41 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./591(iii)/2021-22/                                                          Dated: 16.03.2022

#### TEST REPORTS

Tested for 	:      	Senior Manager (Waste to Energy)

NTPC Vidyut Vyapar Nigam

MSW to Torrefied Charcoal Plant, Varanasi

Near STP Plant, Ramna, U.P.-221011

(M/s Macawber Beekay Pvt. Ltd.)

Ref.		:    	MBPL/NTPCVVNL/SITE/C-1620/3  dated : 11.01.2022

Tax Invoice No. : 	1394 IIT BHU dated 21.02.2022

Details of Sample :      Concrete Mix Design.

Test Required       	    M-25 grade with admixture with Ultra Tech Cement, OPC for roads

__________________________________________________________________________

Sub :  Waste to Energy (600TPD), Ramna, Varanasi – Testing for construction  Materials & design mix for M25

Grade Concrete –Reg.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 43-grade, IS: 269, W-01, M-01, Y-2022.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Conplast SP 430 G8.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./591(iii)/2021-22/                                                          Dated: 16.03.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade with admixture with Ultra Tech Cement, OPC for roads

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade for roads | 0.50 : 1 : 2.21 : 1.17 : 2.18 : 0.5* 0.695 : 1 : 1.71 : 1.04 : 1.75 : 0.5* | By weight By volume |

| Sl. No. | Test | M-25 Grade with admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (mm) | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 80.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 28.5 39.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Conplast SP 430 G8 (Litre) | 175.0 350.0 774.0 410.0 763.0 1.75 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./591(iii)/2021-22/                                                          Dated: 16.03.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design M-25 grade with admixture with Ultra Tech Cement, OPC for roads

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA ( sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 88.05 | 100.0 | - |
|  | 10 mm | 0.30 | 89.5 | - |
|  | 4.75 mm | 0.05 | 0.90 | 3.2 |
|  | 2.36 mm | - | 0.40 | 14.8 |
|  | 1.18 mm | - | 0.05 | 39.6 |
|  | 600 micron | - | - | 74.6 |
|  | 300 micron | - | - | 97.0 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.116 | 6.096 | 3.282 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.57 | 1.80 |
| 4. | Specific Gravity | 2.84 | 2.83 | 2.69 |
| 5. | Water absorption | 0.30 % | 0.35% | 0.90% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./591(iii)/2021-22/                                                          Dated: 16.03.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Senior Manager (Waste to Energy)

NTPC Vidyut Vyapar Nigam

MSW to Torrefied Charcoal Plant, Varanasi

Near STP Plant, Ramna, U.P.-221011

(M/s Macawber Beekay Pvt. Ltd.)

Ref.		:    	MBPL/NTPCVVNL/SITE/C-1620/3  dated : 11.01.2022

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grade,IS : 269, W-01, M-01, Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with admixture with Ultra Tech Cement, OPC for roads

______________________________________________________________________________

Sub :  Waste to Energy (600TPD), Ramna, Varanasi – Testing for construction  Materials & design mix for M25

Grade Concrete –Reg.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 31 % 1.0 mm Expansion 90 Minutes 290 Minutes 26.0 N/mm2 37.5 N/mm2 46.8 N/mm2 1.39 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./591(ii)/2021-22/                                                          Dated: 16.03.2022

#### TEST REPORTS

Tested for 	:      	Senior Manager (Waste to Energy)

NTPC Vidyut Vyapar Nigam

MSW to Torrefied Charcoal Plant, Varanasi

Near STP Plant, Ramna, U.P.-221011

(M/s Macawber Beekay Pvt. Ltd.)

Ref.		:    	MBPL/NTPCVVNL/SITE/C-1620/3  dated : 11.01.2022

Tax Invoice No. : 	1394 IIT BHU dated 21.02.2022

Details of Sample :      Concrete Mix Design.

Test Required       	    (i) M-25 grade with admixture with Ultra Tech Cement, PPC for Buildings &

Foundations

(ii) M-25 grade with admixture with Ultra Tech Cement, PPC for Piling works

__________________________________________________________________________

Sub :  Waste to Energy (600TPD), Ramna, Varanasi – Testing for construction  Materials & design mix for M25

Grade Concrete –Reg.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC IS : 1489, W-01, M-01, Y-2022.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Conplast SP 430 G8.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./591(ii)/2021-22/                                                          Dated: 16.03.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for (i) M-25 grade with admixture with Ultra Tech Cement, PPC for Buildings &  Foundations   (ii) M-25 grade with admixture with Ultra Tech Cement, PPC for Piling works

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade for Buildings & foundations | 0.50 : 1 : 2.20 : 1.23 : 1.84 : 0.8* 0.65 : 1 : 1.59 : 1.02 : 1.38 : 0.8* | By weight By volume |
| 2. | M-25 Grade for piling works | 0.50 : 1 : 2.25 : 1.36 : 1.66 : 1.0* 0.65 : 1 : 1.63 : 1.13 : 1.25 : 1.0* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture with Ultra Tech Cement, PPC for Buildings & Foundations | M-25 grade with admixture with Ultra Tech Cement, PPC for Piling works |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 33.7 |
| 2 | Slump range (mm) | 125-175 | 175-225 |
| 3 | Slump Achieved in the Laboratory (mm) | 155.0 | 185.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 27.0 38.5 | 26.5 38.0 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Conplast SP 430 G8 (Litre) | 180.0 360.0 792.0 443.0 663.0 2.88 | 180.0 360.0 810.0 490.0 598.0 3.60 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./591(ii)/2021-22/                                                          Dated: 16.03.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design (i) M-25 grade with admixture with Ultra Tech Cement, PPC for Buildings &  Foundations   (ii) M-25 grade with admixture with Ultra Tech Cement, PPC for Piling works

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA ( sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 88.05 | 100.0 | - |
|  | 10 mm | 0.30 | 89.5 | - |
|  | 4.75 mm | 0.05 | 0.90 | 3.2 |
|  | 2.36 mm | - | 0.40 | 14.8 |
|  | 1.18 mm | - | 0.05 | 39.6 |
|  | 600 micron | - | - | 74.6 |
|  | 300 micron | - | - | 97.0 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.116 | 6.096 | 3.282 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.57 | 1.80 |
| 4. | Specific Gravity | 2.84 | 2.83 | 2.69 |
| 5. | Water absorption | 0.30 % | 0.35% | 0.90% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./591(ii)/2021-22/                                                          Dated: 16.03.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Senior Manager (Waste to Energy)

NTPC Vidyut Vyapar Nigam

MSW to Torrefied Charcoal Plant, Varanasi

Near STP Plant, Ramna, U.P.-221011

(M/s Macawber Beekay Pvt. Ltd.)

Ref.		:    	MBPL/NTPCVVNL/SITE/C-1620/3  dated : 11.01.2022

Details of Sample:	Cement: Ultra Tech Cement, PPC IS : 1489, W-01, M-01, Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i) M-25 grade with admixture with Ultra Tech Cement, PPC for Buildings &  Foundations  (ii) M-25 grade with admixture with Ultra Tech Cement, PPC for Piling works

______________________________________________________________________________

Sub :  Waste to Energy (600TPD), Ramna, Varanasi – Testing for construction  Materials & design mix for M25

Grade Concrete –Reg.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 145 Minutes 350 Minutes 24.0 N/mm2 34.0 N/mm2 43.6 N/mm2 1.30 2.90 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./591(i)/2021-22/                                                          Dated: 16.03.2022

#### TEST REPORTS

Tested for 	:      	Senior Manager (Waste to Energy)

NTPC Vidyut Vyapar Nigam

MSW to Torrefied Charcoal Plant, Varanasi

Near STP Plant, Ramna, U.P.-221011

(M/s Macawber Beekay Pvt. Ltd.)

Ref.		:    	MBPL/NTPCVVNL/SITE/C-1620/3  dated : 11.01.2022

Tax Invoice No. : 	1394 IIT BHU dated 21.02.2022

Details of Sample :      Concrete Mix Design.

Test Required       	  (i) M-25 grade with admixture with ACC Cement, PPC for Buildings &

Foundations

(ii) M-25 grade with admixture with ACC Cement, PPC for Piling works

__________________________________________________________________________

Sub :  Waste to Energy (600TPD), Ramna, Varanasi – Testing for construction  Materials & design mix for M25

Grade Concrete –Reg.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was ACC Suraksha Power Cement, PPC IS : 1489, W-01, M-01, Y-2022.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Conplast SP 430 G8.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./591(i)/2021-22/                                                          Dated: 16.03.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for (i) M-25 grade with admixture with ACC Cement, PPC for Buildings &

Foundations   (ii) M-25 grade with admixture with ACC Cement, PPC for Piling works

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade for Buildings & foundations | 0.50 : 1 : 2.20 : 1.23 : 1.84 : 0.7* 0.64 : 1 : 1.56 : 1.00 : 1.36 : 0.7* | By weight By volume |
| 2. | M-25 Grade for piling works | 0.50 : 1 : 2.25 : 1.36 : 1.66 : 0.9* 0.64 : 1 : 1.60 : 1.11 : 1.23 : 0.9* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture with ACC Cement, PPC for Buildings & Foundations | M-25 grade with admixture with ACC Cement, PPC for Piling works |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 33.7 |
| 2 | Slump range (mm) | 125-175 | 175-225 |
| 3 | Slump Achieved in the Laboratory (mm) | 170.0 | 200.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 24.0 34.5 | 23.0 34.0 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Conplast SP 430 G8 (Litre) | 180.0 360.0 792.0 443.0 663.0 2.52 | 180.0 360.0 810.0 490.0 598.0 3.24 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./591(i)/2021-22/                                                          Dated: 16.03.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design (i) M-25 grade with admixture

with ACC Cement, PPC for Buildings & Foundations   (ii) M-25 grade with admixture with ACC Cement, PPC

for Piling works

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA ( sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 88.05 | 100.0 | - |
|  | 10 mm | 0.30 | 89.5 | - |
|  | 4.75 mm | 0.05 | 0.90 | 3.2 |
|  | 2.36 mm | - | 0.40 | 14.8 |
|  | 1.18 mm | - | 0.05 | 39.6 |
|  | 600 micron | - | - | 74.6 |
|  | 300 micron | - | - | 97.0 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.116 | 6.096 | 3.282 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.57 | 1.80 |
| 4. | Specific Gravity | 2.84 | 2.83 | 2.69 |
| 5. | Water absorption | 0.30 % | 0.35% | 0.90% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./591(i)/2021-22/                                                          Dated: 16.03.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Senior Manager (Waste to Energy)

NTPC Vidyut Vyapar Nigam

MSW to Torrefied Charcoal Plant, Varanasi

Near STP Plant, Ramna, U.P.-221011

(M/s Macawber Beekay Pvt. Ltd.)

Ref.		:    	MBPL/NTPCVVNL/SITE/C-1620/3  dated : 11.01.2022

Details of Sample:	Cement: ACC Suraksha Power Cement, PPC IS : 1489, W-01, M-01, Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i) M-25 grade with admixture

with ACC Cement, PPC for Buildings & Foundations   (ii) M-25 grade with admixture with ACC Cement, PPC for Piling works

______________________________________________________________________________

Sub :  Waste to Energy (600TPD), Ramna, Varanasi – Testing for construction  Materials & design mix for M25

Grade Concrete –Reg.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 8 % 39 % 1.5 mm Expansion 150 Minutes 375 Minutes 23.6 N/mm2 33.6 N/mm2 43.6 N/mm2 1.28 2.88 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./580/2021-22/                                                          Dated: 15.03.2022

#### TEST REPORTS

Tested for 	:      	Project Manager

UP Jal Nigam (Rural)

Ganga Pollution Prevention Unit-II

Bhagwanpur,Varanasi-221005

(MIRZAPUR GHAZIPUR STPS PRIVATE LIMITED)

Ref.		:    	48/ Mirzapur STP /02   dated 15.01.2022

Tax Invoice No. : 	1383 IIT BHU dated 21.02.2022

Details of Sample :      Concrete Mix Design.

Test Required       	  M-30 grade without admixture with M.P. Birla Cement, Chetak, Sulphate

Resisting Portland Cement

__________________________________________________________________________

Sub :  Regarding Concrete  design mix (M30)

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was M.P. Birla Cement, Chetak, Sulphate Resisting Portland Cement, IS:12330, W-01, M-01, Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./580/2021-22/                                                          Dated: 15.03.2022

#### (Page – 2(i))

#### TEST REPORTS

Recommended Mix Proportion for M-30 grade without admixture with M.P. Birla Cement, Chetak,

Sulphate Resisting Portland cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M-30 Grade | 0.45 : 1 : 1.66 : 1.00 : 1.22 0.666 : 1 : 1.38 : 0.95 : 1.04 | By weight By volume |

| Sl. No. | Test | M-30 grade without admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 39.9 |
| 2 | Slump range (initial) (mm) | 150-200 |
| 3 | Slump range (after 2 hours ) (mm) | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 165.0 |
| 5 | Slump Achieved in the Laboratory(after 2 hours) (mm) | 125.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 32.5 43.0 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 202.5 450.0 747.0 450.0 549.0 |

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./580/2021-22/                                                          Dated: 15.03.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design M-30 grade without

admixture with M.P. Birla Cement, Chetak, Sulphate Resisting Portland cement

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 85.75 | 100.0 | - |
|  | 10 mm | 0.30 | 89.55 | - |
|  | 4.75 mm | 0.05 | 0.90 | 5.0 |
|  | 2.36 mm | - | 0.40 | 18.8 |
|  | 1.18 mm | - | 0.10 | 46.6 |
|  | 600 micron | - | - | 78.8 |
|  | 300 micron | - | - | 95.6 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.139 | 6.091 | 3.438 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.56 | 1.78 |
| 4. | Specific Gravity | 2.82 | 2.80 | 2.68 |
| 5. | Water absorption | 0.65 % | 0.60 % | 1.50% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./580/2021-22/                                                          Dated: 15.03.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Project Manager

UP Jal Nigam (Rural)

Ganga Pollution Prevention Unit-II

Bhagwanpur,Varanasi-221005

(MIRZAPUR GHAZIPUR STPS PRIVATE LIMITED)

Ref.		:    	48/ Mirzapur STP /02   dated 15.01.2022

Details of Sample:	Cement: M.P. Birla Cement, Chetak, Sulphate Resisting Portland Cement,

IS:12330, W-01, M-01, Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-30 grade without admixture

with M.P. Birla Cement, Chetak, Sulphate Resisting Portland cement

______________________________________________________________________________

Sub :  Regarding Concrete  design mix (M30)

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 4 % 30 % 1.5 mm Expansion 145 Minutes 355 Minutes 16.7 N/mm2 32.0 N/mm2 40.5 N/mm2 1.48 2.92 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./518/2021-22/                                                          Dated: 15.03.2022

#### TEST REPORTS

Tested for 	:      	Project Manager

UP Jal Nigam (Rural)

Ganga Pollution Prevention Unit-II

Bhagwanpur,Varanasi-221005

(MIRZAPUR GHAZIPUR STPS PRIVATE LIMITED)

Ref.		:    	48/ Mirzapur STP /02   dated 15.01.2022

Tax Invoice No. : 	1269 IIT BHU dated 03.02.2022

Details of Sample :      Concrete Mix Design.

Test Required       	  M-30 grade with admixture with M.P. Birla Cement, Chetak, Sulphate Resisting

Portland Cement

__________________________________________________________________________

Sub :  Regarding Concrete  design mix (M30)

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was M.P. Birla Cement, Chetak, Sulphate Resisting Portland Cement, IS:12330, W-01, M-01, Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used SikaPlast 5202 NS.                                       .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./518/2021-22/                                                          Dated: 15.03.2022

#### (Page – 2(i))

#### TEST REPORTS

Recommended Mix Proportion for M-30 grade with admixture with M.P. Birla Cement, Chetak, Sulphate

Resisting Portland cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-30 Grade | 0.45 : 1 : 2.03 : 1.17 : 1.43 : 0.5* 0.666 : 1 : 1.69 : 1.11 : 1.22 : 0.5* | By weight By volume |

| Sl. No. | Test | M-30 grade with admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 39.9 |
| 2 | Slump range (initial) (mm) | 150-200 |
| 3 | Slump range (after 2 hours ) (mm) | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 185.0 |
| 5 | Slump Achieved in the Laboratory(after 2 hours) (mm) | 125.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 32.0 42.0 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) SikaPlast 5202 NS (Litre) | 180.0 400.0 812.0 468.0 572.0 2.00 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./518/2021-22/                                                          Dated: 15.03.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design M-30 grade with

admixture with M.P. Birla Cement, Chetak, Sulphate Resisting Portland cement

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 85.75 | 100.0 | - |
|  | 10 mm | 0.30 | 89.55 | - |
|  | 4.75 mm | 0.05 | 0.90 | 5.0 |
|  | 2.36 mm | - | 0.40 | 18.8 |
|  | 1.18 mm | - | 0.10 | 46.6 |
|  | 600 micron | - | - | 78.8 |
|  | 300 micron | - | - | 95.6 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.139 | 6.091 | 3.438 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.56 | 1.78 |
| 4. | Specific Gravity | 2.82 | 2.80 | 2.68 |
| 5. | Water absorption | 0.65 % | 0.60 % | 1.50% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./518/2021-22/                                                          Dated: 15.03.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Project Manager

UP Jal Nigam (Rural)

Ganga Pollution Prevention Unit-II

Bhagwanpur,Varanasi-221005

(MIRZAPUR GHAZIPUR STPS PRIVATE LIMITED)

Ref.		:    	48/ Mirzapur STP /02   dated 15.01.2022

Details of Sample:	Cement: M.P. Birla Cement, Chetak, Sulphate Resisting Portland Cement,

IS:12330, W-01, M-01, Y-2022

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-30 grade with admixture

with M.P. Birla Cement, Chetak, Sulphate Resisting Portland cement

______________________________________________________________________________

Sub :  Regarding Concrete  design mix (M30)

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 4 % 30 % 1.5 mm Expansion 145 Minutes 355 Minutes 16.7 N/mm2 32.0 N/mm2 40.5 N/mm2 1.48 2.92 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./523/2021-22/                                                      Dated: 23.02.2022

#### TEST REPORTS

Tested for 	:      	Senior Manager (FQAl)

NTPC Ltd. Tanda

Vidyut Nagar, Ambedkar Nagar -224238

(M/s Raj Construction )

Ref.		:    	NTPC/Tanda/FQA/20-21/849 Dated 08.01.2022

Tax Invoice No. : 	1287 IIT BHU dated 07.02.2022

Details of Sample :      Concrete Mix Design.

Test Required       	  (i) M-25 grade with admixture with Ultra Tech Cement, OPC

(ii) M-25 grade with admixture with Prism Cement, OPC

__________________________________________________________________________

Sub :  Evaluation of materials and concrete mix design for the  work of “construction of CISF Barrack, Dining Block, Boundary wall, Armory and Gate complex Package in township for Tanda STPP”  on M/s Raj Construction

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was (i) Ultra Tech Cement, OPC 43-grade, IS : 269, Y-2021 (ii) Prism Cement, OPC 43-grade, IS : 269, Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used SikaPlast 5202 NS.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./523/2021-22/                                                      Dated: 23.02.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for (i) M-25 grade with admixture with Ultra Tech Cement, OPC

(ii) M-25 grade with admixture with Prism Cement, OPC

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade with Ultra Tech Cement, OPC | 0.50 : 1 : 2.37 : 1.43 : 1.74 : 0.9* 0.695 : 1 : 1.87 : 1.30 : 1.47 : 0.9* | By weight By volume |
| 2. | M-25 Grade with Prism Cement, OPC | 0.50 : 1 : 2.37 : 1.43 : 1.74 : 0.9* 0.69 : 1 : 1.86 : 1.29 : 1.46 : 0.9* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture with Ultra Tech Cement, OPC | M-25 grade with admixture with Prism Cement, OPC |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 33.7 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 |
| 3 | Slump range (after 1 hours ) (mm) | 125-175 | 125-175 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 195.0 | 190.0 |
| 5 | Slump Achieved in the Laboratory(after 1 hours) (mm) | 135.0 | 140.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 30.0 39.0 | 28.0 37.5 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) SikaPlast 5202 NS (Litre) | 172.5 345.0 818.0 493.0 600.0 3.105 | 172.5 345.0 818.0 493.0 600.0 3.105 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./523/2021-22/                                                      Dated: 23.02.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design (i) M-25 grade with

admixture with Ultra Tech Cement, OPC (ii) M-25 grade with admixture with Prism Cement, OPC

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA ( sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 93.75 | 100.0 | - |
|  | 10 mm | 0.30 | 88.1 | - |
|  | 4.75 mm | 0.05 | 5.55 | 3.4 |
|  | 2.36 mm | - | 0.40 | 16.6 |
|  | 1.18 mm | - | 0.05 | 40.4 |
|  | 600 micron | - | - | 62.4 |
|  | 300 micron | - | - | 90.4 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 7.059 | 6.059 | 3.120 |
| 3. | Bulk Density (Kg./Litre) | 1.65 | 1.53 | 1.76 |
| 4. | Specific Gravity | 2.68 | 2.67 | 2.75 |
| 5. | Water absorption | 0.25 % | 0.25% | 0.96% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./523/2021-22/                                                      Dated: 23.02.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Senior Manager (FQAl)

NTPC Ltd. Tanda

Vidyut Nagar, Ambedkar Nagar -224238

(M/s Raj Construction )

Ref.		:    	NTPC/Tanda/FQA/20-21/849 Dated 08.01.2022

Details of Sample:	Cement: (i) Ultra Tech Cement, OPC 43-grade, IS : 269, Y-2021

(ii) Prism Cement, OPC 43-grade, IS : 269, Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i) M-25 grade with admixture with Ultra Tech Cement, OPC (ii) M-25 grade with admixture with Prism Cement, OPC

______________________________________________________________________________

Sub :  Evaluation of materials and concrete mix design for the  work of “construction of CISF Barrack, Dining Block, Boundary wall, Armory and Gate complex Package in township for Tanda STPP”  on M/s Raj Construction

| Sl.No. | Tests | Results |  |
| --- | --- | --- | --- |
|  |  | Ultra Tech Cement, OPC 43-grade, IS : 269, Y-2021 | Prism Cement, OPC 43-grade, IS : 269, Y-2021 |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 30 % 1.0 mm Expansion 85 Minutes 280 Minutes 26.1 N/mm2 37.5 N/mm2 46.9 N/mm2 1.39 3.10 | 5 % 33 % 1.0 mm Expansion 95 Minutes 290 Minutes 25.2 N/mm2 36.8 N/mm2 46.0 N/mm2 1.38 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./522/2021-22/                                                          Dated: 11.03.2022

#### TEST REPORTS

Tested for 	:      	Senior Manager (FQAl)

NTPC Ltd. Tanda

Vidyut Nagar, Ambedkar Nagar -224238

(M/s Raj Construction )

Ref.		:    	NTPC/Tanda/FQA/20-21/850 Dated 08.01.2022

Tax Invoice No. : 	1284 IIT BHU dated 07.02.2022

Details of Sample :      Concrete Mix Design.

Test Required       	  (i) M-25 grade with admixture with Ultra Tech Cement, OPC

(ii) M-25 grade with admixture with Prism Cement, OPC

__________________________________________________________________________

#### Sub :  Evaluation of materials and concrete mix design for the  work of “construction of Shopping centre, executive Club, Community Centre, Elevators, Fire fighting and Air conditioning systems Package in township for Tanda STPP” on M/s Raj Construction

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was (i) Ultra Tech Cement, OPC 43-grade, IS : 269, Y-2021 (ii) Prism Cement, OPC 43-grade, IS : 269, Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used SikaPlast 5202 NS.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./522/2021-22/                                                          Dated: 11.03.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for (i) M-25 grade with admixture with Ultra Tech Cement, OPC

(ii) M-25 grade with admixture with Prism Cement, OPC

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade with Ultra Tech Cement, OPC | 0.50 : 1 : 2.37 : 1.43 : 1.74 : 0.9* 0.695 : 1 : 1.87 : 1.30 : 1.47 : 0.9* | By weight By volume |
| 2. | M-25 Grade with Prism Cement, OPC | 0.50 : 1 : 2.37 : 1.43 : 1.74 : 0.9* 0.69 : 1 : 1.86 : 1.29 : 1.46 : 0.9* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture with Ultra Tech Cement, OPC | M-25 grade with admixture with Prism Cement, OPC |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 33.7 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 |
| 3 | Slump range (after 1 hours ) (mm) | 125-175 | 125-175 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 195.0 | 190.0 |
| 5 | Slump Achieved in the Laboratory(after 1 hours) (mm) | 135.0 | 140.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 30.0 39.0 | 28.0 37.5 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) SikaPlast 5202 NS (Litre) | 172.5 345.0 818.0 493.0 600.0 3.105 | 172.5 345.0 818.0 493.0 600.0 3.105 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./522/2021-22/                                                          Dated: 11.03.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design (i) M-25 grade with

admixture with Ultra Tech Cement, OPC (ii) M-25 grade with admixture with Prism Cement, OPC

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA ( sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 93.75 | 100.0 | - |
|  | 10 mm | 0.30 | 88.1 | - |
|  | 4.75 mm | 0.05 | 5.55 | 3.4 |
|  | 2.36 mm | - | 0.40 | 16.6 |
|  | 1.18 mm | - | 0.05 | 40.4 |
|  | 600 micron | - | - | 62.4 |
|  | 300 micron | - | - | 90.4 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 7.059 | 6.059 | 3.120 |
| 3. | Bulk Density (Kg./Litre) | 1.65 | 1.53 | 1.76 |
| 4. | Specific Gravity | 2.68 | 2.67 | 2.75 |
| 5. | Water absorption | 0.25 % | 0.25% | 0.96% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./522/2021-22/                                                            Dated: 11.03.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Senior Manager (FQAl)

NTPC Ltd. Tanda

Vidyut Nagar, Ambedkar Nagar -224238

(M/s Raj Construction )

Ref.		:    	NTPC/Tanda/FQA/20-21/850 Dated 08.01.2022

Details of Sample:	Cement: (i) Ultra Tech Cement, OPC 43-grade, IS : 269, Y-2021

(ii) Prism Cement, OPC 43-grade, IS : 269, Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i) M-25 grade with admixture with Ultra Tech Cement, OPC (ii) M-25 grade with admixture with Prism Cement, OPC

______________________________________________________________________________

#### Sub :  Evaluation of materials and concrete mix design for the  work of “construction of Shopping centre, executive Club, Community Centre, Elevators, Fire fighting and Air conditioning systems Package in township for Tanda STPP” on M/s Raj Construction

| Sl.No. | Tests | Results |  |
| --- | --- | --- | --- |
|  |  | Ultra Tech Cement, OPC 43-grade, IS : 269, Y-2021 | Prism Cement, OPC 43-grade, IS : 269, Y-2021 |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 30 % 1.0 mm Expansion 85 Minutes 280 Minutes 26.1 N/mm2 37.5 N/mm2 46.9 N/mm2 1.39 3.10 | 5 % 33 % 1.0 mm Expansion 95 Minutes 290 Minutes 25.2 N/mm2 36.8 N/mm2 46.0 N/mm2 1.38 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./506 /2021-22/                                                      Dated: 11.03.2022

#### TEST REPORTS

Tested for 	:      	Assistant Engineer (Con)/HJP

Office of the Dy. Chief Engineer/Con/II/Hajipur

West of Hajipur railway Station

East Central Railway, Hajipur

(M/s Lord Vishnu Constructions Private Limited)

Ref.			:    	DCE/C/II/HJP/Lord Vishnu  Dated 11.12.2021

Tax Invoice No. : 	 1235 IIT BHU dated 31.01.2022

Details of Sample :      	Concrete Mix Design.

Test Required       	(i) M-20 grade  (ii) M-25 grade

(iii) M-30 grade (iv) M-30 grade for piling

____________________________________________________________________________

Sub.: Design mix of  M-30 for piling, M-30, M-25 & M-20 concrete for the work of  “Earthwork in filling in embankment, blanketing, p.way work, extension of bridges. PF wall, extension of FOB, boundary wall etc. for the construction of line No. 4 at Ghoswar in connection with New BG rail Line HJP-SGL project.

.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489,  Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./506 /2021-22/                                                      Dated: 11.03.2022

#### (Page – 2(i))

#### TEST REPORTS

Recommended Mix Proportion for (i) M-20 grade and (ii) M-25 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M20 Grade | 0.55 : 1 : 2.10 : 1.16 : 2.15 0.721 : 1 : 1.56 : 0.97 : 1.62 | By weight By volume |
| 2. | M25 Grade | 0.50 : 1 : 1.83 : 1.05 : 1.95 0.655 : 1 : 1.36 : 0.88 : 1.47 | By weight By volume |

| Sl. No. | Test | M-20 grade | M-25 grade |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 27.6 | 33.7 |
| 2 | Slump range (mm) | 50-100 | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 90.0 | 85.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 21.5 30.5 | 25.0 35.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 192.5 350.0 735.0 406.0 753.0 | 192.5 385.0 705.0 405.0 751.0 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./506 /2021-22/                                                      Dated: 11.03.2022

#### (Page – 2(ii))

#### TEST REPORTS

Recommended Mix Proportion for (iii) M-30 grade and (iv) M-30 grade for piling

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M-30 grade | 0.45 : 1 : 1.58 : 1.00 : 1.49 0.59 : 1 : 1.18 : 0.84 : 1.12 | By weight By volume |
| 2. | M-30 grade for piling | 0.45 : 1 : 1.54 : 1.05 : 1.28 0.59 : 1 : 1.15 : 0.88 : 0.96 | By weight By volume |

| Sl. No. | Test | M-30 grade | M-30 grade for piling |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 39.9 | 39.9 |
| 2 | Slump range (mm) | 100-150 | 150-200 |
| 3 | Slump Achieved in the Laboratory (mm) | 135.0 | 175.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 29.5 42.5 | 29.0 42.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 198.0 440.0 695.0 440.0 656.0 | 204.75 455.0 701.0 477.0 582.0 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./506 /2021-22/                                                      Dated: 11.03.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design for (i) M-20 grade  (ii) M-25

grade (iii) M-30 grade (iv) M-30 grade for piling

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 87.1 | 100.0 | - |
|  | 10 mm | 0.40 | 86.8 | - |
|  | 4.75 mm | 0.05 | 1.90 | 2.8 |
|  | 2.36 mm | - | 0.25 | 7.8 |
|  | 1.18 mm | - | 0.10 | 23.0 |
|  | 600 micron | - | - | 57.0 |
|  | 300 micron | - | - | 88.0 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.125 | 6.118 | 2.776 |
| 3. | Bulk Density (Kg./Litre) | 1.74 | 1.56 | 1.76 |
| 4. | Specific Gravity | 2.86 | 2.85 | 2.71 |
| 5. | Water absorption | 0.25 % | 0.25 % | 0.71% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./506 /2021-22/                                                      Dated: 11.03.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Assistant Engineer (Con)/HJP

Office of the Dy. Chief Engineer/Con/II/Hajipur

West of Hajipur railway Station

East Central Railway, Hajipur

(M/s Lord Vishnu Constructions Private Limited)

Ref.			:    	DCE/C/II/HJP/Lord Vishnu  Dated 11.12.2021

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489,  Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i) M-20 grade                              (ii) M-25 grade (iii) M-30 grade (iv) M-30 grade for piling

______________________________________________________________________________

Sub.: Design mix of  M-30 for piling, M-30, M-25 & M-20 concrete for the work of  “Earthwork in filling in embankment, blanketing, p.way work, extension of bridges. PF wall, extension of FOB, boundary wall etc. for the construction of line No. 4 at Ghoswar in connection with New BG rail Line HJP-SGL project.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5mm Expansion 150 Minutes 365 Minutes 24.0 N/mm2 34.4 N/mm2 44.1 N/mm2 1.31 2.91 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./490/2021-22/                                                      Dated: 11.03.2022

#### TEST REPORTS

Tested for 	:      	Executive Engineer and Sr. Manager (Civil)-2

B.H.U. Project Varanasi

CPWD, Varanasi

(M/S Eastern Construction Co. )

Ref.		:    	 23(83)/ ch0,Pk0;w0ifj0ok0 -2/2021/1167 Dated: 14-12-2021

Tax Invoice No. : 	1156 IIT BHU dated 04.01.2022

Details of Sample :      Concrete Mix Design.

Test Required       	  M-30 grade with Plasticizer

________________________________________________________________________

Sub :  Design of cement concrete Mix M-30 with plasticizer.

Work: C/o Pocket-A (G+5) of Central Library including Water Supply, Sanitary Installation, Internal Electrical Installation, Fire Fighting System, Fire Alarm System, Lifts, DG Set, Air Conditioning, CCTV and development works at BHU, Varanasi

Agency:  M/s Eastern Construction Company

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489, W-01, M-01, Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used Fosroc Conplast SP 430 G8.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./490/2021-22/                                                      Dated: 11.03.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-30 grade with plasticizer

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-30 Grade | 0.45 : 1 : 1.77 : 1.16 : 1.74 : 0.8* 0.585 : 1 : 1.31 : 0.97 : 1.31 : 0.8* | By weight By volume |

| Sl. No. | Test | M-30 Grade with plasticizer |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 39.9 |
| 2 | Slump range (mm) | 125-175 |
| 3 | Slump Achieved in the Laboratory (mm) | 170.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) i) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 29.5 41.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Fosroc Conplast SP 430 G8 (Litre) | 180.0 400.0 708.0 464.0 696.0 3.20 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./490/2021-22/                                                      Dated: 11.03.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-30 grade with

plasticizer

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 87.55 | 100.0 | - |
|  | 10 mm | 0.35 | 89.05 | - |
|  | 4.75 mm | 0.05 | 7.30 | 2.4 |
|  | 2.36 mm | - | 0.40 | 6.2 |
|  | 1.18 mm | - | 0.05 | 17.4 |
|  | 600 micron | - | - | 49.4 |
|  | 300 micron | - | - | 89.8 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.121 | 6.032 | 2.642 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.55 | 1.75 |
| 4. | Specific Gravity | 2.84 | 2.82 | 2.70 |
| 5. | Water absorption | 0.35 % | 0.55 % | 0.70% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING  LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./490/2021-22/                                                      Dated: 11.03.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Executive Engineer and Sr. Manager (Civil)-2

B.H.U. Project Varanasi

CPWD, Varanasi

(M/S Eastern Construction Co. )

Ref.		:    	 23(83)/ ch0,Pk0;w0ifj0ok0 -2/2021/1167 Dated: 14-12-2021

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489,  W-01, M-01, Y-2022.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-30 grade with

plasticizer

______________________________________________________________________________

Sub :  Design of cement concrete Mix M-30 with plasticizer.

Work: C/o Pocket-A (G+5) of Central Library including Water Supply, Sanitary Installation, Internal Electrical Installation, Fire Fighting System, Fire Alarm System, Lifts, DG Set, Air Conditioning, CCTV and development works at BHU, Varanasi

Agency:  M/s Eastern Construction Company

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 36 % 1.5 mm Expansion 145 Minutes 375 Minutes 24.8 N/mm2 34.9 N/mm2 44.8 N/mm2 1.30 2.90 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./489/2021-22/                                                             Dated: 11.03.2022

#### TEST REPORTS

Tested for 	:      	Assistant Executive Engineer (Civil)-2

B.H.U. Project Varanasi

CPWD, Varanasi

(M/S Eastern Construction Co. )

Ref.		:    	Lk0vfHk0/ch0,Pk0;w0ifj0ok0 -2/2021/Camp Dated: 17-12-2021

Tax Invoice No. : 	1155 IIT BHU dated 04.01.2022

Details of Sample :      Concrete Mix Design.

Test Required       	  M-10 grade with Plasticizer

________________________________________________________________________

Sub :  Design of cement concrete Mix M-10 with plasticizer.

Work: C/o Pocket-A (G+5) of Central Library including Water Supply, Sanitary Installation, Internal Electrical Installation, Fire Fighting System, Fire Alarm System, Lifts, DG Set, Air Conditioning, CCTV and development works at BHU, Varanasi

Agency:  M/s Eastern Construction Company

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489, W-01, M-01, Y-2022

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used Fosroc Conplast SP 430 G8.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./489/2021-22/                                                             Dated: 11.03.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-10 grade with plasticizer

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-10 Grade | 0.70 : 1 : 3.37 : 1.80 : 2.70 : 0.8* 0.91 : 1 : 2.50 : 1.51 : 2.03 : 0.8* | By weight By volume |

| Sl. No. | Test | M-10 Grade with plasticizer |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 13.8 |
| 2 | Slump range (mm) | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 145.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 13.0 20.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Fosroc Conplast SP 430 G8 (Litre) | 178.5 255.0 860.0 459.0 688.0 2.04 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./489/2021-22/                                                             Dated: 11.03.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-10 grade with

plasticizer

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 87.55 | 100.0 | - |
|  | 10 mm | 0.35 | 89.05 | - |
|  | 4.75 mm | 0.05 | 7.30 | 2.4 |
|  | 2.36 mm | - | 0.40 | 6.2 |
|  | 1.18 mm | - | 0.05 | 17.4 |
|  | 600 micron | - | - | 49.4 |
|  | 300 micron | - | - | 89.8 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.121 | 6.032 | 2.642 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.55 | 1.75 |
| 4. | Specific Gravity | 2.84 | 2.82 | 2.70 |
| 5. | Water absorption | 0.35 % | 0.55 % | 0.70% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING  LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./489/2021-22/                                                             Dated: 11.03.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Assistant Executive Engineer (Civil)-2

B.H.U. Project Varanasi

CPWD, Varanasi

(M/S Eastern Construction Co. )

Ref.		:    	Lk0vfHk0/ch0,Pk0;w0ifj0ok0 -2/2021/Camp Dated: 17-12-2021

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489,  W-01, M-01, Y-2022.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-10 grade with

plasticizer

______________________________________________________________________________

Sub :  Design of cement concrete Mix M-10 with plasticizer.

Work: C/o Pocket-A (G+5) of Central Library including Water Supply, Sanitary Installation, Internal Electrical Installation, Fire Fighting System, Fire Alarm System, Lifts, DG Set, Air Conditioning, CCTV and development works at BHU, Varanasi

Agency:  M/s Eastern Construction Company

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 36 % 1.5 mm Expansion 145 Minutes 375 Minutes 24.8 N/mm2 34.9 N/mm2 44.8 N/mm2 1.30 2.90 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./469/2021-22/                                                      Dated: 11.03.2022

#### TEST REPORTS

Tested for 	:      	Sr. Section Engineer (Wks)

N.C.R.-Banda

(Sri Jagannath Transport Corporation)

Ref.		:    		 Nil     Dated : 18-11-2021

Tax Invoice No. : 	1081  IIT BHU dated 24.12.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  M-45 grade with admixture with OPC 53-grade Cement

___________________________________________________________________________

Sub: Regarding design mix concrete for the grade of M-45

Work: Khairar Bhimsen Section  Provision of Limited at  Subway in lieu of Level crossing No. S/74 at  LC No. 140918-9 in between patara-Luthara STN’s L.O.A. No. 8dated-JHS/Engg./E 202/29 at 18/11/2021.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes   (i) 20 mm. (ii) 10 mm

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 53-grade, IS : 269,  Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Fosroc Conplast SP 430 G8

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./469/2021-22/                                                      Dated: 11.03.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-45 grade with admixture with OPC 53-grade Cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-45 Grade | 0.35 : 1 : 1.60 : 1.07 : 1.61 : 0.8* 0.508 : 1 : 1.30 : 0.95 : 1.33 : 0.8* | By weight By volume |

| Sl. No. | Test | M-45 grade with admixture with OPC 53-grade Cement |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 56.6 |
| 2 | Slump range (mm) | 75-125 |
| 3 | Slump Achieved in the Laboratory (mm) | 105.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 45.5 59.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Fosroc Conplast SP 430 G8 (Litre) | 157.5 450.0 720.0 482.0 725.0 3.60 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./469/2021-22/                                                      Dated: 11.03.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-45 grade with

admixture with OPC 53-grade Cement

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA—20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 85.75 | 100.0 | - |
|  | 10 mm | 0.40 | 88.4 | - |
|  | 4.75 mm | 0.05 | 5.55 | 3.8 |
|  | 2.36 mm | - | 0.45 | 15.8 |
|  | 1.18 mm | - | 0.05 | 40.6 |
|  | 600 micron | - | - | 74.4 |
|  | 300 micron | - | - | 95.8 |
|  | 150 micron | - | - | 99.2 |
| 2. | Fineness Modulus | 7.138 | 6.056 | 3.296 |
| 3. | Bulk Density (Kg./Litre) | 1.75 | 1.63 | 1.79 |
| 4. | Specific Gravity | 2.88 | 2.88 | 2.69 |
| 5. | Water absorption | 0.10% | 0.30% | 1.23% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./469/2021-22/                                                      Dated: 11.03.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Sr. Section Engineer (Wks)

N.C.R.-Banda

(Sri Jagannath Transport Corporation)

Ref.		:    		 Nil     Dated : 18-11-2021

Details of Sample:	Cement: Ultra Tech  Cement, OPC 53-grade, IS : 269, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-45 grade with

admixture with OPC 53-grade Cement

______________________________________________________________________________

Sub: Regarding design mix concrete for the grade of M-45

Work: Khairar Bhimsen Section  Provision of Limited at  Subway in lieu of Level crossing No. S/74 at  LC No. 140918-9 in between patara-Luthara STN’s L.O.A. No. 8dated-JHS/Engg./E 202/29 at 18/11/2021.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 2 % 30 % 1.0 mm Expansion 85 Minutes 275 Minutes 32.7 N/mm2 38.4 N/mm2 54.4 N/mm2 1.45 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./468/2021-22/                                                          Dated: 11.03.2022

#### TEST REPORTS

Tested for 	:      	Sr. Section Engineer (Wks)

N.C.R.-Banda

(Sri Jagannath Transport Corporation)

Ref.		:    		 Nil     Dated : 18-11-2021

Tax Invoice No. : 	1080  IIT BHU dated 24.12.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  M-35 grade with admixture with OPC 53-grade Cement

___________________________________________________________________________

Sub: Regarding design mix concrete for the grade of M-35

Work: Khairar Bhimsen Section  Provision of Limited at  Subway in lieu of Level crossing No. S/74 at  LC No. 140918-9 in between patara-Luthara STN’s L.O.A. No. 8dated-JHS/Engg./E 202/29 at 18/11/2021

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes   (i) 20 mm. (ii) 10 mm

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 53-grade, IS : 269,  Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Fosroc Conplast SP 430 G8

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./468/2021-22/                                                          Dated: 11.03.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-35 grade with admixture with OPC 53-grade Cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-35 Grade | 0.40 : 1 : 1.92 : 1.23 : 1.85 : 0.8* 0.58 : 1 : 1.56 : 1.09 : 1.53 : 0.8* | By weight By volume |

| Sl. No. | Test | M-35 grade with admixture with OPC 53-grade Cement |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 45.4 |
| 2 | Slump range (mm) | 75-125 |
| 3 | Slump Achieved in the Laboratory (mm) | 115.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 37.5 50.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Fosroc Conplast SP 430 G8 (Litre) | 158.0 395.0 758.0 486.0 731.0 3.16 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./468/2021-22/                                                          Dated: 11.03.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-35 grade with

admixture with OPC 53-grade Cement

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA—20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 85.75 | 100.0 | - |
|  | 10 mm | 0.40 | 88.4 | - |
|  | 4.75 mm | 0.05 | 5.55 | 3.8 |
|  | 2.36 mm | - | 0.45 | 15.8 |
|  | 1.18 mm | - | 0.05 | 40.6 |
|  | 600 micron | - | - | 74.4 |
|  | 300 micron | - | - | 95.8 |
|  | 150 micron | - | - | 99.2 |
| 2. | Fineness Modulus | 7.138 | 6.056 | 3.296 |
| 3. | Bulk Density (Kg./Litre) | 1.75 | 1.63 | 1.79 |
| 4. | Specific Gravity | 2.88 | 2.88 | 2.69 |
| 5. | Water absorption | 0.10% | 0.30% | 1.23% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./468/2021-22/                                                          Dated: 11.03.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Sr. Section Engineer (Wks)

N.C.R.-Banda

(Sri Jagannath Transport Corporation)

Ref.		:    		 Nil     Dated : 18-11-2021

Details of Sample:	Cement: Ultra Tech  Cement, OPC 53-grade, IS : 269, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-35 grade with

admixture with OPC 53-grade Cement

______________________________________________________________________________

Sub: Regarding design mix concrete for the grade of M-35

Work: Khairar Bhimsen Section  Provision of Limited at  Subway in lieu of Level crossing No. S/74 at  LC No. 140918-9 in between patara-Luthara STN’s L.O.A. No. 8dated-JHS/Engg./E 202/29 at 18/11/2021

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 2 % 30 % 1.0 mm Expansion 85 Minutes 275 Minutes 32.7 N/mm2 38.4 N/mm2 54.4 N/mm2 1.45 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./467/2021-22/                                                      Dated: 11.03.2022

#### TEST REPORTS

Tested for 	:      	Sr. Section Engineer (Wks)

N.C.R.-Banda

(Sri Jagannath Transport Corporation)

Ref.		:    		 Nil     Dated : 18-11-2021

Tax Invoice No. : 	1079  IIT BHU dated 24.12.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  M-20 grade with PPC Cement

___________________________________________________________________________

Sub: Regarding design mix concrete for the grade of M-20

Work: Khairar Bhimsen Section  Provision of Limited at  Subway in lieu of Level crossing No. S/74 at  LC No. 140918-9 in between patara-Luthara STN’s L.O.A. No. 8dated-JHS/Engg./E 202/29 at 18/11/2021

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes   (i) 20 mm. (ii) 10 mm

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was DOUBLE  BULL Cement, PPC, IS : 1489, W-44, M-11, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./467/2021-22/                                                      Dated: 11.03.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-20 grade with PPC Cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M-20 Grade | 0.55 : 1 : 2.17 : 1.17 : 2.18 0.699 : 1 : 1.54 : 0.91 : 1.58 | By weight By volume |

| Sl. No. | Test | M-20 grade |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 27.6 |
| 2 | Slump range (mm) | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 85.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 22.0 30.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 189.75 345.0 749.0 404.0 752.0 |

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./467/2021-22/                                                      Dated: 11.03.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-20 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA—20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 85.75 | 100.0 | - |
|  | 10 mm | 0.40 | 88.4 | - |
|  | 4.75 mm | 0.05 | 5.55 | 3.8 |
|  | 2.36 mm | - | 0.45 | 15.8 |
|  | 1.18 mm | - | 0.05 | 40.6 |
|  | 600 micron | - | - | 74.4 |
|  | 300 micron | - | - | 95.8 |
|  | 150 micron | - | - | 99.2 |
| 2. | Fineness Modulus | 7.138 | 6.056 | 3.296 |
| 3. | Bulk Density (Kg./Litre) | 1.75 | 1.63 | 1.79 |
| 4. | Specific Gravity | 2.88 | 2.88 | 2.69 |
| 5. | Water absorption | 0.10% | 0.30% | 1.23% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./467/2021-22/                                                      Dated: 11.03.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Sr. Section Engineer (Wks)

N.C.R.-Banda

(Sri Jagannath Transport Corporation)

Ref.		:    		 Nil     Dated : 18-11-2021

Details of Sample:	Cement: DOUBLE  BULL Cement, PPC, IS : 1489, W-44, M-11, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-20 grade with PPC Cement

______________________________________________________________________________

Sub: Regarding design mix concrete for the grade of M-20

Work: Khairar Bhimsen Section  Provision of Limited at  Subway in lieu of Level crossing No. S/74 at  LC No. 140918-9 in between patara-Luthara STN’s L.O.A. No. 8dated-JHS/Engg./E 202/29 at 18/11/2021

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 4 % 33 % 1.5 mm Expansion 165 Minutes 345 Minutes 23.1 N/mm2 26.7 N/mm2 36.7 N/mm2 1.27 2.87 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./447/2021-22/                                                      Dated: 08.03.2022

#### TEST REPORTS

Tested for 	:      	Upper Project Manager

U.P.R.N.N. Ltd.

Kaushambi Unit

D-6/D-7, Industrial Estate Area

Structural Centre, Naini, Prayagraj-211010

(M/s Highway Construction Co.)

Ref.		:    		465/ byk0/dkS'kkECkh bdkÃ/jk0fu0fu0/  ikyhVsfDUkd / 2021 Dated : 27-10-2021

Tax Invoice No. : 	 1036 IIT BHU dated 14.12.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  M-25 grade

___________________________________________________________________________

Sub: Construction of Government Polytechnic, Fafamau, Prayagraj

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes   (i) 20 mm. (ii) 10 mm

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489, W-47, M-11, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./447/2021-22/                                                      Dated: 08.03.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 1.85 : 1.18 : 1.76 0.65 : 1 : 1.37 : 0.99 : 1.32 | By weight By volume |

| Sl. No. | Test | M-25 grade |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (mm) | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 75.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 26.5 37.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 192.5 385.0 712.0 454.0 678.0 |

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./447/2021-22/                                                      Dated: 08.03.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA—20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 85.25 | 100.0 | - |
|  | 10 mm | 0.20 | 85.9 | - |
|  | 4.75 mm | 0.05 | 0.95 | 2.4 |
|  | 2.36 mm | - | 0.40 | 9.8 |
|  | 1.18 mm | - | 0.05 | 30.4 |
|  | 600 micron | - | - | 64.2 |
|  | 300 micron | - | - | 91.8 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.143 | 6.127 | 2.976 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.55 | 1.75 |
| 4. | Specific Gravity | 2.83 | 2.84 | 2.68 |
| 5. | Water absorption | 0.20% | 0.40% | 1.45% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./447/2021-22/                                                      Dated: 08.03.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Upper Project Manager

U.P.R.N.N. Ltd.

Kaushambi Unit

D-6/D-7, Industrial Estate Area

Structural Centre, Naini, Prayagraj-211010

(M/s Highway Construction Co.)

Ref.		:    		465/ byk0/dkS'kkECkh bdkÃ/jk0fu0fu0/  ikyhVsfDUkd / 2021 Dated : 27-10-2021

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489, W-47, M-11, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade

______________________________________________________________________________

Sub: Construction of Government Polytechnic, Fafamau, Prayagraj

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 150 Minutes 385 Minutes 25.5 N/mm2 34.8 N/mm2 45.1 N/mm2 1.30 2.90 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./ 448(i)/2021-22/                                                          Dated: 09.03.2022

#### TEST REPORTS

Tested for 	:      	Staff Officer (Civil)

Northern Coalfields Ltd,

Block-B Area, Dist. Singrauli (M.P.)

(M/s Heavy Engineering Corporation Ltd.)

Ref.		:    		NCL/Block-B/SO (Civil)/2021/1310   Dated : 23-11-2021

Tax Invoice No. : 	1038 IIT BHU dated 14.12.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  M-20 grade with admixture and M-25 grade with admixture

__________________________________________________________________________

Sub :  Designing of M20 & M25 RCC mix design for  Construction of 2 nos. Sub-stations for CHP at Block-B OCP, Gorbi.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC IS :1489, W-47, M-11, Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Dr. FIXIT 101 LW+.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./ 448(i)/2021-22/                                                          Dated: 09.03.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-20 grade with admixture and M-25 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-20 Grade | 0.55 : 1 : 2.11 : 1.27 : 2.36 : 0.4* 0.715 : 1 : 1.57 : 1.04 : 1.75 : 0.4* | By weight By volume |
| 2. | M-25 Grade | 0.55 : 1 : 1.83 : 1.15 : 2.13 : 0.4* 0.65 : 1 : 1.36 : 0.94 : 1.58 : 0.4* | By weight By volume |

| Sl. No. | Test | M-20 grade with admixture | M-25 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 27.6 | 33.7 |
| 2 | Slump range (mm) | 25-75 | 25-75 |
| 3 | Slump Achieved in the Laboratory (mm) | 65.0 | 60.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 21.0 31.5 | 26.0 36.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Dr. FIXIT 101 LW+ (Litre) | 187.0 340.0 717.0 432.0 802.0 1.36 | 187.5 375.0 686.0 431.0 799.0 1.50 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./ 448(i)/2021-22/                                                          Dated: 09.03.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design M-20 grade with

admixture and M-25 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA ( sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 87.9 | 100.0 | - |
|  | 10 mm | 0.20 | 86.1 | - |
|  | 4.75 mm | 0.05 | 5.1 | 2.4 |
|  | 2.36 mm | - | 0.45 | 6.2 |
|  | 1.18 mm | - | 0.05 | 16.4 |
|  | 600 micron | - | - | 68.6 |
|  | 300 micron | - | - | 97.8 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.119 | 6.083 | 2.904 |
| 3. | Bulk Density (Kg./Litre) | 1.75 | 1.59 | 1.75 |
| 4. | Specific Gravity | 2.99 | 2.93 | 2.68 |
| 5. | Water absorption | 0.20 % | 0.60 % | 1.31 % |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./ 448(i)/2021-22/                                                          Dated: 09.03.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Staff Officer (Civil)

Northern Coalfields Ltd,

Block-B Area, Dist. Singrauli (M.P.)

(M/s Heavy Engineering Corporation Ltd.)

Ref.		:    		NCL/Block-B/SO (Civil)/2021/1310   Dated : 23-11-2021

Details of Sample:	Cement: Ultra Tech Cement, PPC IS :1489, W-47, M-11, Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-20 grade with admixture and M-25 grade with admixture

______________________________________________________________________________

Sub :  Designing of M20 & M25 RCC mix design for  Construction of 2 nos. Sub-stations for CHP at Block-B OCP, Gorbi.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 145 Minutes 380 Minutes 25.5 N/mm2 34.8 N/mm2 45.1 N/mm2 1.30 2.90 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./447/2021-22/                                                      Dated: 08.03.2022

#### TEST REPORTS

Tested for 	:      	Upper Project Manager

U.P.R.N.N. Ltd.

Kaushambi Unit

D-6/D-7, Industrial Estate Area

Structural Centre, Naini, Prayagraj-211010

(M/s Highway Construction Co.)

Ref.		:    		465/ byk0/dkS'kkECkh bdkÃ/jk0fu0fu0/  ikyhVsfDUkd / 2021 Dated : 27-10-2021

Tax Invoice No. : 	 1036 IIT BHU dated 14.12.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  M-25 grade

___________________________________________________________________________

Sub: Construction of Government Polytechnic, Fafamau, Prayagraj

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes   (i) 20 mm. (ii) 10 mm

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489, W-47, M-11, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./447/2021-22/                                                      Dated: 08.03.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 1.85 : 1.18 : 1.76 0.65 : 1 : 1.37 : 0.99 : 1.32 | By weight By volume |

| Sl. No. | Test | M-25 grade |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (mm) | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 75.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 26.5 37.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 192.5 385.0 712.0 454.0 678.0 |

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./447/2021-22/                                                      Dated: 08.03.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA—20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 85.25 | 100.0 | - |
|  | 10 mm | 0.20 | 85.9 | - |
|  | 4.75 mm | 0.05 | 0.95 | 2.4 |
|  | 2.36 mm | - | 0.40 | 9.8 |
|  | 1.18 mm | - | 0.05 | 30.4 |
|  | 600 micron | - | - | 64.2 |
|  | 300 micron | - | - | 91.8 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.143 | 6.127 | 2.976 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.55 | 1.75 |
| 4. | Specific Gravity | 2.83 | 2.84 | 2.68 |
| 5. | Water absorption | 0.20% | 0.40% | 1.45% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./447/2021-22/                                                      Dated: 08.03.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Upper Project Manager

U.P.R.N.N. Ltd.

Kaushambi Unit

D-6/D-7, Industrial Estate Area

Structural Centre, Naini, Prayagraj-211010

(M/s Highway Construction Co.)

Ref.		:    		465/ byk0/dkS'kkECkh bdkÃ/jk0fu0fu0/  ikyhVsfDUkd / 2021 Dated : 27-10-2021

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489, W-47, M-11, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade

______________________________________________________________________________

Sub: Construction of Government Polytechnic, Fafamau, Prayagraj

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 150 Minutes 385 Minutes 25.5 N/mm2 34.8 N/mm2 45.1 N/mm2 1.30 2.90 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./408/2021-22/                                                      Dated: 08.03.2022

#### TEST REPORTS

Tested for 	:      	Project Manager

UP Jal Nigam, Ganga Pollution Control Unit,

14 M.L.D. S.T.P. Campus,

Ramaipatti, Pakka Pokhara,Mirzapur-231001

(M/s Sombansi Enviro  Engg Private Limited)

Ref.		:    	1174/ dk;Z -10/26     dated 02.11.2021

Tax Invoice No. : 	977 IIT BHU dated 06.12.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  M-25 grade and M-30 grade

___________________________________________________________________________

Sub: Regarding Material Testing and Mix Design.

Work: Construction of  25 K.L.D. Co-Treatment Plant at Vindhyachal under Amrit Scheme.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes   (i) 20 mm. (ii) 10 mm

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 43-grade, IS : 269, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./408/2021-22/                                                      Dated: 08.03.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade and M-30 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 1.94 : 1.04 : 1.93 0.705 : 1 : 1.56 : 0.93 : 1.56 | By weight By volume |
| 1. | M-30 Grade | 0.45 : 1 : 1.65 : 0.92 : 1.72 0.635 : 1 : 1.33 : 0.83 : 1.39 | By weight By volume |

| Sl. No. | Test | M-25 grade | M-30 grade |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 |
| 2 | Slump range (mm) | 50-100 | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 60.0 | 65.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 28.0 35.0 | 31.5 43.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 190.0 380.0 737.0 395.0 733.0 | 191.25 425.0 701.0 391.0 731.0 |

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./408/2021-22/                                                      Dated: 08.03.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade and M-30

grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA—20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 85.4 | 100.0 | - |
|  | 10 mm | 0.20 | 89.4 | - |
|  | 4.75 mm | 0.05 | 6.4 | 4.2 |
|  | 2.36 mm | - | 0.40 | 15.4 |
|  | 1.18 mm | - | 0.10 | 44.0 |
|  | 600 micron | - | - | 76.2 |
|  | 300 micron | - | - | 95.2 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.144 | 6.037 | 3.340 |
| 3. | Bulk Density (Kg./Litre) | 1.74 | 1.57 | 1.75 |
| 4. | Specific Gravity | 2.83 | 2.81 | 2.65 |
| 5. | Water absorption | 0.65% | 0.60% | 1.58% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./408/2021-22/                                                      Dated: 08.03.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Project Manager

UP Jal Nigam, Ganga Pollution Control Unit,

14 M.L.D. S.T.P. Campus,

Ramaipatti, Pakka Pokhara,Mirzapur-231001

(M/s Sombansi Enviro  Engg Private Limited)

Ref.		:    	1174/ dk;Z -10/26     dated 02.11.2021

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grade, IS : 269, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade and M-30 grade

______________________________________________________________________________

Sub: Regarding Material Testing and Mix Design.

Work: Construction of  25 K.L.D. Co-Treatment Plant at Vindhyachal under Amrit Scheme.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 32 % 1.0 mm Expansion 120 Minutes 250 Minutes 26.4 N/mm2 37.9 N/mm2 47.2 N/mm2 1.41 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./513/2021-22/                                                       Dated: 23.02.2022

#### TEST REPORTS

Tested for 	:      	Sr. Manager (FQA)

FQA, NTPC-Vindhyachal

P.O. – Vindhanagar

Distt. Singrauli

(M/s Sangavi Enterprises)

Ref.		:    	VIN/IMS/FQA/DOC/2021 (Civil) /508  Dated: 15-12-2021

Tax Invoice No. : 	1252 IIT BHU dated 31.01.2022

Details of Sample :      Concrete Mix Design.

Test Required       	M-25 grade with admixture

__________________________________________________________________________

Sub :  Concrete Mix design of M-25 Grade.

Work: “Construction of New Fire Station Building at VSTPP”

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Emami Double Bull Cement, PPC IS :1489, W-47, M-11, Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Auramix 300.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./513/2021-22/                                                       Dated: 23.02.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.11 : 1.40 : 2.09 : 0.5* 0.645 : 1 : 1.53 : 1.09 : 1.53 : 0.5* | By weight By volume |

| Sl. No. | Test | M-25 Grade with admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (mm) | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 140.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 24.5 34.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Auramix 300 (Litre) | 177.5 355.0 749.0 497.0 742.0 1.775 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./513/2021-22/                                                       Dated: 23.02.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design M-25 grade with

admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA ( sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.95 | 100.0 | - |
|  | 10 mm | 0.30 | 85.65 | - |
|  | 4.75 mm | 0.05 | 2.40 | 2.0 |
|  | 2.36 mm | - | 0.40 | 7.6 |
|  | 1.18 mm | - | 0.05 | 37.0 |
|  | 600 micron | - | - | 59.4 |
|  | 300 micron | - | - | 94.4 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 7.127 | 6.115 | 2.992 |
| 3. | Bulk Density (Kg./Litre) | 1.76 | 1.66 | 1.78 |
| 4. | Specific Gravity | 3.00 | 2.98 | 2.72 |
| 5. | Water absorption | 1.06 % | 1.42 % | 0.77% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./513/2021-22/                                                       Dated: 23.02.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Sr. Manager (FQA)

FQA, NTPC-Vindhyachal

P.O. – Vindhanagar

Distt. Singrauli

(M/s Sangavi Enterprises)

Ref.		:    	VIN/IMS/FQA/DOC/2021 (Civil) /508  Dated: 15-12-2021

Details of Sample:	Cement: Emami Double Bull Cement, PPC IS :1489, W-47, M-11, Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with

admixture

______________________________________________________________________________

Sub :  Concrete Mix design of M-25 Grade.

Work: “Construction of New Fire Station Building at VSTPP”

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 33 % 1.5 mm Expansion 170 Minutes 310 Minutes 23.6 N/mm2 34.8 N/mm2 43.7 N/mm2 1.29 2.89 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./ 493/2021-22/                                                      Dated: 23.02.2022

#### TEST REPORTS

Tested for 	:      	M/s Kasana Builders Pvt. Ltd.

C-23, Sector-40, Noida-201303

Distt. Gautam Budh Nagar (U.P.)

Ref.		:    	KBPL/ARS/VARN/EPC/21-22   dated 31.12.2021

Tax Invoice No. : 	1180 IIT BHU dated 13.01.2022

Details of Sample :      Concrete Mix Design.

Test Required       	  (i) M-15 grade with admixture (ii) M-25 grade with admixture

(iii) M-30 grade with admixture

__________________________________________________________________________

Sub :  Concrete Mix  Design

Work:  Construction on EPC basis of Atal Residential School at Varanasi (U.P.), India.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489,   Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Sika Plastiment 2001 NS.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./ 493/2021-22/                                                      Dated: 23.02.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for (i) M-15 grade with admixture (ii) M-25 grade with admixture and

(iii) M-30 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-15 Grade | 0.60 : 1 : 2.80 : 1.53 : 2.30 : 0.6* 0.768 : 1 : 2.06 : 1.20 : 1.68 : 0.6* | By weight By volume |
| 2. | M-25 Grade | 0.50 : 1 : 2.15 : 1.28 : 1.92 : 0.6* 0.64 : 1 : 1.58 : 1.01 : 1.40 : 0.6* | By weight By volume |
| 3. | M-30 Grade | 0.45 : 1 : 1.85 : 1.15 : 1.72 : 0.6* 0.576 : 1 : 1.36 : 0.90 : 1.26 : 0.6* | By weight By volume |

| Sl. No. | Test | M-15 grade with admixture | M-25 grade with admixture | M-30 grade with admixture |
| --- | --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 20.8 | 33.7 | 39.9 |
| 2 | Slump range (mm) | 100-150 | 100-150 | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 130.0 | 125.0 | 120.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 15.0 24.0 | 27.0 35.5 | 29.5 42.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Sika Plastiment 2001 NS (Litre) | 177.0 295.0 826.0 451.0 679.0 1.77 | 177.5 355.0 763.0 454.0 682.0 2.13 | 177.75 395.0 731.0 454.0 679.0 2.37 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./ 493/2021-22/                                                      Dated: 23.02.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design (i) M-15 grade with

admixture (ii) M-25 grade with admixture and (iii) M-30 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA ( sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 87.9 | 100.0 | - |
|  | 10 mm | 0.10 | 86.4 | - |
|  | 4.75 mm | 0.05 | 2.90 | 5.2 |
|  | 2.36 mm | - | 0.55 | 20.4 |
|  | 1.18 mm | - | 0.05 | 49.6 |
|  | 600 micron | - | - | 78.4 |
|  | 300 micron | - | - | 95.6 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.120 | 6.101 | 3.482 |
| 3. | Bulk Density (Kg./Litre) | 1.75 | 1.63 | 1.74 |
| 4. | Specific Gravity | 2.85 | 2.84 | 2.64 |
| 5. | Water absorption | 0.20 % | 0.10 % | 1.06% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./ 493/2021-22/                                                      Dated: 23.02.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	M/s Kasana Builders Pvt. Ltd.

C-23, Sector-40, Noida-201303

Distt. Gautam Budh Nagar (U.P.)

Ref.		:    	KBPL/ARS/VARN/EPC/21-22   dated 31.12.2021

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489,   Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i) M-15 grade with admixture (ii) M-25 grade with admixture and (iii) M-30 grade with admixture

______________________________________________________________________________

Sub :  Concrete Mix  Design

Work:  Construction on EPC basis of Atal Residential School at Varanasi (U.P.), India.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 36 % 1.5 mm Expansion 145 Minutes 380 Minutes 24.0 N/mm2 33.9 N/mm2 44.0 N/mm2 1.28 2.88 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./494(ii)/2021-22/                                                      Dated: 22.02.2022

#### TEST REPORTS

Tested for 	:      	Sr. Manager (FQA-Civil)

NTPC Ltd. Tanda

Vidyut Nagar, Ambedkar Nagar -224238

(M/s Ashish Enterprises)

Ref.		:    	NTPC/Tanda/FQA/20-21/839  Dated 29.12.2021

Tax Invoice No. : 	1192 IIT BHU dated 17.01.2022

Details of Sample :      Concrete Mix Design.

Test Required       	  (i) M-25 grade with admixture with PPC Cement

__________________________________________________________________________

Sub :  Evaluation of materials and concrete mix design for the  work of “construction of D-type quarters, HOP bungalows and guiest house in township package for NTPC Tanda” on M/s Ashish Enterprises.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was M.P. Birla Perfect Plus Cement, PPC IS : 1489, Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Auramix 350.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./494(ii)/2021-22/                                                      Dated: 22.02.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for  M-25 grade with admixture with PPC Cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade with PPC | 0.50 : 1 : 2.46 : 1.35 : 1.65 : 0.6* 0.68 : 1 : 1.85 : 1.19 : 1.38 : 0.6* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture with PPC Cement |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (initial) (mm) | 150-200 |
| 3 | Slump range (after 1 hours ) (mm) | 125-175 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 190.0 |
| 5 | Slump Achieved in the Laboratory(after 1 hours) (mm) | 155.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 26.5 37.0 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Auramix 350 (Litre) | 172.5 345.0 849.0 466.0 569.0 2.07 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./494(ii)/2021-22/                                                      Dated: 22.02.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design M-25 grade with

admixture with PPC Cement

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA ( sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 95.25 | 100.0 | - |
|  | 10 mm | 0.40 | 86.9 | - |
|  | 4.75 mm | 0.05 | 0.75 | 3.2 |
|  | 2.36 mm | - | 0.30 | 13.4 |
|  | 1.18 mm | - | 0.05 | 38.0 |
|  | 600 micron | - | - | 73.8 |
|  | 300 micron | - | - | 96.8 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 7.043 | 6.120 | 3.240 |
| 3. | Bulk Density (Kg./Litre) | 1.63 | 1.54 | 1.81 |
| 4. | Specific Gravity | 2.71 | 2.67 | 2.69 |
| 5. | Water absorption | 0.50 % | 0.81 % | 1.26% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./494(ii)/2021-22/                                                      Dated: 22.02.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Sr. Manager (FQA-Civil)

NTPC Ltd. Tanda

Vidyut Nagar, Ambedkar Nagar -224238

(M/s Ashish Enterprises)

Ref.		:    	NTPC/Tanda/FQA/20-21/839  Dated 29.12.2021

Details of Sample:	Cement: M.P. Birla Perfect Plus Cement, PPC IS : 1489, Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with

admixture with PPC Cement

______________________________________________________________________________

Sub :  Evaluation of materials and concrete mix design for the  work of “construction of D-type quarters, HOP bungalows and guiest house in township package for NTPC Tanda” on M/s Ashish Enterprises.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 36 % 1.5 mm Expansion 150 Minutes 375 Minutes 23.2 N/mm2 34.5 N/mm2 44.4 N/mm2 1.36 2.96 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./494(i)/2021-22/                                                      Dated: 22.02.2022

#### TEST REPORTS

Tested for 	:      	Sr. Manager (FQA-Civil)

NTPC Ltd. Tanda

Vidyut Nagar, Ambedkar Nagar -224238

(M/s Ashish Enterprises)

Ref.		:    	NTPC/Tanda/FQA/20-21/839  Dated 29.12.2021

Tax Invoice No. : 	1192 IIT BHU dated 17.01.2022

Details of Sample :      Concrete Mix Design.

Test Required       	  (i) M-25 grade with admixture with OPC Cement

__________________________________________________________________________

Sub :  Evaluation of materials and concrete mix design for the  work of “construction of D-type quarters, HOP bungalows and guiest house in township package for NTPC Tanda” on M/s Ashish Enterprises.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Prism Cement, OPC-43 grade, IS : 269, Y-2022.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Auramix 350.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./494(i)/2021-22/                                                      Dated: 22.02.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for  M-25 grade with admixture with OPC Cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade with OPC | 0.50 : 1 : 2.53 : 1.39 : 1.70 : 0.6* 0.73 : 1 : 2.04 : 1.32 : 1.52 : 0.6* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture with OPC Cement |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (initial) (mm) | 150-200 |
| 3 | Slump range (after 1 hours ) (mm) | 125-175 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 180.0 |
| 5 | Slump Achieved in the Laboratory(after 1 hours) (mm) | 140.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 27.5 38.5 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Auramix 350 (Litre) | 170.0 340.0 860.0 473.0 578.0 2.04 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./494(i)/2021-22/                                                      Dated: 22.02.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design M-25 grade with

admixture with OPC Cement

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA ( sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 95.25 | 100.0 | - |
|  | 10 mm | 0.40 | 86.9 | - |
|  | 4.75 mm | 0.05 | 0.75 | 3.2 |
|  | 2.36 mm | - | 0.30 | 13.4 |
|  | 1.18 mm | - | 0.05 | 38.0 |
|  | 600 micron | - | - | 73.8 |
|  | 300 micron | - | - | 96.8 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 7.043 | 6.120 | 3.240 |
| 3. | Bulk Density (Kg./Litre) | 1.63 | 1.54 | 1.81 |
| 4. | Specific Gravity | 2.71 | 2.67 | 2.69 |
| 5. | Water absorption | 0.50 % | 0.81 % | 1.26% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./494(i)/2021-22/                                                      Dated: 22.02.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Sr. Manager (FQA-Civil)

NTPC Ltd. Tanda

Vidyut Nagar, Ambedkar Nagar -224238

(M/s Ashish Enterprises)

Ref.		:    	NTPC/Tanda/FQA/20-21/839  Dated 29.12.2021

Details of Sample:	Cement: Prism Cement, OPC-43 grade, IS : 269, Y-2022.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with

admixture with OPC Cement

______________________________________________________________________________

Sub :  Evaluation of materials and concrete mix design for the  work of “construction of D-type quarters, HOP bungalows and guiest house in township package for NTPC Tanda” on M/s Ashish Enterprises.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 5 % 34 % 1.0 mm Expansion 90 Minutes 285 Minutes 25.3 N/mm2 36.7 N/mm2 46.0 N/mm2 1.46 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./ 432/2021-22/                                                      Dated: 10.02.2022

#### TEST REPORTS

Tested for 	:      	M/s Anjana Tiwari

Krishna Nagar Private Colony,

Gorakhpur

Ref.		:    		Nil    Dated : 01-12-2021

Tax Invoice No. : 	1014 IIT BHU dated 10.12.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  M-25 grade with admixture and M-30 grade with admixture

__________________________________________________________________________

Sub :  Mix Design  for  8.5 MLD S.T.P. and related works under Balrampur Sewerage Scheme Phase-1 Packet 1

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was ACC Cement, PPC, IS : 1489, W-48, M-11,  Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Sika Plastiment 2001 NS.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./ 432/2021-22/                                                      Dated: 10.02.2022

#### (Page – 2(i))

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade with admixture and M-30 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.10 : 1.27 : 1.91 : 0.5* 0.66 : 1 : 1.57 : 1.09 : 1.49 : 0.5* | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 1.81 : 1.14 : 1.70 : 0.5* 0.594 : 1 : 1.35 : 0.98 : 1.33 : 0.5* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture | M-30 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 |
| 2 | Slump range (mm) | 100-150 | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 140.0 | 135.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 24.0 34.5 | 27.5 42.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Sika Plastiment 2001 NS (Litre) | 177.5 355.0 746.0 451.0 678.0 1.775 | 177.75 395.0 715.0 450.0 672.0 1.975 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./ 432/2021-22/                                                      Dated: 10.02.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design M-25 grade with

admixture and M-30 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA ( sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 85.55 | 100.0 | - |
|  | 10 mm | 0.10 | 93.75 | - |
|  | 4.75 mm | 0.05 | 6.9 | 4.2 |
|  | 2.36 mm | - | 0.55 | 14.0 |
|  | 1.18 mm | - | 0.05 | 32.6 |
|  | 600 micron | - | - | 60.4 |
|  | 300 micron | - | - | 92.6 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.143 | 5.988 | 3.028 |
| 3. | Bulk Density (Kg./Litre) | 1.69 | 1.54 | 1.77 |
| 4. | Specific Gravity | 2.77 | 2.68 | 2.70 |
| 5. | Water absorption | 0.10 % | 0.50 % | 0.92% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./ 432/2021-22/                                                      Dated: 10.02.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	M/s Anjana Tiwari

Krishna Nagar Private Colony,

Gorakhpur

Ref.		:    		Nil    Dated : 01-12-2021

Details of Sample:	Cement: ACC Cement, PPC, IS : 1489, W-48, M-11,  Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with admixture and M-30 grade with admixture

______________________________________________________________________________

Sub :  Mix Design  for  8.5 MLD S.T.P. and related works under Balrampur Sewerage Scheme Phase-1 Packet 1

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 9 % 40 % 1.5 mm Expansion 160 Minutes 390 Minutes 23.1 N/mm2 34.5 N/mm2 41.2 N/mm2 1.32 2.92 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./449/2021-22/                                                           Dated: 10.02.2022

#### TEST REPORTS

Tested for 	:      	Dy. G M (Engg.)

Office of Addl. General Manager (Engg.)

NBCC  (India) Ltd.,

IIT BHU Works, Near  Morvi Hostel,

BHU, Varanasi, (U.P.)

(M/s OSS Construction Private Limited)

Ref.		:    	 NBCC/IIT BHU/Faculty Aptt. /OSS/2021/55   Dated: 22.11.2021

Tax Invoice No. : 	1043  IIT BHU dated 16.12.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  M-25 grade with admixture

__________________________________________________________________________

Sub :  Concrete Mix Design M-25.

Project: Construction of Faculty Apartment (S+10) (Two blocks) near Vishwakarma Hostel at IIT BHU, Varanasi

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (Crushed sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489,   Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Muraplast FRV 341.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./449/2021-22/                                                           Dated: 10.02.2022

#### (Page – 2(i))

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade with admixture

| Grade of Concrete |  | Water : Cement : Crushed : CA-(10) : CA- (20) : Admixture Sand | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 1.99 : 1.09 : 1.64 : 1.1* 0.65 : 1 : 1.46 : 0.91 : 1.23 : 1.1* | By weight By volume |

| Sl. No. | Test | M-25 Grade with admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (mm) | 125-175 |
| 3 | Slump Achieved in the Laboratory (mm) | 130.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 24.0 34.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Crushed Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Muraplast FRV 341 (Litre) | 192.5 385.0 766.0 420.0 632.0 4.235 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./449/2021-22/                                                           Dated: 10.02.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix- M-25 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (Crushed sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 91.55 | 100.0 | - |
|  | 10 mm | 0.40 | 87.75 | - |
|  | 4.75 mm | 0.05 | 19.75 | 5.6 |
|  | 2.36 mm | - | 1.45 | 33.4 |
|  | 1.18 mm | - | 0.10 | 56.0 |
|  | 600 micron | - | - | 69.4 |
|  | 300 micron | - | - | 79.0 |
|  | 150 micron | - | - | 88.2 |
| 2. | Fineness Modulus | 7.080 | 5.910 | 3.316 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.56 | 1.77 |
| 4. | Specific Gravity | 2.73 | 2.83 | 2.68 |
| 5. | Water absorption | 1.06 % | 0.35 % | 1.59% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./449/2021-22/                                                           Dated: 10.02.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Dy. G M (Engg.)

Office of Addl. General Manager (Engg.)

NBCC  (India) Ltd.,

IIT BHU Works, Near  Morvi Hostel,

BHU, Varanasi, (U.P.)

(M/s OSS Construction Private Limited)

Ref.		:    	 NBCC/IIT BHU/Faculty Aptt. /OSS/2021/55   Dated: 22.11.2021

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489,  Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with admixture

______________________________________________________________________________

Sub :  Concrete Mix Design M-25.

Project: Construction of Faculty Apartment (S+10) (Two blocks) near Vishwakarma Hostel at IIT BHU, Varanasi

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 145 Minutes 375 Minutes 23.6 N/mm2 33.6 N/mm2 43.6 N/mm2 1.30 2.90 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./471(i)/2021-22/                                                      Dated: 09.02.2022

#### TEST REPORTS

Tested for 	:      	DGM-FQA

NTPC SSTPS, NTPC Limited

Singrauli Super Thermal Power Station, Po- Shaktinagar

Distt-Sonebhadra, U.P.- 231222

(M/s INHIBEO WATER SOLUTION AND TECHNOLOGIES PRIVATE LIMITED)

Ref.		:    	 031/FQA/Dec    Dated 09.12.2021

Tax Invoice No. : 	001106 IIT (BHU) dated 29.12.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  M-30 grade with admixture

__________________________________________________________________________

Sub :  Evaluation of material and concrete mix design (PO No. 5500038421)

Work: Design Supply and installation of Sewage Treatment Plant based on principle of moving Bed Bio reactor

MBBR for township of 1800 KLD capacity NTPC SSTPS.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 43-grade, IS : 269, W-49, M-12, Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Sika Plastiment 2001 NS.                                       .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./471(i)/2021-22/                                                      Dated: 09.02.2022

#### (Page – 2(i))

#### TEST REPORTS

Recommended Mix Proportion for M-30 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-30 Grade | 0.45 : 1 : 1.94 : 1.27 : 1.90 : 0.5* 0.635 : 1 : 1.58 : 1.09 : 1.52 : 0.5* | By weight By volume |

| Sl. No. | Test | M-30 Grade with admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 39.9 |
| 2 | Slump range (mm) | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 140.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 29.0 41.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Sika Plastiment 2001 NS (Litre) | 173.25 385.0 747.0 489.0 732.0 1.925 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./471(i)/2021-22/                                                      Dated: 09.02.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design M-30 grade with

admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 90.4 | 100.0 | - |
|  | 10 mm | 0.50 | 86.05 | - |
|  | 4.75 mm | 0.05 | 6.75 | 1.6 |
|  | 2.36 mm | - | 0.40 | 5.2 |
|  | 1.18 mm | - | 0.05 | 16.2 |
|  | 600 micron | - | - | 71.2 |
|  | 300 micron | - | - | 97.8 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.091 | 6.068 | 2.910 |
| 3. | Bulk Density (Kg./Litre) | 1.76 | 1.65 | 1.73 |
| 4. | Specific Gravity | 2.95 | 2.93 | 2.69 |
| 5. | Water absorption | 0.15 % | 0.25 % | 0.82% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./471(i)/2021-22/                                                      Dated: 09.02.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	DGM-FQA

NTPC SSTPS, NTPC Limited

Singrauli Super Thermal Power Station, Po- Shaktinagar

Distt-Sonebhadra, U.P.- 231222

(M/s INHIBEO WATER SOLUTION AND TECHNOLOGIES PRIVATE LIMITED)

Ref.		:    	 031/FQA/Dec    Dated 09.12.2021

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grade, IS : 269, W-49, M-12, Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-30 grade with admixture

______________________________________________________________________________

Sub :  Evaluation of material and concrete mix design (PO No. 5500038421)

Work: Design Supply and installation of Sewage Treatment Plant based on principle of moving Bed Bio reactor

MBBR for township of 1800 KLD capacity NTPC SSTPS.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 30 % 1.0 mm Expansion 90 Minutes 275 Minutes 26.0 N/mm2 35.9 N/mm2 46.5 N/mm2 1.41 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./514/2021-22/ 	                                                  Dated: 04.02.2022

#### TEST REPORTS

Tested for 	:      	General Manager (Civil)

Nigahi Area

Northern Coalfields Limited

Post: Nigahi Proejct, Distt. Singrauli, M.P. – 486884

(M/s Gopal Enterprises)

Ref.		:    		NGH/Civil /2021-22/360 dated : 31-10-2021

Tax Invoice No. : 	1253  IIT BHU dated 31.01.2021

Details of Sample :      Concrete Mix Design.

Test Required       	 M-40 grade

___________________________________________________________________________

Sub.: Testing of Materials

Name of work:   Construction of approach road to wharf wall and inter-link road along with drain at Nigahi Project.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes   (i) 20 mm. (ii) 10 mm

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 43-grade, IS : 269, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./514/2021-22/ 	                                                  Dated: 04.02.2022

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-40 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M-40 Grade | 0.375 : 1 : 1.24 : 0.93 : 1.40 0.525 : 1 : 0.99 : 0.83 : 1.13 | By weight By volume |

| Sl. No. | Test | M-40 grade |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 50.9 |
| 2 | Slump range (mm) | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 75.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 41.0 52.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 187.5 500.0 620.0 465.0 700.0 |

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./514/2021-22/ 	                                                  Dated: 04.02.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-40 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA—20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.1 | 100.0 | - |
|  | 10 mm | 0.15 | 86.2 | - |
|  | 4.75 mm | 0.05 | 1.5 | 2.6 |
|  | 2.36 mm | - | 0.40 | 7.2 |
|  | 1.18 mm | - | 0.10 | 20.4 |
|  | 600 micron | - | - | 49.8 |
|  | 300 micron | - | - | 89.6 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.137 | 6.112 | 2.686 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.56 | 1.75 |
| 4. | Specific Gravity | 2.85 | 2.81 | 2.70 |
| 5. | Water absorption | 0.20% | 0.66% | 1.16% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./514/2021-22/ 	                                                  Dated: 04.02.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	General Manager (Civil)

Nigahi Area

Northern Coalfields Limited

Post: Nigahi Proejct, Distt. Singrauli, M.P. – 486884

(M/s Gopal Enterprises)

Ref.		:    		NGH/Civil /2021-22/360 dated : 31-10-2021

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grade, IS : 269, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-40 grade

______________________________________________________________________________

Sub.: Testing of Materials

Name of work:   Construction of approach road to wharf wall and inter-link road along with drain at Nigahi Project.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 31 % 1.0 mm Expansion 85 Minutes 270 Minutes 25.6 N/mm2 35.3 N/mm2 46.0 N/mm2 1.40 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./433/2021-22/                                                      Dated: 02.02.2022

#### TEST REPORTS

Tested for 	:      	Project Manager

UP Jal Nigam (Rural)

Ganga Pollution Prevention Unit-II

Bhagwanpur,Varanasi-221005

(MIRZAPUR GHAZIPUR STPS PRIVATE LIMITED)

Ref.		:    	1324/ Mirzapur STP /02   dated 26.11.2021

Tax Invoice No. : 	1015 IIT BHU dated 10.12.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  M30 grade with without admixture and  M30 grade with admixture

__________________________________________________________________________

Sub :  Regarding Concrete  design mix (M30) with  & without  Admixture

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 43-grade, IS : 269,   Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used SikaPlast 5202 NS.                                       .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./433/2021-22/                                                      Dated: 02.02.2022

#### (Page – 2(i))

#### TEST REPORTS

Recommended Mix Proportion for M-30 grade without admixture and  M-30 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-30 Grade | 0.45 : 1 : 1.68 : 1.01 : 1.24 : - 0.63 : 1 : 1.32 : 0.91 : 1.00 : - | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 2.01 : 1.20 : 1.48 : 0.7* 0.63 : 1 : 1.58 : 1.08 : 1.20 : 0.7* | By weight By volume |

| Sl. No. | Test | M-30 grade without admixture | M-30 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 39.9 | 39.9 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 |
| 3 | Slump range (after 2 hours ) (mm) | 100-150 | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 180.0 | 185.0 |
| 5 | Slump Achieved in the Laboratory(after 2 hours) (mm) | 120.0 | 125.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 29.0 41.5 | 33.5 43.5 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) SikaPlast 5202 NS (Litre) | 202.5 450.0 756.0 455.0 558.0 - | 180.0 400.0 804.0 480.0 592.0 2.80 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./433/2021-22/                                                      Dated: 02.02.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design M-30 grade without

admixture and  M-30 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 85.75 | 100.0 | - |
|  | 10 mm | 0.30 | 89.55 | - |
|  | 4.75 mm | 0.05 | 0.90 | 5.0 |
|  | 2.36 mm | - | 0.40 | 18.8 |
|  | 1.18 mm | - | 0.10 | 46.6 |
|  | 600 micron | - | - | 78.8 |
|  | 300 micron | - | - | 95.6 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.139 | 6.091 | 3.438 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.56 | 1.78 |
| 4. | Specific Gravity | 2.82 | 2.80 | 2.68 |
| 5. | Water absorption | 0.65 % | 0.60 % | 1.50% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./433/2021-22/                                                      Dated: 02.02.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Project Manager

UP Jal Nigam (Rural)

Ganga Pollution Prevention Unit-II

Bhagwanpur,Varanasi-221005

(MIRZAPUR GHAZIPUR STPS PRIVATE LIMITED)

Ref.		:    	1324/ Mirzapur STP /02   dated 26.11.2021

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grade, IS : 269,   Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-30 grade without admixture and  M-30 grade with admixture

______________________________________________________________________________

Sub :  Regarding Concrete  design mix (M30) with  & without  Admixture

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 32 % 1.0 mm Expansion 115 Minutes 325 Minutes 30.3 N/mm2 36.0 N/mm2 46.3 N/mm2 1.40 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./371/2021-22/                                                      Dated: 11.01.2022

#### TEST REPORTS

Tested for 	:      	Assistant Executive Engineer

B.H.U. Project Varanasi

CPWD, Varanasi

(M/s Harisumir Vaastu Infracreations Private Limited)

Ref.		:    	Lk0dk0vfHk0@ch0,Pk0;w0izks0¼fl0½-2/2021/camp dated: 18-11-2021

Tax Invoice No. : 	000922 IIT (BHU) dated 26.11.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  M-30 grade with admixture

__________________________________________________________________________

Sub :  Design of cement concrete Mix M-30 with admixture.

Work: Renovation of Main Block Building SS Hospital with services and other related works (Package-I), BHU,

Varanasi (SH:SITC of fire Fighting and UG Tank balance work)

Agency: M/s Harisumir Vaastu Infracreations Private Limited

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489,   Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Fosroc Conplast SP 430 G8.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./371/2021-22/                                                      Dated: 11.01.2022

#### (Page – 2(i))

#### TEST REPORTS

Recommended Mix Proportion for M-30 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-30 Grade | 0.45 : 1 : 1.82 : 1.21 : 1.81 : 0.8* 0.585 : 1 : 1.35 : 1.01 : 1.36 : 0.8* | By weight By volume |

| Sl. No. | Test | M-30 Grade with admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 39.9 |
| 2 | Slump range (mm) | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 130.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 30.5 42.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Fosroc Conplast SP 430 G8 (Litre) | 175.5 390.0 710.0 472.0 706.0 3.12 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./371/2021-22/                                                      Dated: 11.01.2022

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix- M-30 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 88.6 | 100.0 | - |
|  | 10 mm | 0.25 | 88.55 | - |
|  | 4.75 mm | 0.05 | 6.90 | 2.4 |
|  | 2.36 mm | - | 0.30 | 5.8 |
|  | 1.18 mm | - | 0.10 | 19.6 |
|  | 600 micron | - | - | 57.4 |
|  | 300 micron | - | - | 86.8 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.111 | 6.072 | 2.710 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.55 | 1.75 |
| 4. | Specific Gravity | 2.84 | 2.84 | 2.67 |
| 5. | Water absorption | 0.35 % | 0.25 % | 1.02% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./371/2021-22/                                                      Dated: 11.01.2022

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Assistant Executive Engineer

B.H.U. Project Varanasi

CPWD, Varanasi

(M/s Harisumir Vaastu Infracreations Private Limited)

Ref.		:    	Lk0dk0vfHk0@ch0,Pk0;w0izks0¼fl0½-2/2021/camp dated: 18-11-2021

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489,  Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-30 grade with admixture

______________________________________________________________________________

Sub :  Design of cement concrete Mix M-30 with admixture.

Work: Renovation of Main Block Building SS Hospital with services and other related works (Package-I), BHU,

Varanasi (SH:SITC of fire Fighting and UG Tank balance work)

Agency: M/s Harisumir Vaastu Infracreations Private Limited

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 150 Minutes 375 Minutes 23.6 N/mm2 33.6 N/mm2 43.6 N/mm2 1.30 2.90 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./359/2021-22/                                                              Dated: 30.12.2021

#### TEST REPORTS

Tested for 	:      	Dy. General Manager (Engg.)

NBCC  (India) Ltd.,

IIT BHU Works, Near  Morvi Hostel,

BHU, Varanasi, (U.P.)

(M/s OSS Construction Private Limited)

Ref.		:    	NBCC/IIT BHU/Faculty /OSS/2021/35   Dated: 22.10.2021

Tax Invoice No. : 	000873 IIT (BHU) dated 17.11.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  M-25 grade with admixture

__________________________________________________________________________

Sub :  Concrete Mix Design M-25.

Project: Construction of Faculty Apartment (S+10) (Two blocks) near Vishwakarma Hostel at IIT BHU, Varanasi

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489,   Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Sikament- 3070 NS.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./359/2021-22/                                                              Dated: 30.12.2021

#### (Page – 2(i))

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.40 : 1.26 : 1.89 : 0.8* 0.65 : 1 : 1.76 : 1.05 : 1.42 : 0.8* | By weight By volume |

| Sl. No. | Test | M-25 Grade with admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (mm) | 125-175 |
| 3 | Slump Achieved in the Laboratory (mm) | 160.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 25.0 35.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Sikament- 3070 NS (Litre) | 172.5 345.0 830.0 435.0 652.0 2.76 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./359/2021-22/                                                              Dated: 30.12.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix- M-25 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 91.55 | 100.0 | - |
|  | 10 mm | 0.40 | 87.75 | - |
|  | 4.75 mm | 0.05 | 19.75 | 3.6 |
|  | 2.36 mm | - | 1.45 | 13.0 |
|  | 1.18 mm | - | 0.10 | 36.0 |
|  | 600 micron | - | - | 69.0 |
|  | 300 micron | - | - | 93.2 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.080 | 5.910 | 3.138 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.56 | 1.77 |
| 4. | Specific Gravity | 2.73 | 2.83 | 2.70 |
| 5. | Water absorption | 1.06 % | 0.35 % | 0.88% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./359/2021-22/                                                              Dated: 30.12.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Dy. General Manager (Engg.)

NBCC  (India) Ltd.,

IIT BHU Works, Near  Morvi Hostel,

BHU, Varanasi, (U.P.)

(M/s OSS Construction Private Limited)

Ref.		:    	NBCC/IIT BHU/Faculty /OSS/2021/35   Dated: 22.10.2021

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489,  Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with admixture

______________________________________________________________________________

Sub :  Concrete Mix Design M-25.

Project: Construction of Faculty Apartment (S+10) (Two blocks) near Vishwakarma Hostel at IIT BHU, Varanasi

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 145 Minutes 375 Minutes 23.6 N/mm2 33.6 N/mm2 43.6 N/mm2 1.30 2.90 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./406/2021-22/ 	                                                  Dated: 22.12.2021

#### TEST REPORTS

Tested for 	:      	XEN/Const/PRYJ

Office of the Dy. Chief Engineer/Const.-I

N.C. Railway, Prayagraj

(M/s S & P Infrastructure Developers Private Limited)

Ref.		:    		75-W/247/DY.CE/PRYJ  Dated : 04-03-2021

Tax Invoice No. : 	 000941 IIT BHU dated 26.03.2021

Details of Sample :      Concrete Mix Design.

Test Required       	 M-20 grade, M-30 grade and M-35 grade

___________________________________________________________________________

Sub.: Mix Design for M-20, M-30 & M-35 Grade of Concrete

Work: Earth work in embankment and cutting including blanketing construction of minor Bridges Retaining/Toe wall RUB/LHS extension, Side Drains and other allied works between Unchdih Including (Km 778.00)-Prayagraj Chheoki (Km 816.00) in C/W 3rd line between Pt. Deen Dayal Upadhyay  Jn.-Prayagraj of Prayagraj Division.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes   (i) 20 mm. (ii) 10 mm

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Prism Cement, OPC 43-grade, IS : 269, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./406/2021-22/ 	                                                  Dated: 22.12.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-20 grade, M-30 grade and M-35 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M-20 Grade | 0.55 : 1 : 2.20 : 1.21 : 2.25 0.776 : 1 : 1.76 : 1.09 : 1.83 | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 1.64 : 0.99 : 1.83 0.635 : 1 : 1.31 : 0.89 : 1.49 | By weight By volume |
| 3. | M-35 Grade | 0.40 : 1 : 1.40 : 0.88 : 1.63 0.564 : 1 : 1.12 : 0.79 : 1.33 | By weight By volume |

| Sl. No. | Test | M-20 grade | M-30 grade | M-35 grade |
| --- | --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 27.6 | 39.9 | 45.4 |
| 2 | Slump range (mm) | 50-100 | 50-100 | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 70.0 | 70.0 | 65.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 24.0 32.0 | 33.5 42.5 | 39.0 46.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 187.0 340.0 748.0 411.0 765.0 | 187.2 416.0 682.0 412.0 761.0 | 186.0 465.0 651.0 409.0 758.0 |

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./406/2021-22/ 	                                                  Dated: 22.12.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-20 grade, M-30 grade and M-35 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA—20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 89.3 | 100.0 | - |
|  | 10 mm | 0.20 | 86.85 | - |
|  | 4.75 mm | 0.05 | 5.40 | 2.6 |
|  | 2.36 mm | - | 0.30 | 7.4 |
|  | 1.18 mm | - | 0.10 | 22.8 |
|  | 600 micron | - | - | 57.4 |
|  | 300 micron | - | - | 86.8 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.105 | 6.074 | 2.760 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.57 | 1.76 |
| 4. | Specific Gravity | 2.84 | 2.83 | 2.69 |
| 5. | Water absorption | 0.15% | 0.20% | 1.00 % |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./406/2021-22/ 	                                                  Dated: 22.12.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	XEN/Const/PRYJ

Office of the Dy. Chief Engineer/Const.-I

N.C. Railway, Prayagraj

(M/s S & P Infrastructure Developers Private Limited)

Ref.		:    		75-W/247/DY.CE/PRYJ  Dated : 04-03-2021

Details of Sample:	Cement: Prism Cement, OPC 43-grade, IS : 269, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-20 grade,

M-30 grade and M-35 grade

______________________________________________________________________________

Sub.: Mix Design for M-20, M-30 & M-35 Grade of Concrete

Work: Earth work in embankment and cutting including blanketing construction of minor Bridges Retaining/Toe wall RUB/LHS extension, Side Drains and other allied works between Unchdih Including (Km 778.00)-Prayagraj Chheoki (Km 816.00) in C/W 3rd line between Pt. Deen Dayal Upadhyay  Jn.-Prayagraj of Prayagraj Division.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 5 % 34 % 1.0 mm Expansion 85 Minutes 275 Minutes 25.2 N/mm2 35.7 N/mm2 45.3 N/mm2 1.41 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./298/2021-22/                                                      Dated: 24.11.2021

#### TEST REPORTS

Tested for 	:      	Upper Project  Manager

Gorakhpur Unit

U.P.R.N.N. Ltd.

Auditorium  Site, Front of Karmal School,

Civil lines, Gorakhpur-273001

Ref.		:    		751/v0ifj0izc0/xks0/ ys£k /jk0fu0fu0 /20 21-22  Dated : 12-08-2021

Tax Invoice No. : 	 000699 IIT BHU dated 05.10.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  M-25 grade with admixture

__________________________________________________________________________

Sub :  Mix design  for Rajkiya Audyogik Prashikshan Sansthan Itawa, Siddhartha Nagar

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489,  W-41, M-10,  Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Auramix 200.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./298/2021-22/                                                      Dated: 24.11.2021

#### (Page – 2(i))

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.33 : 1.21 : 2.24 : 0.5* 0.65 : 1 : 1.68 : 1.01 : 1.68 : 0.5* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (mm) | 75-125 |
| 3 | Slump Achieved in the Laboratory (mm) | 105.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 24.0 37.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Auramix 200 (Litre) | 170.0 340.0 789.0 411.0 762.0 1.70 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./298/2021-22/                                                      Dated: 24.11.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix- M-25 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 87.55 | 100.0 | - |
|  | 10 mm | 0.25 | 88.55 | - |
|  | 4.75 mm | 0.05 | 5.40 | 5.2 |
|  | 2.36 mm | - | 0.40 | 23.0 |
|  | 1.18 mm | - | 0.05 | 43.2 |
|  | 600 micron | - | - | 67.2 |
|  | 300 micron | - | - | 92.8 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 7.122 | 6.056 | 3.302 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.55 | 1.79 |
| 4. | Specific Gravity | 2.84 | 2.83 | 2.74 |
| 5. | Water absorption | 0.10 % | 0.31 % | 1.53% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./298/2021-22/                                                      Dated: 24.11.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Upper Project  Manager

Gorakhpur Unit

U.P.R.N.N. Ltd.

Auditorium  Site, Front of Karmal School,

Civil lines, Gorakhpur-273001

Ref.		:    		751/v0ifj0izc0/xks0/ ys£k /jk0fu0fu0 /20 21-22  Dated : 12-08-2021

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489,  W-41, M-10,  Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with admixture

______________________________________________________________________________

Sub :  Mix design  for Rajkiya Audyogik Prashikshan Sansthan Itawa, Siddhartha Nagar

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 140 Minutes 370 Minutes 23.6 N/mm2 33.5 N/mm2 43.6 N/mm2 1.30 2.90 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./297/2021-22/                                                           Dated: 21.12.2021

#### TEST REPORTS

Tested for 	:      	Upper Project  Manager

Gorakhpur Unit

U.P.R.N.N. Ltd.

Auditorium  Site, Front of Karmal School,

Civil lines, Gorakhpur-273001

Ref.		:    		702/v0ifj0izc0/xks0/ ys£k /jkfufu /20 21-22  Dated : 04-08-2021

Tax Invoice No. : 	 000698 IIT BHU dated 05.10.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  (i)   M-10 grade with admixture   (ii)  M-20 grade with admixture

(iii)   M-25 grade with admixture  (iv)  M-30 grade with admixture

__________________________________________________________________________

Sub :  Mix design  for Siddhartha Vishwavidyalaya Kapilwastu Siddhartha Nagar

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489,  W-41, M-10,  Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Auramix 200.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./297/2021-22/                                                           Dated: 21.12.2021

#### (Page – 2(i))

#### TEST REPORTS

Recommended Mix Proportion for (i)   M-10 grade with admixture (ii)  M-20 grade with admixture for

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-10 Grade | 0.65 : 1 : 3.38 : 1.71 : 2.56 : 0.5* 0.845 : 1 : 2.45 : 1.43 : 1.92 : 0.5* | By weight By volume |
| 2. | M-20 Grade | 0.55 : 1 : 2.65 : 1.45 : 2.18 : 0.5* 0.715 : 1 : 1.92 : 1.22 : 1.64 : 0.5* | By weight By volume |

| Sl. No. | Test | M-10 grade with admixture | M-20 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 13.8 | 27.6 |
| 2 | Slump range (mm) | 100-150 | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 130.0 | 125.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2 | 13.0 22.0 | 20.5 32.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Auramix 200 (Litre) | 172.25 265.0 896.0 453.0 678.0 1.325 | 173.25 315.0 835.0 457.0 686.0 1.575 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./297/2021-22/                                                           Dated: 21.12.2021

#### (Page – 2(ii))

#### TEST REPORTS

Recommended Mix Proportion for (iii)   M-25 grade with admixture (iv)  M-30 grade with admixture for

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.33 : 1.33 : 1.99 : 0.5* 0.65 : 1 : 1.69 : 1.12 : 1.50 : 0.5* | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 2.00 : 1.20 : 1.79 : 0.5* 0.585 : 1 : 1.45 : 1.01 : 1.35 : 0.5* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture | M-30 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 |
| 2 | Slump range (mm) | 100-150 | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 125.0 | 125.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 23.0 36.0 | 27.0 42.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Auramix 200 (Litre) | 172.25 345.0 804.0 459.0 687.0 1.725 | 173.25 385.0 770.0 462.0 689.0 1.925 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./297/2021-22/                                                           Dated: 21.12.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design   (i)   M-10 grade with

admixture   (ii)  M-20 grade with admixture  (iii)   M-25 grade with admixture  (iv)  M-30 grade with

admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 87.55 | 100.0 | - |
|  | 10 mm | 0.25 | 88.55 | - |
|  | 4.75 mm | 0.05 | 5.40 | 5.2 |
|  | 2.36 mm | - | 0.40 | 23.0 |
|  | 1.18 mm | - | 0.05 | 43.2 |
|  | 600 micron | - | - | 67.2 |
|  | 300 micron | - | - | 92.8 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 7.122 | 6.056 | 3.302 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.55 | 1.79 |
| 4. | Specific Gravity | 2.84 | 2.83 | 2.74 |
| 5. | Water absorption | 0.10 % | 0.31 % | 1.53% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./297/2021-22/                                                           Dated: 21.12.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Upper Project  Manager

Gorakhpur Unit

U.P.R.N.N. Ltd.

Auditorium  Site, Front of Karmal School,

Civil lines, Gorakhpur-273001

Ref.		:    		702/v0ifj0izc0/xks0/ ys£k /jkfufu /20 21-22  Dated : 04-08-2021

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489,  W-41, M-10,  Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i)   M-10 grade with

admixture   (ii)  M-20 grade with admixture  (iii)   M-25 grade with admixture

(iv)  M-30 grade with admixture

______________________________________________________________________________

Sub :  Mix design  for Siddhartha Vishwavidyalaya Kapilwastu Siddhartha Nagar

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 140 Minutes 370 Minutes 23.6 N/mm2 33.5 N/mm2 43.6 N/mm2 1.30 2.90 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./343/ 2021-22/                                                       Dated: 21.12.2021

#### TEST REPORTS

Tested for 	:      	Additional General Manager

NTPC Limited

Korba Super Thermal Power Station

Korba (C.G.)

(M/s Mehrotra Buildcon Private Limited)

Ref.		:    	NTPC/KORBA/FGD/2021/    Dated 14.10.2021

Tax Invoice No. : 	 000822 IIT BHU dated 10.11.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  (i)  M-20 grade with admixture (ii) M-25 grade with admixture

(iii)  M-30 grade with admixture (iv) M-35 grade with admixture

__________________________________________________________________________

Sub :  Concrete Mix Design for NTPC Korba FGD Project Stage-I, II & III (3x200MW+3x500 MW +1x500MW), Korba

(C.G.)

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489, W-39, M-10, Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2009.

5.   Saturated surface dry aggregates were used as per IS: 10262-2009.

6.   Degree of control - Good.

7.   Admixture used Fosroc Auramix 201.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./343/ 2021-22/                                                       Dated: 21.12.2021

#### (Page – 2(i))

#### TEST REPORTS

Recommended Mix Proportion (i) M-20 grade with admixture (ii) M-25 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-20 Grade | 0.55 : 1 : 2.67 : 1.54 : 1.88 : 0.8* 0.715 : 1 : 1.95 : 1.23 : 1.40 : 0.8* | By weight By volume |
| 2. | M-25 Grade | 0.50 : 1 : 2.50 : 1.41 : 1.73 : 0.8* 0.65 : 1 : 1.83 : 1.12 : 1.29 : 0.8* | By weight By volume |

| Sl. No. | Test | M-20 grade with admixture | M-25 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 27.6 | 33.7 |
| 2 | Slump range (initial) (mm) | 175-225 | 175-225 |
| 3 | Slump range (after 90 minutes) (mm) | 125-175 | 125-175 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 200.0 | 195.0 |
| 5 | Slump Achieved in the Laboratory(after 90 minutes) (mm) | 135.0 | 140.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 20.5 28.5 | 24.0 35.0 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Fosroc Auramix 201 (Litre) | 176.0 320.0 854.0 493.0 602.0 2.56 | 175.0 350.0 875.0 494.0 606.0 2.80 |

*  Admixture used percentage by weight of cement.

Note: % of super plasticizer dose may slightly vary depending upon site situation

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./343/ 2021-22/                                                       Dated: 21.12.2021

#### (Page – 2(ii))

#### TEST REPORTS

Recommended Mix Proportion (iii) M-30 grade with admixture (iv) M-35 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-30 Grade | 0.45 : 1 : 2.03 : 1.27 : 1.54 : 0.8* 0.585 : 1 : 1.48 : 1.01 : 1.14 : 0.8* | By weight By volume |
| 2. | M-35 Grade | 0.40 : 1 : 1.71 : 1.11 : 1.36 : 0.8* 0.52 : 1 : 1.25 : 0.89 : 1.01 : 0.8* | By weight By volume |

| Sl. No. | Test | M-30 grade with admixture | M-35 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 39.9 | 45.4 |
| 2 | Slump range (initial) (mm) | 175-225 | 175-225 |
| 3 | Slump range (after 90 minutes) (mm) | 125-175 | 125-175 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 185.0 | 190.0 |
| 5 | Slump Achieved in the Laboratory(after 90 minutes) (mm) | 135.0 | 135.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 30.5 41.5 | 34.0 47.0 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Fosroc Auramix 201 (Litre) | 175.5 390.0 792.0 495.0 601.0 3.12 | 176.0 440.0 752.0 488.0 598.0 3.52 |

*  Admixture used percentage by weight of cement.

Note: % of super plasticizer dose may slightly vary depending upon site situation

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./343/ 2021-22/                                                       Dated: 21.12.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design   (i)  M-20 grade with

admixture (ii) M-25 grade with admixture (iii)  M-30 grade with admixture (iv) M-35 grade with

admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 88.25 | 100.0 | - |
|  | 10 mm | 0.55 | 92.55 | - |
|  | 4.75 mm | 0.05 | 1.55 | 3.2 |
|  | 2.36 mm | - | 0.45 | 13.8 |
|  | 1.18 mm | - | 0.05 | 39.8 |
|  | 600 micron | - | - | 73.8 |
|  | 300 micron | - | - | 95.0 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 7.112 | 6.054 | 3.242 |
| 3. | Bulk Density (Kg./Litre) | 1.75 | 1.63 | 1.78 |
| 4. | Specific Gravity | 2.85 | 2.81 | 2.70 |
| 5. | Water absorption | 0.15 % | 0.60 % | 1.03% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./343/ 2021-22/                                                       Dated: 21.12.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Additional General Manager

NTPC Limited

Korba Super Thermal Power Station

Korba (C.G.)

(M/s Mehrotra Buildcon Private Limited)

Ref.		:    	NTPC/KORBA/FGD/2021/    Dated 14.10.2021

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489, W-39, M-10, Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i)  M-20 grade with

admixture (ii) M-25 grade with admixture (iii)  M-30 grade with admixture (iv) M-35 grade with admixture

______________________________________________________________________________

Sub :  Concrete Mix Design for NTPC Korba FGD Project Stage-I, II & III (3x200MW+3x500 MW +1x500MW), Korba

(C.G.)

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 36 % 1.5 mm Expansion 145 Minutes 370 Minutes 24.0 N/mm2 33.9 N/mm2 44.0 N/mm2 1.30 2.90 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./445/2021-22/ 	                                                           Dated: 13.12.2021

#### TEST REPORTS

Tested for 	:      	Assistant Engineer,

B.H.U. Project Varanasi-3, CPWD, Varanasi

(M/s Kalyani Engineering Company)

Ref.		:    	Lk0vfHk0@ch0,Pk0;w0ifj0ok0-3/122 dated: 25-08-2021

Tax Invoice No. : 	 1033 IIT BHU  dated 10.12.2021

Details of Sample :      Concrete Mix Design.

Test Required       	 M-25 grade without plasticizer and M-25 grade with plasticizer

__________________________________________________________________________

Sub.: Design of cement concrete mix M-25 with plasticizer and without plasticizer.

Work: C/o Renovation and ext4ension of Pt. Omkarnath Thakur Auditorium at BHU, Varanasi including Intenrla EI, Fire fighting system, Fire alarm system, Audio Video & Stage lighting, DG set, Air conditioning, CCTV & Networking system and UPS system at BHU, Varanasi

Agency:  M/s Kalyani Engineering Company

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489,   Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2009.

5.   Saturated surface dry aggregates were used as per IS: 10262-2009.

6.   Degree of control - Good.

7.   Admixture used Sikament-2001 NS.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./445/2021-22/ 	                                                           Dated: 13.12.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade without plasticizer and M-25 grade with plasticizer

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 1.84 : 1.10 : 1.66 : - 0.645 : 1 : 1.30 : 0.86 : 1.24 : - | By weight By volume |
| 2. | M-25 Grade | 0.50 : 1 : 2.20 : 1.27 : 1.91 : 0.6* 0.645 : 1 : 1.55 : 0.99 : 1.42 : 0.6* | By weight By volume |

| Sl. No. | Test | M-25 grade without plasticizer | M-25 grade with plasticizer |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 33.7 |
| 2 | Slump range (mm) | 100-150 | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 125.0 | 130.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 23.0 35.5 | 22.0 35.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Sikament-2001 NS (Litre) | 197.5 395.0 727.0 435.0 656.0 - | 177.5 355.0 785.0 451.0 678.0 2.13 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./445/2021-22/ 	                                                           Dated: 13.12.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade without

plasticizer and M-25grade with plasticizer

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.25 | 100.0 | - |
|  | 10 mm | 0.35 | 86.75 | - |
|  | 4.75 mm | 0.05 | 14.75 | 5.6 |
|  | 2.36 mm | - | 2.55 | 19.2 |
|  | 1.18 mm | - | 1.05 | 46.2 |
|  | 600 micron | - | 0.10 | 77.8 |
|  | 300 micron | - | - | 96.4 |
|  | 150 micron | - | - | 99.4 |
| 2. | Fineness Modulus | 7.134 | 5.948 | 3.446 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.65 | 1.83 |
| 4. | Specific Gravity | 2.83 | 2.85 | 2.72 |
| 5. | Water absorption | 0.20 % | 0.26 % | 0.71% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./445/2021-22/ 	                                                           Dated: 13.12.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Assistant Engineer,

B.H.U. Project Varanasi-3, CPWD, Varanasi

(M/s Kalyani Engineering Company)

Ref.		:    	Lk0vfHk0@ch0,Pk0;w0ifj0ok0-3/122 dated: 25-08-2021

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489,   Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade without

plasticizer and M-25 grade with plasticizer

______________________________________________________________________________

Sub.: Design of cement concrete mix M-25 with plasticizer and without plasticizer.

Work: C/o Renovation and ext4ension of Pt. Omkarnath Thakur Auditorium at BHU, Varanasi including Intenrla EI, Fire fighting system, Fire alarm system, Audio Video & Stage lighting, DG set, Air conditioning, CCTV & Networking system and UPS system at BHU, Varanasi

Agency:  M/s Kalyani Engineering Company

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 36 % 1.5 mm Expansion 150 Minutes 380 Minutes 26.0 N/mm2 35.3 N/mm2 46.0 N/mm2 1.29 2.79 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./344/2021-22/                                                          Dated: 13.12.2021

#### TEST REPORTS

Tested for 	:      	General Manager (Tech)

M/s NCC Limited

Prabhapuram Colony, Opposite Sai Hospital Road

Beside DAV Public School, Robertsganj, Distt. Sonbhadra-231216

Ref.		:    	LWSM-PATWADH/NCC/2021/027   dated 14.10.2021

Tax Invoice No. : 	 000823 IIT BHU dated 10.11.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  (i)   M-25 grade with admixture

(ii)  M-30 grade with admixture

_____________________________________________________________________________

Sub :  design Mix M-25 & M-30 grade

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 43-grade, IS : 269, W-39, M-09, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used Sika Plastiment 2001 NS.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./344/2021-22/                                                          Dated: 13.12.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for (i)   M-25 grade with admixture (ii)  M-30 grade with admixture for

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.24 : 1.30 : 2.41 : 0.5* 0.70 : 1 : 1.78 : 1.17 : 1.95 : 0.5* | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 1.91 : 1.16 : 2.14 : 0.5* 0.63 : 1 : 1.52 : 1.04 : 1.73 : 0.5* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture | M-30 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 |
| 2 | Slump range (mm) | 25-75 | 25-75 |
| 3 | Slump Achieved in the Laboratory (mm) | 65.0 | 55.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 26.0 35.5 | 31.0 42.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Sika Plastiment 2001 NS (Litre) | 167.5 335.0 750.0 436.0 807.0 1.675 | 168.75 375.0 716.0 435.0 803.0 1.875 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./344/2021-22/                                                          Dated: 13.12.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design (i)   M-25 grade with

admixture (ii)  M-30 grade with admixture for

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 96.4 | 100.0 | - |
|  | 10 mm | 0.80 | 87.9 | - |
|  | 4.75 mm | 0.05 | 0.75 | 2.0 |
|  | 2.36 mm | - | 0.30 | 5.8 |
|  | 1.18 mm | - | 0.05 | 25.4 |
|  | 600 micron | - | - | 61.0 |
|  | 300 micron | - | - | 91.0 |
|  | 150 micron | - | - | 99.2 |
| 2. | Fineness Modulus | 7.028 | 6.110 | 2.844 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.56 | 1.76 |
| 4. | Specific Gravity | 2.85 | 2.85 | 2.69 |
| 5. | Water absorption | 0.10 % | 0.25 % | 1.15% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING  LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./344/2021-22/                                                          Dated: 13.12.2021

#### TEST REPORTS

Tested for 	:      	General Manager (Tech)

M/s NCC Limited

Prabhapuram Colony, Opposite Sai Hospital Road

Beside DAV Public School, Robertsganj, Distt. Sonbhadra-231216

Ref.		:    	LWSM-PATWADH/NCC/2021/027   dated 14.10.2021

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grade, IS : 269, W-39, M-09, Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i)   M-25 grade with

admixture (ii)  M-30 grade with admixture for

______________________________________________________________________________

Sub :  design Mix M-25 & M-30 grade

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 31 % 1.0 mm Expansion 115 Minutes 240 Minutes 26.5 N/mm2 38.0 N/mm2 47.3 N/mm2 1.40 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./274/2021-22/                                                         Dated: 09.12.2021

#### TEST REPORTS

Tested for 	:      	Dy. Project Manager

U.P. State Bridge Corporation Ltd.

Bridge Construction Unit Chandauli, House No. 19/2, Kailashpuri,

Near Mayabind Nursing Home,  Pt. Deen Dayal Upadhyay Nagar,

Distt. Chandauli-232101

Ref.		:    		 695/   fufonk dk;Z / 21-22  Dated : 19-07-20201

Tax Invoice No. : 	 000649 IIT BHU dated 21.09.2021

Details of Sample :      Concrete Mix Design.

Test Required       	(i)  M-35 grade for substructure, (ii)  M-35 grade for Pile work

(iii) M-40 grade for Deck slab & Crash barrier

__________________________________________________________________________

#### Sub :  Construction of Railway Span adjacent Viaduct span of 05 Nos., ROBs (excluding approaches) in lieu of level crossing No. 70/C/E, 71/C/E,

#### 75/C/T, 78/C/E and 81/C/T of Mughalsarai Division of east Central Railway. (Reg. Testing of Concrete Mix Design).

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 43-grade, IS : 269, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Conplast SP 430 G8.                                .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./274/2021-22/                                                         Dated: 09.12.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion (i)  M-35 grade for substructure (ii)  M-35 grade for Pile work (iii) M-40 grade for Deck slab & Crash barrier

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-35 Grade for substructure | 0.40 : 1 : 1.73 : 0.95 : 1.77 : 0.8* 0.552 : 1 : 1.33 : 0.83 : 1.41 : 0.8* | By weight By volume |
| 2. | M-35 Grade for Pile work | 0.40 : 1 : 1.75 : 1.05 : 1.58 : 0.8* 0.552 : 1 : 1.34 : 0.91 : 1.26 : 0.8* | By weight By volume |
| 3. | M-40 Grade for Deck Slab & Crash Barrier | 0.38 : 1 : 1.64 : 0.90 : 1.68 : 0.8* 0.524 : 1 : 1.26 : 0.78 : 1.34 : 0.8* | By weight By volume |

| Sl. No. | Test | M-35 Grade for substructure | M-35 Grade for Pile work | M-40 Grade for Deck Slab & Crash Barrier |
| --- | --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 45.4 | 45.4 | 50.9 |
| 2 | Slump range (initial) (mm) | 150-200 | 175-225 | 150-200 |
| 3 | Slump range (one hour ) (mm) | 125-175 | 150-200 | 125-175 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 195.0 | 210.0 | 200.0 |
| 5 | Slump Achieved in the Laboratory(after one hour (mm) | 125.0 | 165.0 | 130.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 35.0 46.5 | 37.0 48.0 | 40.5 52.5 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Conplast SP 430 G8 (Litre) | 168.0 420.0 727.0 399.0 743.0 3.36 | 170.0 425.0 744.0 446.0 672.0 3.40 | 167.2 440.0 722.0 396.0 739.0 3.52 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./274/2021-22/                                                         Dated: 09.12.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design (i)  M-35 grade for substructure (ii)  M-35 grade for Pile work (iii) M-40 grade for Deck slab & Crash barrier

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | F A (Sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 88.05 | 100.0 | - |
|  | 10 mm | 0.40 | 87.3 | - |
|  | 4.75 mm | 0.05 | 19.5 | 4.2 |
|  | 2.36 mm | - | 0.80 | 13.0 |
|  | 1.18 mm | - | 0.05 | 35.0 |
|  | 600 micron | - | - | 63.8 |
|  | 300 micron | - | - | 90.2 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.115 | 5.924 | 3.052 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.59 | 1.80 |
| 4. | Specific Gravity | 2.76 | 2.81 | 2.65 |
| 5. | Water absorption | 0.25 % | 0.47 % | 1.32% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./274/2021-22/                                                         Dated: 09.12.2021

#### TEST REPORTS

Tested for 	:      	Dy. Project Manager

U.P. State Bridge Corporation Ltd.

Bridge Construction Unit Chandauli, House No. 19/2, Kailashpuri,

Near Mayabind Nursing Home,  Pt. Deen Dayal Upadhyay Nagar,

Distt. Chandauli-232101

Ref.		:    		 695/   fufonk dk;Z / 21-22  Dated : 19-07-20201

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grade, IS : 269, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design  (i)  M-35 grade for

substructure (ii)  M-35 grade for Pile work (iii) M-40 grade for Deck slab & Crash barrier

______________________________________________________________________________

#### Sub :  Construction of Railway Span adjacent Viaduct span of 05 Nos., ROBs (excluding approaches) in lieu of level crossing No. 70/C/E, 71/C/E,

#### 75/C/T, 78/C/E and 81/C/T of Mughalsarai Division of east Central Railway. (Reg. Testing of Concrete Mix Design).

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3% 30 % 1.0 mm Expansion 110 Minutes 325 Minutes 32.1 N/mm2 41.1 N/mm2 48.0 N/mm2 1.38 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./19(iv)/2021-22/ 	                                                     Dated: 09.12.2021

#### TEST REPORTS

Tested for 	:      	Head-Buxar Project Site,

Larsen and Toubro  Limited,

2x660 MW,  Buxar Thermal Power Project

Ref.		:     	 L & T POWER/SJVN/BUXAR/EPC/21-22/25 Dated 10-09-2021

L & T POWER/SJVN/BUXAR/EPC/19-20/11A  Dated : 22-02-2021

Details of Sample :      	Concrete Mix Design with MP Birla Cement, OPC, chhatarpur source of aggregate, River sand and admixture BASF MASTER POLYHEED 8127

Test Required       	 (i) M-25 grade for civil work (ii) M-30 grade (iii)  M-35 grade (iv) M-40 grade

__________________________________________________________________________

Work: Concrete Mix Design

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (River sand) are given in Appendix –‘A’.

- The Cement supplied was M.P. Birla Concrecem Cement, OPC 43-grade, IS : 269, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used BASF MASTER POLYHEED 8127.                                .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./19(iv)/2021-22/ 	                                                     Dated: 09.12.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion (i) M-25 grade for civil work (ii) M-30 grade (iii)  M-35 grade (iv) M-40 grade

| Grade of Concrete |  | Water : Cement : River : CA-(10) : CA- (20) : Admixture Sand | Remark |
| --- | --- | --- | --- |
| 1. | M-25 grade for civil work | 0.48 : 1 : 1.96 : 1.21 : 1.81 : 0.8* 0.706 : 1 : 1.65 : 1.13 : 1.57 : 0.8* | By weight By volume |
| 2. | M-30 grade | 0.45 : 1 : 1.77 : 1.14 : 1.71 : 0.8* 0.662 : 1 : 1.49 : 1.06 : 1.49 : 0.8* | By weight By volume |
| 3. | M-35 grade | 0.40 : 1 : 1.60 : 1.07 : 1.61 : 0.9* 0.588 : 1 : 1.34 : 1.00 : 1.40 : 0.9* | By weight By volume |
| 4. | M-40 grade | 0.38 : 1 : 1.51 : 1.05 : 1.58 : 1.0* 0.559 : 1 : 1.27 : 0.98 : 1.37 : 1.0* | By weight By volume |

| Sl. No. | Test | M-25 grade for civil work | M-30 grade | M-35 grade | M-40 grade |
| --- | --- | --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 | 45.4 | 50.9 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 | 150-200 | 150-200 |
| 3 | Slump range (after one hour ) (mm) | 100-150 | 100-150 | 100-150 | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 190.0 | 190.0 | 185.0 | 180.0 |
| 5 | Slump Achieved in the Laboratory(after one hour)(mm) | 145.0 | 140.0 | 135.0 | 130.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 28.5 36.0 | 31.5 42.5 | 35.0 47.5 | 38.5 53.0 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) River Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) BASF MASTER POLYHEED 8127 (Litre) | 180.0 375.0 735.0 454.0 679.0 3.00 | 180.0 400.0 708.0 456.0 684.0 3.20 | 172.0 430.0 688.0 460.0 692.0 3.87 | 169.1 445.0 672.0 467.0 703.0 4.45 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./19(iv)/2021-22/ 	                                                     Dated: 09.12.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design (i) M-25 grade for civil work

(ii) M-30 grade (iii)  M-35 grade (iv) M-40 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | River sand |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 99.5 | 100.0 | - |
|  | 10 mm | 3.25 | 90.15 | - |
|  | 4.75 mm | 0.05 | 9.75 | 1.8 |
|  | 2.36 mm | - | 0.40 | 6.4 |
|  | 1.18 mm | - | 0.05 | 24.6 |
|  | 600 micron | - | - | 54.8 |
|  | 300 micron | - | - | 90.6 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 6.972 | 5.997 | 2.770 |
| 3. | Bulk Density (Kg./Litre) | 1.69 | 1.58 | 1.75 |
| 4. | Specific Gravity | 2.70 | 2.78 | 2.67 |
| 5. | Water absorption | 0.50 % | 0.55 % | 1.09% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./19(iv)/2021-22/ 	                                                     Dated: 09.12.2021

#### TEST REPORTS

Tested for 	:      	Head-Buxar Project Site,

Larsen and Toubro  Limited,

2x660 MW,  Buxar Thermal Power Project

Ref.		:     	 L & T POWER/SJVN/BUXAR/EPC/21-22/25 Dated 10-09-2021

L & T POWER/SJVN/BUXAR/EPC/19-20/11A  Dated : 22-02-2021

Details of Sample:	Cement: M.P. Birla Concrecem Cement, OPC 43-grade, IS : 269, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design  Design (i) M-25 grade for civil work   (ii) M-30 grade (iii)  M-35 grade (iv) M-40 grade

______________________________________________________________________________

Work: Concrete Mix Design

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 26 % 1.0 mm Expansion 115 Minutes 315 Minutes 26.8 N/mm2 36.7 N/mm2 46.5 N/mm2 1.47 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./19(iii)/2021-22/ 	                                                     Dated: 09.12.2021

#### TEST REPORTS

Tested for 	:      	Head-Buxar Project Site,

Larsen and Toubro  Limited,

2x660 MW,  Buxar Thermal Power Project

Ref.		:     	 L & T POWER/SJVN/BUXAR/EPC/21-22/25 Dated 10-09-2021

L & T POWER/SJVN/BUXAR/EPC/19-20/11A  Dated : 22-02-2021

Details of Sample :      	Concrete Mix Design with MP Birla Cement OPC, chhatarpur source of aggregate, River Sand and admixture CICO PLAST SUPER C711 & FOSROC SP430 ES2

Test Required       	 (i) M-25 grade for piling  work with CICO PLAST SUPER C711

(ii) M-25 grade for piling work with FOSROC SP430 ES2

__________________________________________________________________________

Work: Concrete Mix Design

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (River sand) are given in Appendix –‘A’.

- The Cement supplied was M.P. Birla Concrecem Cement, OPC 43-grade, IS : 269, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used CICO PLAST SUPER C711 /  FOSROC SP430 ES2.                                .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./19(iii)/2021-22/ 	                                                     Dated: 09.12.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion (i) M-25 grade for piling  work with CICO PLAT SUPER C711  (ii) M-25 grade

for piling work with FOSROC SP430 ES2

| Grade of Concrete |  | Water : Cement : River : CA-(10) : CA- (20) : Admixture Sand | Remark |
| --- | --- | --- | --- |
| 1. | M-25 grade with CICO PLAST SUPER C711 | 0.48 : 1 : 1.88 : 1.20 : 1.46 : 0.7* 0.706 : 1 : 1.58 : 1.12 : 1.27 : 0.7* | By weight By volume |
| 2. | M-25 grade with FOSROC SP430 ES2 | 0.48 : 1 : 1.88 : 1.20 : 1.46 : 0.7* 0.706 : 1 : 1.58 : 1.12 : 1.27 : 0.7* | By weight By volume |

| Sl. No. | Test | M-25 grade for Pile work with CICO PLAST SUPER C711 | M-25 grade for Pile work with FOSROC SP430 ES2 |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 33.7 |
| 2 | Slump range (initial) (mm) | 200-250 | 200-250 |
| 3 | Slump range (after one hour ) (mm) | 150-200 | 150-200 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 220.0 | 230.0 |
| 5 | Slump Achieved in the Laboratory(after one hour)(mm) | 175.0 | 180.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 26.5 35.5 | 25.5 35.0 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) River Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) )CICO PLAST SUPER C711 / FOSROC SP430 ES2 (Litre) | 192.0 400.0 752.0 480.0 584.0 2.80 | 192.0 400.0 752.0 480.0 584.0 2.80 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./19(iii)/2021-22/ 	                                                     Dated: 09.12.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design (i) M-25 grade for piling  work with

CICO PLAT SUPER C711  (ii) M-25 grade for piling work with FOSROC SP430 ES2

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | River sand |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 99.5 | 100.0 | - |
|  | 10 mm | 3.25 | 90.15 | - |
|  | 4.75 mm | 0.05 | 9.75 | 1.8 |
|  | 2.36 mm | - | 0.40 | 6.4 |
|  | 1.18 mm | - | 0.05 | 24.6 |
|  | 600 micron | - | - | 54.8 |
|  | 300 micron | - | - | 90.6 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 6.972 | 5.997 | 2.770 |
| 3. | Bulk Density (Kg./Litre) | 1.69 | 1.58 | 1.75 |
| 4. | Specific Gravity | 2.70 | 2.78 | 2.67 |
| 5. | Water absorption | 0.50 % | 0.55 % | 1.09% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./19(iii)/2021-22/ 	                                                     Dated: 09.12.2021

#### TEST REPORTS

Tested for 	:      	Head-Buxar Project Site,

Larsen and Toubro  Limited,

2x660 MW,  Buxar Thermal Power Project

Ref.		:     	 L & T POWER/SJVN/BUXAR/EPC/21-22/25 Dated 10-09-2021

L & T POWER/SJVN/BUXAR/EPC/19-20/11A  Dated : 22-02-2021

Details of Sample:	Cement: M.P. Birla Concrecem Cement, OPC 43-grade, IS : 269, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design  Design (i) M-25 grade for piling  work with CICO PLAT SUPER C711  (ii) M-25 grade for piling work with FOSROC SP430 ES2

______________________________________________________________________________

Work: Concrete Mix Design

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 26 % 1.0 mm Expansion 115 Minutes 315 Minutes 26.8 N/mm2 36.7 N/mm2 46.5 N/mm2 1.47 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./19(ii)/2021-22/ 	                                                     Dated: 09.12.2021

#### TEST REPORTS

Tested for 	:      	Head-Buxar Project Site,

Larsen and Toubro  Limited,

2x660 MW,  Buxar Thermal Power Project

Ref.		:     	 L & T POWER/SJVN/BUXAR/EPC/21-22/25 Dated 10-09-2021

L & T POWER/SJVN/BUXAR/EPC/19-20/11A  Dated : 22-02-2021

Details of Sample :      	Concrete Mix Design with Ultra Tech Cement OPC, chhatarpur source of aggregate, Crushed sand and admixture BASF MASTER POLYHEED 8127

Test Required       	 (i) M-25 grade for civil work (ii) M-25 grade for piling work,  (iii) M-30 grade          (iv)  M-35 grade

__________________________________________________________________________

Work: Concrete Mix Design

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (crushed sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 43-grade, IS : 269, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used BASF MASTER POLYHEED 8127.                                .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./19(ii)/2021-22/ 	                                                     Dated: 09.12.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion (i) M-25 grade for civil work  (ii)  M-25 grade for piling work, (iii) M-30 grade

(iv)  M-35 grade

| Grade of Concrete |  | Water : Cement : Crushed : CA-(10) : CA- (20) : Admixture Sand | Remark |
| --- | --- | --- | --- |
| 1. | M-25 grade for civil work | 0.50 : 1 : 2.14 : 1.20 : 1.80 : 1.2* 0.72 : 1 : 1.68 : 1.09 : 1.53 : 1.2* | By weight By volume |
| 2. | M-25 grade for pile work | 0.50 : 1 : 2.00 : 1.16 : 1.42 : 0.9* 0.72 : 1 : 1.57 : 1.06 : 1.21 : 0.9* | By weight By volume |
| 3. | M-30 grade | 0.45 : 1 : 1.85 : 1.08 : 1.62 : 1.2* 0.648 : 1 : 1.46 : 0.98 : 1.38 : 1.2* | By weight By volume |
| 4. | M-35 grade | 0.40 : 1 : 1.62 : 0.99 : 1.48 : 1.2* 0.576 : 1 : 1.28 : 0.90 : 1.26 : 1.2* | By weight By volume |

| Sl. No. | Test | M-25 grade for civil work | M-25 grade for Pile work | M-30 grade | M-35 grade |
| --- | --- | --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 33.7 | 39.9 | 45.4 |
| 2 | Slump range (initial) (mm) | 150-200 | 200-250 | 150-200 | 150-200 |
| 3 | Slump range (after one hour ) (mm) | 100-150 | 150-200 | 100-150 | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 180.0 | 210.0 | 170.0 | 170.0 |
| 5 | Slump Achieved in the Laboratory(after one hour)(mm) | 130.0 | 180.0 | 130.0 | 130.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2 | 25.0 34.0 | 27.0 34.5 | 28.0 41.0 | 32.5 46.0 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Crushed Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) BASF MASTER POLYHEED 8127 (Litre) | 185.0 370.0 792.0 444.0 666.0 4.44 | 200.0 400.0 800.0 664.0 568.0 3.60 | 184.5 410.0 759.0 443.0 664.0 4.92 | 180.0 450.0 729.0 445.0 666.0 5.40 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./19(ii)/2021-22/ 	                                                     Dated: 09.12.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design (i) M-25 grade for civil work

(ii)  M-25 grade for piling work, (iii) M-30 grade (iv)  M-35 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | Crushed sand |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 99.5 | 100.0 | - |
|  | 10 mm | 3.25 | 90.15 | - |
|  | 4.75 mm | 0.05 | 9.75 | 3.4 |
|  | 2.36 mm | - | 0.40 | 24.0 |
|  | 1.18 mm | - | 0.05 | 41.0 |
|  | 600 micron | - | - | 59.0 |
|  | 300 micron | - | - | 76.8 |
|  | 150 micron | - | - | 88.8 |
| 2. | Fineness Modulus | 6.972 | 5.997 | 2.936 |
| 3. | Bulk Density (Kg./Litre) | 1.69 | 1.58 | 1.83 |
| 4. | Specific Gravity | 2.70 | 2.78 | 2.81 |
| 5. | Water absorption | 0.50 % | 0.55 % | 0.77% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./19(ii)/2021-22/ 	                                                     Dated: 09.12.2021

#### TEST REPORTS

Tested for 	:      	Head-Buxar Project Site,

Larsen and Toubro  Limited,

2x660 MW,  Buxar Thermal Power Project

Ref.		:     	 L & T POWER/SJVN/BUXAR/EPC/21-22/25 Dated 10-09-2021

L & T POWER/SJVN/BUXAR/EPC/19-20/11A  Dated : 22-02-2021

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grade, IS : 269, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design  Design (i) M-25 grade for civil work  (ii)  M-25 grade for piling work, (iii) M-30 grade (iv)  M-35 grade

______________________________________________________________________________

Work: Concrete Mix Design

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 30 % 1.0 mm Expansion 110 Minutes 320 Minutes 32.4 N/mm2 40.9 N/mm2 47.6 N/mm2 1.44 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./19(i)/2021-22/ 	                                                     Dated: 09.12.2021

#### TEST REPORTS

Tested for 	:      	Head-Buxar Project Site,

Larsen and Toubro  Limited,

2x660 MW,  Buxar Thermal Power Project

Ref.		:     	 L & T POWER/SJVN/BUXAR/EPC/21-22/25 Dated 10-09-2021

L & T POWER/SJVN/BUXAR/EPC/19-20/11A  Dated : 22-02-2021

Details of Sample :      	Concrete Mix Design with MP Birla Cement OPC, chhatarpur source of aggregate, Crushed sand and admixture BASF MASTER POLYHEED 8127

Test Required       	 (i) M-25 grade for civil work (ii) M-25 grade for piling work, (iii) M-30 grade           (iv)  M-35 grade

__________________________________________________________________________

Work: Concrete Mix Design

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (crushed sand) are given in Appendix –‘A’.

- The Cement supplied was M.P. Birla Concrecem Cement, OPC 43-grade, IS : 269, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used BASF MASTER POLYHEED 8127.                                .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./19(i)/2021-22/ 	                                                     Dated: 09.12.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion (i) M-25 grade for civil work  (ii)  M-25 grade for piling work, (iii) M-30 grade

(iv)  M-35 grade

| Grade of Concrete |  | Water : Cement : Crushed : CA-(10) : CA- (20) : Admixture Sand | Remark |
| --- | --- | --- | --- |
| 1. | M-25 grade for civil work | 0.50 : 1 : 2.14 : 1.20 : 1.80 : 1.1* 0.735 : 1 : 1.72 : 1.12 : 1.57 : 1.1* | By weight By volume |
| 2. | M-25 grade for pile work | 0.50 : 1 : 2.00 : 1.16 : 1.42 : 0.8* 0.735 : 1 : 1.61 : 1.08 : 1.24 : 0.8* | By weight By volume |
| 3. | M-30 grade | 0.45 : 1 : 1.85 : 1.08 : 1.62 : 1.1* 0.662 : 1 : 1.49 : 1.01 : 1.41 : 1.1* | By weight By volume |
| 4. | M-35 grade | 0.40 : 1 : 1.62 : 0.99 : 1.48 : 1.1* 0.588 : 1 : 1.30 : 0.92 : 1.29 : 1.1* | By weight By volume |

| Sl. No. | Test | M-25 grade for civil work | M-25 grade for Pile work | M-30 grade | M-35 grade |
| --- | --- | --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 33.7 | 39.9 | 45.4 |
| 2 | Slump range (initial) (mm) | 150-200 | 200-250 | 150-200 | 150-200 |
| 3 | Slump range (after one hour ) (mm) | 100-150 | 150-200 | 100-150 | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 185.0 | 210.0 | 175.0 | 160.0 |
| 5 | Slump Achieved in the Laboratory(after one hour)(mm) | 135.0 | 170.0 | 125.0 | 120.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 26.5 34.5 | 29.0 35.0 | 30.0 41.5 | 33.0 46.0 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Crushed Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) BASF MASTER POLYHEED 8127 (Litre) | 185.0 370.0 792.0 444.0 666.0 4.07 | 200.0 400.0 800.0 464.0 568.0 3.20 | 184.5 410.0 759.0 443.0 664.0 4.51 | 180.0 450.0 729.0 445.0 666.0 4.95 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./19(i)/2021-22/ 	                                                     Dated: 09.12.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design (i) M-25 grade for civil work

(ii)  M-25 grade for piling work, (iii) M-30 grade (iv)  M-35 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | Crushed sand |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 99.5 | 100.0 | - |
|  | 10 mm | 3.25 | 90.15 | - |
|  | 4.75 mm | 0.05 | 9.75 | 3.4 |
|  | 2.36 mm | - | 0.40 | 24.0 |
|  | 1.18 mm | - | 0.05 | 41.0 |
|  | 600 micron | - | - | 59.0 |
|  | 300 micron | - | - | 76.8 |
|  | 150 micron | - | - | 88.8 |
| 2. | Fineness Modulus | 6.972 | 5.997 | 2.936 |
| 3. | Bulk Density (Kg./Litre) | 1.69 | 1.58 | 1.83 |
| 4. | Specific Gravity | 2.70 | 2.78 | 2.81 |
| 5. | Water absorption | 0.50 % | 0.55 % | 0.77% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./19(i)/2021-22/ 	                                                     Dated: 09.12.2021

#### TEST REPORTS

Tested for 	:      	Head-Buxar Project Site,

Larsen and Toubro  Limited,

2x660 MW,  Buxar Thermal Power Project

Ref.		:     	 L & T POWER/SJVN/BUXAR/EPC/21-22/25 Dated 10-09-2021

L & T POWER/SJVN/BUXAR/EPC/19-20/11A  Dated : 22-02-2021

Details of Sample:	Cement: M.P. Birla Concrecem Cement, OPC 43-grade, IS : 269, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design  Design (i) M-25 grade for civil work  (ii)  M-25 grade for piling work, (iii) M-30 grade (iv)  M-35 grade

______________________________________________________________________________

Work: Concrete Mix Design

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 26 % 1.0 mm Expansion 115 Minutes 315 Minutes 26.8 N/mm2 36.7 N/mm2 46.5 N/mm2 1.47 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./363/2021-22/ 	                                                  Dated: 07.12.2021

#### TEST REPORTS

Tested for 	:      	Area General Manager

Northern Coalfields Limited

Dudhichua Area

Post-Khadia, Distt Sonebhadra, U.P.-231222

(M/s HAMTEK TECHNOLOGIES INDIA PRIVATE LIMITED)

Ref.		:    		GM/DCH/21-22/1794  Dated : 20-10-2021

Tax Invoice No. : 	 000901 IIT BHU dated 24.11.2021

Details of Sample :      Concrete Mix Design.

Test Required       	 M-20 grade, M-25 grade and M-30 grade

___________________________________________________________________________

Sub: Design Mix of RCC grades.

Work: “Planning, design, Engineering, Construction, Fabrication, Erection, Supply Installation, Testing, Trial run and commissioning of Mechanised Loading at Wharf wall of Dudhichua OCP consisting of all civil, structural, electrical and Mechanical works and all other accessories and facilities required to make it complete in all respect on turnkey basis and operational and maintenance of plant for five years.

Name of contactor: M/s HAMTEK TECHNOLOGIES INDIA PRIVATE LIMITED

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes   (i) 20 mm. (ii) 10 mm

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech  Cement,  PPC, IS : 1489, W-42, M-10,  Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./363/2021-22/ 	                                                  Dated: 07.12.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-20 grade, M-25 grade and M-30 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M-20 Grade | 0.55 : 1 : 2.10 : 1.26 : 1.89 0.732 : 1 : 1.61 : 1.00 : 1.42 | By weight By volume |
| 2. | M-25 Grade | 0.50 : 1 : 1.82 : 1.15 : 1.72 0.665 : 1 : 1.40 : 0.92 : 1.29 | By weight By volume |
| 3. | M-30 Grade | 0.45 : 1 : 1.56 : 1.03 : 1.54 0.599 : 1 : 1.20 : 0.82 : 1.16 | By weight By volume |

| Sl. No. | Test | M-20 grade | M-25 grade | M-30 grade |
| --- | --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 27.6 | 33.7 | 39.9 |
| 2 | Slump range (mm) | 125-175 | 125-175 | 125-175 |
| 3 | Slump Achieved in the Laboratory (mm) | 140.0 | 135.0 | 130.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 20.0 29.5 | 23.5 35.5 | 28.0 42.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 198.0 360.0 756.0 454.0 680.0 | 198.0 396.0 721.0 455.0 681.0 | 198.0 440.0 687.0 453.0 678.0 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./363/2021-22/ 	                                                  Dated: 07.12.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-20 grade, M-25 grade and M-30 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA—20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 89.25 | 100.0 | - |
|  | 10 mm | 0.40 | 87.55 | - |
|  | 4.75 mm | 0.05 | 7.55 | 1.8 |
|  | 2.36 mm | - | 0.45 | 5.4 |
|  | 1.18 mm | - | 0.05 | 18.6 |
|  | 600 micron | - | - | 47.6 |
|  | 300 micron | - | - | 85.8 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.103 | 6.044 | 2.582 |
| 3. | Bulk Density (Kg./Litre) | 1.77 | 1.67 | 1.73 |
| 4. | Specific Gravity | 2.94 | 2.92 | 2.68 |
| 5. | Water absorption | 0.15% | 0.35 % | 1.32% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./363/2021-22/ 	                                                  Dated: 07.12.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Area General Manager

Northern Coalfields Limited

Dudhichua Area

Post-Khadia, Distt Sonebhadra, U.P.-231222

(M/s HAMTEK TECHNOLOGIES INDIA PRIVATE LIMITED)

Ref.		:    		GM/DCH/21-22/1794  Dated : 20-10-2021

Details of Sample:	Cement: Ultra Tech  Cement,  PPC, IS : 1489,  W-42, M-10,  Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-20 grade,                    M-25 grade and M-30 grade

______________________________________________________________________________

Sub: Design Mix of RCC grades.

Work: “Planning, design, Engineering, Construction, Fabrication, Erection, Supply Installation, Testing, Trial run and commissioning of Mechanised Loading at Wharf wall of Dudhichua OCP consisting of all civil, structural, electrical and Mechanical works and all other accessories and facilities required to make it complete in all respect on turnkey basis and operational and maintenance of plant for five years.

Name of contactor: M/s HAMTEK TECHNOLOGIES INDIA PRIVATE LIMITED

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7% 37 % 1.5 mm Expansion 145 Minutes 370 Minutes 26.1 N/mm2 35.5 N/mm2 45.7 N/mm2 1.33 2.93 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

RUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./351/2021-22/ 	                                                  Dated: 07.12.2021

#### TEST REPORTS

Tested for 	:      	Area General Manager

Northern Coalfields Limited

Amlohri Project, P.O.-Amlohri

Distt. Singrauli-M.P.- 486887

(M/s HAMTEK TECHNOLOGIES INDIA PRIVATE LIMITED)

Ref.		:    		NCL/AML/AGM/RLS/3099   Dated : 07-10-2021

Tax Invoice No. : 	 000850 IIT BHU dated 12.11.2021

Details of Sample :      Concrete Mix Design.

Test Required       	 M-20 grade, M-25 grade and M-30 grade

___________________________________________________________________________

Sub: Design Mix of RCC grades.

Work: “Planning, design, Engineering, Construction, Fabrication, Erection, Supply Installation, Testing, Trial run and commissioning of Coal Handling Plant Wharf Wall Siding (2.0 MTPA) for Amlohri OCP consisting of all Civil, structural, electrical and Mechanical works and all other accessories and facilities required to make it complete in all respect on turnkey basis and operational and maintenance of plant for five years.

Name of contactor: M/s HAMTEK TECHNOLOGIES INDIA PRIVATE LIMITED

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes   (i) 20 mm. (ii) 10 mm

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech  Cement,  PPC, IS : 1489, W-42, M-10,  Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./351/2021-22/ 	                                                  Dated: 07.12.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-20 grade, M-25 grade and M-30 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M-20 Grade | 0.55 : 1 : 2.10 : 1.26 : 1.89 0.732 : 1 : 1.61 : 1.00 : 1.42 | By weight By volume |
| 2. | M-25 Grade | 0.50 : 1 : 1.82 : 1.15 : 1.72 0.665 : 1 : 1.40 : 0.92 : 1.29 | By weight By volume |
| 3. | M-30 Grade | 0.45 : 1 : 1.56 : 1.03 : 1.54 0.599 : 1 : 1.20 : 0.82 : 1.16 | By weight By volume |

| Sl. No. | Test | M-20 grade | M-25 grade | M-30 grade |
| --- | --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 27.6 | 33.7 | 39.9 |
| 2 | Slump range (mm) | 125-175 | 125-175 | 125-175 |
| 3 | Slump Achieved in the Laboratory (mm) | 140.0 | 135.0 | 130.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 20.0 29.5 | 23.5 35.5 | 28.0 42.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 198.0 360.0 756.0 454.0 680.0 | 198.0 396.0 721.0 455.0 681.0 | 198.0 440.0 687.0 453.0 678.0 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./351/2021-22/ 	                                                  Dated: 07.12.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-20 grade, M-25 grade and M-30 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA—20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 89.25 | 100.0 | - |
|  | 10 mm | 0.40 | 87.55 | - |
|  | 4.75 mm | 0.05 | 7.55 | 1.8 |
|  | 2.36 mm | - | 0.45 | 5.4 |
|  | 1.18 mm | - | 0.05 | 18.6 |
|  | 600 micron | - | - | 47.6 |
|  | 300 micron | - | - | 85.8 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.103 | 6.044 | 2.582 |
| 3. | Bulk Density (Kg./Litre) | 1.77 | 1.67 | 1.73 |
| 4. | Specific Gravity | 2.94 | 2.92 | 2.68 |
| 5. | Water absorption | 0.15% | 0.35 % | 1.32% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./351/2021-22/ 	                                                  Dated: 07.12.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Area General Manager

Northern Coalfields Limited

Amlohri Project, P.O.-Amlohri

Distt. Singrauli-M.P.- 486887

(M/s HAMTEK TECHNOLOGIES INDIA PRIVATE LIMITED)

Ref.		:    		NCL/AML/AGM/RLS/3099   Dated : 07-10-2021

Details of Sample:	Cement: Ultra Tech  Cement,  PPC, IS : 1489,  W-42, M-10,  Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-20 grade,                    M-25 grade and M-30 grade

______________________________________________________________________________

Sub: Design Mix of RCC grades.

Work: “Planning, design, Engineering, Construction, Fabrication, Erection, Supply Installation, Testing, Trial run and commissioning of Coal Handling Plant Wharf Wall Siding (2.0 MTPA) for Amlohri OCP consisting of all Civil, structural, electrical and Mechanical works and all other accessories and facilities required to make it complete in all respect on turnkey basis and operational and maintenance of plant for five years.

Name of contactor: M/s HAMTEK TECHNOLOGIES INDIA PRIVATE LIMITED

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7% 37 % 1.5 mm Expansion 145 Minutes 370 Minutes 26.1 N/mm2 35.5 N/mm2 45.7 N/mm2 1.33 2.93 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./324(ii) 2021-22/                                                        Dated: 06.12.2021

#### TEST REPORTS

Tested for 	:      	Project  Manager

Larsen & Toubro Ltd.

RWS-Ghazipur

Ref.		:    		 LTCD/WET/WSD-BU/UP-JJM/VTWSS/PM-GZP-2020-21/061 Dated 13-09-2021

Tax Invoice No. : 	 000770 IIT BHU dated 20.10.2021

Details of Sample :      Concrete Mix Design.

Test Required       	    (i)  M-25 grade with admixture with Dehri Sand

(ii) M-30 grade with admixture with Dehri Sand

_____________________________________________________________________________

Sub :  Source approval and concrete mix design for various civil work to be carried out for RWS Ghazipur Tube well

Scheme under UPJJM.

Project : RWS-EMPANELMENT, Tube Well Scheme Ghazipur

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 43-grade, IS : 269, Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used Auramix 2000 .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./324(ii) 2021-22/                                                        Dated: 06.12.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion (i) M-25 grade with admixture and Dehri Sand  (ii) M-30 grade with  admixture

and Dehri Sand

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.30 : 1.35 : 2.03 : 0.5* 0.69 : 1 : 1.79 : 1.19 : 1.64 : 0.5* | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 1.98 : 1.21 : 1.82 : 0.5* 0.621 : 1 : 1.54 : 1.06 : 1.47 : 0.5* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture and Dehri Sand | M-30 grade with admixture and Dehri Sand |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 |
| 2 | Slump range (initial) (mm) | 100-150 | 100-150 |
| 3 | Slump range (after one hour ) (mm) | 50-100 | 50-100 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 135.0 | 145.0 |
| 5 | Slump Achieved in the Laboratory(after one hour) (mm) | 75.0 | 80.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 26.0 35.5 | 31.5 41.5 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Auramix 2000 (Litre) | 172.5 345.0 795.0 466.0 700.0 1.725 | 173.25 385.0 762.0 466.0 700.0 1.925 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./324(ii) 2021-22/                                                        Dated: 06.12.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design (i) M-25 grade with admixture

and Dehri Sand  (ii) M-30 grade with  admixture and Dehri Sand

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm (Source : Dala) | CA—10 mm (Source : Dala) | FA (sand) (Source : Dehri Sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.05 | 100.0 | - |
|  | 10 mm | 0.40 | 88.25 | - |
|  | 4.75 mm | 0.05 | 7.05 | 3.6 |
|  | 2.36 mm | - | 0.40 | 12.6 |
|  | 1.18 mm | - | 0.10 | 37.0 |
|  | 600 micron | - | - | 70.0 |
|  | 300 micron | - | - | 93.8 |
|  | 150 micron | - | - | 99.4 |
| 2. | Fineness Modulus | 7.135 | 6.042 | 3.164 |
| 3. | Bulk Density (Kg./Litre) | 1.71 | 1.57 | 1.77 |
| 4. | Specific Gravity | 2.86 | 2.84 | 2.68 |
| 5. | Water absorption | 0.20% | 0.48 % | 1.15% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING  LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./324(ii) 2021-22/                                                        Dated: 06.12.2021

#### TEST REPORTS

Tested for 	:      	Project  Manager

Larsen & Toubro Ltd.

RWS-Ghazipur

Ref.		:    		 LTCD/WET/WSD-BU/UP-JJM/VTWSS/PM-GZP-2020-21/061 Dated 13-09-2021

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grade, IS : 269, Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i) M-25 grade with admixture and Dehri Sand  (ii) M-30 grade with admixture and Dehri Sand

______________________________________________________________________________

Sub :  Source approval and concrete mix design for various civil work to be carried out for RWS Ghazipur Tube well

Scheme under UPJJM.

Project : RWS-EMPANELMENT, Tube Well Scheme Ghazipur

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 30 % 1.0 mm Expansion 115 Minutes 330 Minutes 31.9 N/mm2 40.7 N/mm2 47.6 N/mm2 1.38 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./324(i) 2021-22/                                                        Dated: 06.12.2021

#### TEST REPORTS

Tested for 	:      	Project  Manager

Larsen & Toubro Ltd.

RWS-Ghazipur

Ref.		:    		 LTCD/WET/WSD-BU/UP-JJM/VTWSS/PM-GZP-2020-21/061 Dated 13-09-2021

Tax Invoice No. : 	 000770 IIT BHU dated 20.10.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  (i)  M-25 grade with admixture with Chopan Sand

(ii) M-30 grade with admixture with Chopan Sand

_____________________________________________________________________________

Sub :  Source approval and concrete mix design for various civil work to be carried out for RWS Ghazipur Tube well

Scheme under UPJJM.

Project : RWS-EMPANELMENT, Tube Well Scheme Ghazipur

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 43-grade, IS : 269, Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used Auramix 2000 .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./324(i) 2021-22/                                                        Dated: 06.12.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion (i) M-25 grade with admixture and Chopan Sand  (ii) M-30 grade with admixture

and Chopan Sand

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.23 : 1.38 : 2.07 : 0.5* 0.69 : 1 : 1.75 : 1.21 : 1.67 : 0.5* | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 1.91 : 1.23 : 1.85 : 0.5* 0.621 : 1 : 1.50 : 1.08 : 1.49 : 0.5* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture and Chopan Sand | M-30 grade with admixture and Chopan Sand |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 |
| 2 | Slump range (initial) (mm) | 100-150 | 100-150 |
| 3 | Slump range (after one hour ) (mm) | 50-100 | 50-100 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 130.0 | 140.0 |
| 5 | Slump Achieved in the Laboratory(after one hour) (mm) | 70.0 | 75.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 25.0 34.5 | 30.5 41.0 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Auramix 2000 (Litre) | 172.5 345.0 770.0 476.0 714.0 1.725 | 173.25 385.0 735.0 474.0 712.0 1.925 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./324(i) 2021-22/                                                        Dated: 06.12.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design (i) M-25 grade with admixture

and Chopan Sand  (ii) M-30 grade with admixture and Chopan Sand

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm (Source : Dala) | CA—10 mm (Source : Dala) | FA (sand) (Source : Chopan Sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.05 | 100.0 | - |
|  | 10 mm | 0.40 | 88.25 | - |
|  | 4.75 mm | 0.05 | 7.05 | 1.8 |
|  | 2.36 mm | - | 0.40 | 7.6 |
|  | 1.18 mm | - | 0.10 | 26.2 |
|  | 600 micron | - | - | 59.6 |
|  | 300 micron | - | - | 90.4 |
|  | 150 micron | - | - | 99.2 |
| 2. | Fineness Modulus | 7.135 | 6.042 | 2.848 |
| 3. | Bulk Density (Kg./Litre) | 1.71 | 1.57 | 1.76 |
| 4. | Specific Gravity | 2.86 | 2.84 | 2.66 |
| 5. | Water absorption | 0.20% | 0.48 % | 1.35% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING  LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./324(i) 2021-22/                                                        Dated: 06.12.2021

#### TEST REPORTS

Tested for 	:      	Project  Manager

Larsen & Toubro Ltd.

RWS-Ghazipur

Ref.		:    		 LTCD/WET/WSD-BU/UP-JJM/VTWSS/PM-GZP-2020-21/061 Dated 13-09-2021

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grade, IS : 269, Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i) M-25 grade with admixture and Chopan Sand  (ii) M-30 grade with admixture and Chopan Sand

______________________________________________________________________________

Sub :  Source approval and concrete mix design for various civil work to be carried out for RWS Ghazipur Tube well

Scheme under UPJJM.

Project : RWS-EMPANELMENT, Tube Well Scheme Ghazipur

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 30 % 1.0 mm Expansion 115 Minutes 330 Minutes 31.9 N/mm2 40.7 N/mm2 47.6 N/mm2 1.38 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./320(i)/2021-22/                                                      Dated: 30.11.2021

#### TEST REPORTS

Tested for 	:      	AGM (FQA)

NTPC Limited

Kahalgaon Super Thermal Power Project

P.O. Kahalgaon, Distt. Bhagalpur (Bihar)

(M/s BHEL )

Ref.		:    	COA-6916/NTPC/FQA/2021 dated 09.10.2021

Tax Invoice No. : 	 000761 IIT BHU dated 13.10.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  (i)   M-25 grade with admixture for RCC work

(ii)  M-30 grade with admixture for Pile work

_____________________________________________________________________________

Sub :  Evaluation of materials and concrete mix design

Work: Flue Gas Desulphurisation  (FGD) System Package for Kahalgaon STPS Stage & II

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was NUVOCO Dura Guard Cement, PPC, IS : 1489, Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used Sikament 2004 NS.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./320(i)/2021-22/                                                      Dated: 30.11.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for (i)   M-25 grade with admixture for RCC work   (ii)  M-30 grade with

admixture for Pile work

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.45 : 1 : 1.90 : 1.16 : 1.74 : 0.6* 0.558 : 1 : 1.33 : 0.91 : 1.25 : 0.6* | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 1.87 : 1.23 : 1.51 : 0.8* 0.558 : 1 : 1.31 : 0.97 : 1.08 : 0.8* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture for RCC work | M-30 grade with admixture for Pile work |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 |
| 2 | Slump range (mm) | 100-150 | 150-200 |
| 3 | Slump Achieved in the Laboratory (mm) | 135.0 | 180.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 24.0 35.0 | 26.5 40.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Sikament 2004 NS (Litre) | 175.5 390.0 741.0 452.0 679.0 2.34 | 180.0 400.0 748.0 492.0 604.0 3.20 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./320(i)/2021-22/                                                      Dated: 30.11.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design (i)   M-25 grade with

admixture for RCC work   (ii)  M-30 grade with admixture for Pile work

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm (Source: Mirzachowki, Jharkhand) | CA—10 mm (Source: Mirzachowki, Jharkhand) | FA (sand) (Source : Paniwara, Banka ) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 85.55 | 100.0 | - |
|  | 10 mm | 0.15 | 87.55 | - |
|  | 4.75 mm | 0.05 | 7.50 | 2.2 |
|  | 2.36 mm | - | 0.55 | 5.4 |
|  | 1.18 mm | - | 0.05 | 19.8 |
|  | 600 micron | - | - | 46.4 |
|  | 300 micron | - | - | 96.2 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.143 | 6.040 | 2.690 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.58 | 1.77 |
| 4. | Specific Gravity | 2.82 | 2.78 | 2.75 |
| 5. | Water absorption | 1.01% | 2.15 % | 0.87% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING  LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./320(i)/2021-22/                                                      Dated: 30.11.2021

#### TEST REPORTS

Tested for 	:      	AGM (FQA)

NTPC Limited

Kahalgaon Super Thermal Power Project

P.O. Kahalgaon, Distt. Bhagalpur (Bihar)

(M/s BHEL )

Ref.		:    	COA-6916/NTPC/FQA/2021 dated 09.10.2021

Details of Sample:	Cement: NUVOCO Dura Guard Cement, PPC, IS : 1489, Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i)   M-25 grade with

admixture for RCC work   (ii)  M-30 grade with admixture for Pile work

______________________________________________________________________________

Sub :  Evaluation of materials and concrete mix design

Work: Flue Gas Desulphurisation  (FGD) System Package for Kahalgaon STPS Stage & II

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 36 % 1.5 mm Expansion 150 Minutes 365 Minutes 21.1 N/mm2 31.9 N/mm2 40.9 N/mm2 1.24 2.74 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./362/2021-22/                                                          Dated: 25.11.2021

#### TEST REPORTS

Tested for 	:      	Cluster Head

M/s Vensa Infrastructure Limited

ARS Project, Ayodhya

Ref.		:    		VIL/AED/ARS/UPPWD/ (2021-22)-001  Dated 20.09.2021

Tax Invoice No. : 	 000888 IIT BHU dated 18.11.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  (i)   M-20 grade, M-25 grade and M-30 grade with admixture with Ultra Tech Cement

(ii)   M-20 grade, M-25 grade and M-30 grade with admixture with ACC Suraksha

Cement

(iii)  M-20 grade, M-25 grade and M-30 grade with admixture with M.P. Birla Multicem

Cement

_____________________________________________________________________________

#### Sub :  Concrete Mix designs for Atal Residential School Project Ayodhya for Grade M-20, M-25 & M30 with 3 types of

#### cement

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was (i) Ultra Tech Cement, PPC, IS : 1489, W-33, M-08, Y-2021 (ii) ACC Suraksha Power Cement, PPC IS:1489, W-32, M-08, Y-2021 (iii) M.P. Birla Multicem Cement, PPC, IS : 1489, W-33, M-08, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used Conplast SP 430 G8 .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./362/2021-22/                                                          Dated: 25.11.2021

#### (Page – 2(i))

#### TEST REPORTS

Recommended Mix Proportion   (i)   M-20 grade, M-25 grade and M-30 grade with admixture with

Ultra Tech Cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-20 Grade | 0.55 : 1 : 2.58 : 1.42 : 1.74 : 0.7* 0.71 : 1 : 1.84 : 1.17 : 1.34 : 0.7* | By weight By volume |
| 2. | M-25 Grade | 0.50 : 1 : 2.28 : 1.31 : 1.60 : 0.7* 0.645 : 1 : 1.62 : 1.08 : 1.23 : 0.7* | By weight By volume |
| 3. | M-30 Grade | 0.45 : 1 : 1.96 : 1.18 : 1.44 : 0.7* 0.581 : 1 : 1.40 : 0.98 : 1.11 : 0.7* | By weight By volume |

| Sl. No. | Test | M-20 grade with Ultra Tech Cement | M-25 grade with Ultra Tech Cement | M-30 grade with Ultra Tech Cement |
| --- | --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 27.6 | 33.7 | 39.9 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 | 150-200 |
| 3 | Slump range (after one hour ) (mm) | 100-150 | 100-150 | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 185.0 | 185.0 | 180.0 |
| 5 | Slump Achieved in the Laboratory(after one hour)(mm) | 135.0 | 140.0 | 135.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 24.0 32.5 | 26.5 35.5 | 31.0 42.0 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Conplast SP 430 G8 (Litre) | 181.5 330.0 851.0 469.0 574.0 2.31 | 180.0 360.0 821.0 472.0 576.0 2.52 | 180.0 400.0 784.0 472.0 576.0 2.80 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./362/2021-22/                                                          Dated: 25.11.2021

#### (Page – 2(ii))

#### TEST REPORTS

Recommended Mix Proportion  (i)   M-20 grade, M-25 grade and M-30 grade with admixture with

ACC Suraksha Cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-20 Grade | 0.55 : 1 : 2.57 : 1.42 : 1.73 : 0.7* 0.688 : 1 : 1.77 : 1.14 : 1.29 : 0.7* | By weight By volume |
| 2. | M-25 Grade | 0.50 : 1 : 2.28 : 1.31 : 1.60 : 0.7* 0.625 : 1 : 1.57 : 1.05 : 1.19 : 0.7* | By weight By volume |
| 3. | M-30 Grade | 0.45 : 1 : 1.96 : 1.18 : 1.44 : 0.7* 0.563 : 1 : 1.35 : 0.95 : 1.07 : 0.7* | By weight By volume |

| Sl. No. | Test | M-20 grade with ACC Suraksha Cement | M-25 grade with ACC Suraksha Cement | M-30 grade with ACC Suraksha Cement |
| --- | --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 27.6 | 33.7 | 39.9 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 | 150-200 |
| 3 | Slump range (after one hour ) (mm) | 100-150 | 100-150 | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 175.0 | 170.0 | 175.0 |
| 5 | Slump Achieved in the Laboratory(after one hour)(mm) | 120.0 | 115.0 | 115.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 21.0 28.5 | 25.0 34.0 | 28.5 41.0 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Conplast SP 430 G8 (Litre) | 181.5 330.0 848.0 469.0 571.0 2.31 | 180.0 360.0 821.0 472.0 576.0 2.52 | 180.0 400.0 784.0 472.0 576.0 2.80 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./362/2021-22/                                                          Dated: 25.11.2021

#### (Page – 2(iii))

#### TEST REPORTS

Recommended Mix Proportion  (i)   M-20 grade, M-25 grade and M-30 grade with admixture with

M.P. Birla Multicem Cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-20 Grade | 0.55 : 1 : 2.58 : 1.42 : 1.74 : 0.7* 0.688 : 1 : 1.78 : 1.14 : 1.29 : 0.7* | By weight By volume |
| 2. | M-25 Grade | 0.50 : 1 : 2.28 : 1.31 : 1.60 : 0.7* 0.625 : 1 : 1.57 : 1.05 : 1.19 : 0.7* | By weight By volume |
| 3. | M-30 Grade | 0.45 : 1 : 1.96 : 1.18 : 1.44 : 0.7* 0.563 : 1 : 1.35 : 0.95 : 1.07 : 0.7* | By weight By volume |

| Sl. No. | Test | M-20 grade M.P. Birla Multicem Cement | M-25 grade M.P. Birla Multicem Cement | M-30 grade M.P. Birla Multicem Cement |
| --- | --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 27.6 | 33.7 | 39.9 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 | 150-200 |
| 3 | Slump range (after one hour ) (mm) | 100-150 | 100-150 | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 180.0 | 180.0 | 180.0 |
| 5 | Slump Achieved in the Laboratory(after one hour)(mm) | 130.0 | 135.0 | 135.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 22.0 30.0 | 26.0 34.5 | 30.0 41.5 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Conplast SP 430 G8 (Litre) | 181.5 330.0 848.0 469.0 574.0 2.31 | 180.0 360.0 821.0 472.0 576.0 2.52 | 180.0 400.0 784.0 472.0 576.0 2.80 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./362/2021-22/                                                          Dated: 25.11.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design   (i)   M-20 grade, M-25 grade

and M-30 grade with admixture with Ultra Tech Cement  (ii)   M-20 grade, M-25 grade and M-30 grade

with admixture with ACC Suraksha Cement (iii)   M-20 grade, M-25 grade and M-30 grade with admixture

with M.P. Birla Multicem Cement

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 90.9 | 100.0 | - |
|  | 10 mm | 1.25 | 87.75 | - |
|  | 4.75 mm | 0.05 | 2.30 | 5.0 |
|  | 2.36 mm | - | 0.20 | 16.0 |
|  | 1.18 mm | - | 0.05 | 37.4 |
|  | 600 micron | - | - | 69.0 |
|  | 300 micron | - | - | 89.0 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 7.078 | 6.097 | 3.152 |
| 3. | Bulk Density (Kg./Litre) | 1.68 | 1.56 | 1.81 |
| 4. | Specific Gravity | 2.69 | 2.77 | 2.72 |
| 5. | Water absorption | 0.10 % | 0.25 % | 1.07% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING  LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./362/2021-22/                                                          Dated: 25.11.2021

#### TEST REPORTS

Tested for 	:      	Cluster Head

M/s Vensa Infrastructure Limited

ARS Project, Ayodhya

Ref.		:    		VIL/AED/ARS/UPPWD/ (2021-22)-001  Dated 20.09.2021

Details of Sample:	Cement: (i) Ultra Tech Cement, PPC, IS : 1489, W-33, M-08, Y-2021

(ii) ACC Suraksha Power Cement, PPC IS:1489, W-32, M-08, Y-2021

(iii) M.P. Birla Multicem Cement, PPC, IS : 1489, W-33, M-08, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i)  M-20 grade, M-25 grade and M-30 grade with admixture with Ultra Tech Cement  (ii)  M-20 grade, M-25 grade and M-30 grade with admixture with ACC Suraksha Cement (iii)   M-20 grade,

M-25 grade and M-30 grade with admixture with M.P. Birla Multicem Cement

______________________________________________________________________________

#### Sub :  Concrete Mix designs for Atal Residential School Project Ayodhya for Grade M-20, M-25 & M30 with 3 types of

#### cement

| Sl.No. | Tests | Results |  |  |
| --- | --- | --- | --- | --- |
|  |  | (i) Ultra Tech Cement, PPC, IS : 1489, W-33, M-08, Y-2021 | (ii) ACC Suraksha Power Cement, PPC IS:1489, W-32, M-08, Y-2021 | (iii) M.P. Birla Multicem Cement, PPC, IS : 1489, W-33, M-08, Y-2021 |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 37 % 1.5 mm Expansion 145 Minutes 365 Minutes 26.0 N/mm2 35.3 N/mm2 46.0 N/mm2 1.29 2.89 | 8 % 39 % 1.5 mm Expansion 155 Minutes 385 Minutes 22.7 N/mm2 34.3 N/mm2 41.1 N/mm2 1.25 2.86 | 6 % 36 % 1.5 mm Expansion 150 Minutes 370 Minutes 24.7 N/mm2 35.1 N/mm2 44.7 N/mm2 1.25 2.85 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./326/2021-22/ 	                                                  Dated: 24.11.2021

#### TEST REPORTS

Tested for 	:      	Executive Engineer

OBRA ‘C’ CIVIL, Construction Division-I

U.P.R. Vidyut Utpadan Nigam Ltd.

P.O. : Obra-231219,District : Sonebhadra

(M/s Sincere Construction)

Ref.		:    		359/OCCCD-I/IIT BHU   Dated : 04-10-20201

Tax Invoice No. : 	 000772 IIT BHU dated 20.10.2021

Details of Sample :      Concrete Mix Design.

Test Required       	 M-10 grade, M-15 grade and M-25 grade ___________________________________________________________________________

Work: Work of 2nd raising of Chakari Ash dyke and allied civil work for 2x660 MW Obra ‘C’ Thermal Power Extension

Project.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes   (i) 20 mm. (ii) 10 mm

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech  Cement,  PPC, IS : 1489,  Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./326/2021-22/ 	                                                  Dated: 24.11.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-10 grade, M-15 grade and M-25 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M-10 Grade | 0.70 : 1 : 3.05 : 1.54 : 2.31 0.903 : 1 : 2.24 : 1.27 : 1.71 | By weight By volume |
| 2. | M-15 Grade | 0.60 : 1 : 2.43 : 1.32 : 1.98 0.774 : 1 : 1.78 : 1.09 : 1.47 | By weight By volume |
| 3. | M-25 Grade | 0.48 : 1 : 1.77 : 1.05 : 1.57 0.619 : 1 : 1.30 : 0.86 : 1.16 | By weight By volume |

| Sl. No. | Test | M-10 grade | M-15 grade | M-25 grade |
| --- | --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 13.8 | 20.8 | 33.7 |
| 2 | Slump range (mm) | 100-150 | 100-150 | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 130.0 | 125.0 | 115.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2 | 11.0 17.5 | 16.5 23.0 | 24.0 35.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 196.0 280.0 854.0 431.0 647.0 | 196.8 328.0 797.0 433.0 650.0 | 196.8 410.0 726.0 431.0 644.0 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./326/2021-22/ 	                                                  Dated: 24.11.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-10 grade,               M-15 grade and M-25 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA—20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 88.4 | 100.0 | - |
|  | 10 mm | 0.15 | 90.55 | - |
|  | 4.75 mm | 0.05 | 8.40 | 3.2 |
|  | 2.36 mm | - | 0.75 | 13.6 |
|  | 1.18 mm | - | 0.05 | 40.2 |
|  | 600 micron | - | - | 81.6 |
|  | 300 micron | - | - | 98.2 |
|  | 150 micron | - | - | 99.2 |
| 2. | Fineness Modulus | 7.114 | 6.003 | 3.360 |
| 3. | Bulk Density (Kg./Litre) | 1.74 | 1.57 | 1.76 |
| 4. | Specific Gravity | 2.84 | 2.79 | 2.74 |
| 5. | Water absorption | 0.35% | 0.46 % | 0.95% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./326/2021-22/ 	                                                  Dated: 24.11.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Executive Engineer

OBRA ‘C’ CIVIL, Construction Division-I

U.P.R. Vidyut Utpadan Nigam Ltd.

P.O. : Obra-231219,District : Sonebhadra

(M/s Sincere Construction)

Ref.		:    		359/OCCCD-I/IIT BHU   Dated : 04-10-20201

Tax Invoice No. : 	 000772 IIT BHU dated 20.10.2021

Details of Sample:	Cement: Ultra Tech  Cement,  PPC, IS : 1489,  Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-10 grade,

M-15 grade and M-25 grade

______________________________________________________________________________

Work: Work of 2nd raising of Chakari Ash dyke and allied civil work for 2x660 MW Obra ‘C’ Thermal Power Extension

Project.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 36 % 1.5 mm Expansion 149 Minutes 360 Minutes 25.2 N/mm2 36.1 N/mm2 45.1 N/mm2 1.29 2.79 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./312/2021-22/ 	                                                  Dated: 24.11.2021

#### TEST REPORTS

Tested for 	:      	Jt. GM/Civil

Project Head

M/s IRCON INTERNATIONAL LIMITED

Nabinagar Super Thermal Power Project,

N.P.G.C. Ltd. Near Shivpur Gate,

Via-Nabinagar Road, P.O. Ankhora Rly. Station,

Distt. Aurangabad (Bihar)-824303

(M/s G S Malhotra)

Ref.		:    	IRCON/2057/NSTP/FOB/Pkg-7/2145 dated 22.09.2021

Tax Invoice No. : 	000721 IIT BHU dated 06.10.2021

Details of Sample :      Concrete Mix Design.

Test Required       	 M-25 grade

___________________________________________________________________________

Sub.: Construction of Foot Over Bridge (FOB) across railway siding tracks at inside plant of NPGCL in connection with setting up of coal transportation system for Nabinagar Super Thermal Power Project (3x660 MW0 of NPGC Ltd.ner Ankoraha Railway Station, Anurangabad, Bihar, Package (7)- Regarding  Design Mix of M25 grade of concrete.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes   (i) 20 mm. (ii) 10 mm

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech  Cement,  PPC, IS : 1489, W-33, M-09, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./312/2021-22/ 	                                                  Dated: 24.11.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 1.87 : 1.05 : 1.95 0.64 : 1 : 1.37 : 0.87 : 1.46 | By weight By volume |

| Sl. No. | Test | M-25 grade |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (mm) | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 65.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 26.5 35.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 190.0 380.0 711.0 399.0 741.0 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./312/2021-22/ 	                                                  Dated: 24.11.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA—20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 89.5 | 100.0 | - |
|  | 10 mm | 0.20 | 88.30 | - |
|  | 4.75 mm | 0.05 | 5.10 | 2.8 |
|  | 2.36 mm | - | 0.90 | 5.8 |
|  | 1.18 mm | - | 0.05 | 15.8 |
|  | 600 micron | - | - | 45.2 |
|  | 300 micron | - | - | 96.0 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.102 | 6.057 | 2.646 |
| 3. | Bulk Density (Kg./Litre) | 1.71 | 1.54 | 1.75 |
| 4. | Specific Gravity | 2.81 | 2.81 | 2.75 |
| 5. | Water absorption | 0.30% | 0.20 % | 0.83% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./312/2021-22/ 	                                                  Dated: 24.11.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Jt. GM/Civil

Project Head

M/s IRCON INTERNATIONAL LIMITED

Nabinagar Super Thermal Power Project,

N.P.G.C. Ltd. Near Shivpur Gate,

Via-Nabinagar Road, P.O. Ankhora Rly. Station,

Distt. Aurangabad (Bihar)-824303

(M/s G S Malhotra)

Ref.		:    	IRCON/2057/NSTP/FOB/Pkg-7/2145 dated 22.09.2021

Details of Sample:	Cement: Ultra Tech  Cement,  PPC, IS : 1489, W-33, M-09, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade

______________________________________________________________________________

Sub.: Construction of Foot Over Bridge (FOB) across railway siding tracks at inside plant of NPGCL in connection with setting up of coal transportation system for Nabinagar Super Thermal Power Project (3x660 MW0 of NPGC Ltd.ner Ankoraha Railway Station, Anurangabad, Bihar, Package (7)- Regarding  Design Mix of M25 grade of concrete.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 36 % 1.5 mm Expansion 135 Minutes 350 Minutes 25.2 N/mm2 35.1 N/mm2 45.0 N/mm2 1.28 2.78 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./258/2021-22/                                                         Dated: 12.11.2021

#### TEST REPORTS

Tested for 	:      	Dy. General Manager (Engg.)

NBCC  (India) Ltd.,

Old Warden Office, Morvi Hostel,

IIT(BHU), Varanasi, (U.P.)

(M/s Chandralekha Construction Pvt. Ltd.)

Ref.		:    	 NBCC/IIT-BHU/Morvi Hostel/CCPL/2021/02     Dated: 15-07-2021

Tax Invoice No. : 	 000807 IIT BHU dated 29.10.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  M-25 grade with admixture

________________________________________________________________________

Sub :  Concrete Design Mix M-25 .

Project: Z-2150 Construction of Morvi Hostel-II (S+10) and Dining Block (2nd, 3rd & 4th floor) behind existing

Morvi Hostel at IIT BHU, Varanasi, U.P

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489, Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used Sika Plastiment  2001 NS.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./258/2021-22/                                                         Dated: 12.11.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.01 : 1.28 : 1.92 : 0.6* 0.65 : 1 : 1.46 : 1.05 : 1.46 : 0.6* | By weight By volume |

| Sl. No. | Test | M-25 Grade with admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (mm) | 125-175 |
| 3 | Slump Achieved in the Laboratory (mm) | 145.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 23.5 34.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Sika Plastiment 2001 NS (Litre) | 180.0 360.0 724.0 461.0 691.0 2.16 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./258/2021-22/                                                         Dated: 12.11.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade with

admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 79.05 | 100.0 | - |
|  | 10 mm | 0.20 | 88.4 | - |
|  | 4.75 mm | 0.15 | 2.95 | 2.8 |
|  | 2.36 mm | - | 0.20 | 8.4 |
|  | 1.18 mm | - | 0.05 | 23.4 |
|  | 600 micron | - | - | 49.2 |
|  | 300 micron | - | - | 88.0 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.206 | 6.084 | 2.708 |
| 3. | Bulk Density (Kg./Litre) | 1.71 | 1.59 | 1.79 |
| 4. | Specific Gravity | 2.82 | 2.79 | 2.64 |
| 5. | Water absorption | 0.25 % | 0.35 % | 1.65% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING  LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./258/2021-22/                                                         Dated: 12.11.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Dy. General Manager (Engg.)

NBCC  (India) Ltd.,

Old Warden Office, Morvi Hostel,

IIT(BHU), Varanasi, (U.P.)

(M/s Chandralekha Construction Pvt. Ltd.)

Ref.		:    	 NBCC/IIT-BHU/Morvi Hostel/CCPL/2021/02     Dated: 15-07-2021

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489 Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with

admixture

______________________________________________________________________________

Sub :  Concrete Design Mix M-25 .

Project: Z-2150 Construction of Morvi Hostel-II (S+10) and Dining Block (2nd, 3rd & 4th floor) behind existing

Morvi Hostel at IIT BHU, Varanasi, U.P

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 145 Minutes 375 Minutes 23.1 N/mm2 32.4 N/mm2 42.7 N/mm2 1.30 2.90 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./325/2021-22/                                                          Dated: 05.11.2021

#### TEST REPORTS

Tested for 	:      	Project Manager

M/s Vensa Infrastructure Limited

Medical College Project Gonda

Ref.		:    		VIL/QC/GOND/21-22/01  dated 10.09.2021

Tax Invoice No. : 	 000771 IIT BHU dated 20.10.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  (i)   M-25 grade with admixture with MP Birla Multicem Cement

(ii)  M-30 grade with admixture with MP Birla Multicem Cement

_____________________________________________________________________________

#### Sub :  Concrete Mix designs for Government Medical College Project  Gonda, U.P. for Grade M25 & M30 – Grades

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was MP Birla Multicem Cement, PPC, IS : 1489, Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used Conplast SP 430 G8 .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./325/2021-22/                                                          Dated: 05.11.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion (i)   M-25 grade with admixture, (ii)  M-30 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.28 : 1.27 : 1.55 : 0.7* 0.63 : 1 : 1.58 : 1.02 : 1.16 : 0.7* | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 1.97 : 1.14 : 1.39 : 0.7* 0.567 : 1 : 1.36 : 0.92 : 1.04 : 0.7* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture | M-30 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 |
| 3 | Slump range (after one hour ) (mm) | 100-150 | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 190.0 | 190.0 |
| 5 | Slump Achieved in the Laboratory(after one hour) (mm) | 140.0 | 135.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 23.5 34.5 | 31.5 42.0 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Conplast SP 430 G8 (Litre) | 180.0 360.0 821.0 457.0 558.0 2.52 | 180.0 400.0 788.0 456.0 556.0 2.80 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./325/2021-22/                                                          Dated: 05.11.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design (i)   M-25 grade with

admixture, (ii)  M-30 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 88.25 | 100.0 | - |
|  | 10 mm | 0.75 | 86.75 | - |
|  | 4.75 mm | 0.05 | 1.90 | 7.0 |
|  | 2.36 mm | - | 0.15 | 18.8 |
|  | 1.18 mm | - | 0.05 | 42.4 |
|  | 600 micron | - | - | 71.6 |
|  | 300 micron | - | - | 95.6 |
|  | 150 micron | - | - | 99.6 |
| 2. | Fineness Modulus | 7.110 | 6.112 | 3.350 |
| 3. | Bulk Density (Kg./Litre) | 1.69 | 1.57 | 1.82 |
| 4. | Specific Gravity | 2.68 | 2.61 | 2.73 |
| 5. | Water absorption | 0.10 % | 0.30 % | 0.91% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING  LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./325/2021-22/                                                          Dated: 05.11.2021

#### TEST REPORTS

Tested for 	:      	Project Manager

M/s Vensa Infrastructure Limited

Medical College Project Gonda

Ref.		:    		VIL/QC/GOND/21-22/01  dated 10.09.2021

Details of Sample:	Cement: MP Birla Multicem Cement, PPC, IS : 1489, Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i)   M-25 grade with

admixture, (ii)  M-30 grade with admixture

______________________________________________________________________________

#### Sub :  Concrete Mix designs for Government Medical College Project  Gonda, U.P. for Grade M25 & M30 – Grades

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 36 % 1.5 mm Expansion 140 Minutes 380 Minutes 23.9 N/mm2 34.5 N/mm2 43.6 N/mm2 1.26 2.86 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./271/2021-22/                                                        Dated: 05.11.2021

#### TEST REPORTS

Tested for 	:      	V S A INFRA PROJECTS PRIVATE LIMITED

WARD NO 5, floor-, , LOK MANYA TILAK NAGAR,

Chandauli, Uttar Pradesh, 232104

Ref.			:    	VSAIPPL-SMC JV/ IIT BHU/2021-22/01  dated 02.09.2021

Tax Invoice No. : 	 000629 IIT BHU dated 16.09.2021

Details of Sample :      	Concrete Mix Design.

Test Required       	 M-20 grade,  M-25 grade & M-30 grade

___________________________________________________________________________

Sub: Design Mix reports for M20, M25 & M30 Grade  for Chandauli Project site under the Jal jeevan Mission “Har Ghar Jal”  Scheme under SWSM Utter Pradesh, Lucknow

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes   (i) 20 mm. (ii) 10 mm

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech  Cement,  OPC 43-grade, IS : 269, W-37, M-08, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./271/2021-22/                                                        Dated: 05.11.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-20 grade,  M-25 grade & M-30 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M20 Grade | 0.55 : 1 : 2.27 : 1.20 : 2.22 0.77 : 1 : 1.79 : 1.07 : 1.79 | By weight By volume |
| 2. | M25 Grade | 0.50 : 1 : 1.98 : 1.08 : 2.01 0.70 : 1 : 1.56 : 0.96 : 1.62 | By weight By volume |
| 3. | M30 Grade | 0.45 : 1 : 1.71 : 0.98 : 1.82 0.63 : 1 : 1.35 : 0.87 : 1.46 | By weight By volume |

| Sl. No. | Test | M-20 grade | M-25 grade | M-30 grade |
| --- | --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 27.6 | 33.7 | 39.9 |
| 2 | Slump range (mm) | 50-100 | 50-100 | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 85.0 | 80.0 | 75.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 22.5 31.0 | 30.0 36.0 | 33.5 42.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 187.0 340.0 772.0 408.0 755.0 | 187.5 375.0 743.0 405.0 754.0 | 186.75 415.0 710.0 407.0 755.0 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./271/2021-22/                                                        Dated: 05.11.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-20 grade,                     M-25 grade & M-30 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA—20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 85.7 | 100.0 | - |
|  | 10 mm | 0.25 | 85.5 | - |
|  | 4.75 mm | 0.05 | 11.4 | 1.4 |
|  | 2.36 mm | - | 0.40 | 10.0 |
|  | 1.18 mm | - | 0.10 | 35.0 |
|  | 600 micron | - | - | 72.2 |
|  | 300 micron | - | - | 94.0 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.140 | 6.026 | 3.116 |
| 3. | Bulk Density (Kg./Litre) | 1.74 | 1.57 | 1.78 |
| 4. | Specific Gravity | 2.86 | 2.82 | 2.72 |
| 5. | Water absorption | 0.20% | 0.37 % | 0.66% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./271/2021-22/                                                        Dated: 05.11.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	V S A INFRA PROJECTS PRIVATE LIMITED

WARD NO 5, floor-, , LOK MANYA TILAK NAGAR,

Chandauli, Uttar Pradesh, 232104

Ref.			:    	VSAIPPL-SMC JV/ IIT BHU/2021-22/01  dated 02.09.2021

Details of Sample:	Cement: Ultra Tech  Cement,  OPC 43-grade, IS : 269, W-37, M-08,  Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-20 grade,                     M-25 grade & M-30 grade

______________________________________________________________________________

Sub: Design Mix reports for M20, M25 & M30 Grade  for Chandauli Project site under the Jal jeevan Mission “Har Ghar Jal”  Scheme under SWSM Utter Pradesh, Lucknow

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 30 % 1.0 mm Expansion 85 Minutes 270 Minutes 26.7 N/mm2 36.8 N/mm2 47.2 N/mm2 1.40 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./269/2021-22/                                                         Dated: 05.11.2021

#### TEST REPORTS

Tested for 	:      	M/s Welspun Enterprises Limited

2B-1470-3, Floor-1-2 Floor, Ahmed Bag,

Court Road, Saharanpur, U.P.- 247001

Ref.		:    		WKIJV/UPWS/AMETHI/2021-22/043   dated 16-08-2021

Tax Invoice No. : 	 000622 IIT BHU dated 14.09.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  M-25 grade with admixture and M-30 grade with admixture

_____________________________________________________________________________

#### Sub :  “Land Allocation for Survey, design Preparation of DPR, Construction, Commissioning and O & M for 10 years of Rural Water Supply Projects in the District of Amethi per Request for Proposal for Division-Ayodhya, under Jal Jeevan Mission” Regarding concrete Mix Design for M25 & M30 grade concrete

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 43-grade, IS:269, W-32, M-08, Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used BASF, Master Polyheed 8350.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./269/2021-22/                                                         Dated: 05.11.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion M-25 grade with admixture and M-30 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.13 : 1.48 : 1.80 : 0.8* 0.685 : 1 : 1.91 : 1.28 : 1.50 : 0.8* | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 1.82 : 1.32 : 1.61 : 0.8* 0.617 : 1 : 1.63 : 1.14 : 1.34 : 0.8* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture | M-30 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 |
| 2 | Slump range (initial) (mm) | 125-175 | 125-175 |
| 3 | Slump range (after two hour ) (mm) | 75-125 | 75-125 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 175.0 | 170.0 |
| 5 | Slump Achieved in the Laboratory(after two hour) (mm) | 115.0 | 110.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2 | 27.5 35.0 | 30.5 41.5 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) BASF, Master Polyheed 8350 (Litre) | 172.5 345.0 735.0 511.0 621.0 2.76 | 173.25 385.0 701.0 508.0 620.0 3.08 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./269/2021-22/                                                         Dated: 05.11.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade with

admixture and M-30 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 85.5 | 100.0 | - |
|  | 10 mm | 0.30 | 86.2 | - |
|  | 4.75 mm | 0.05 | 11.6 | 1.2 |
|  | 2.36 mm | - | 3.05 | 3.2 |
|  | 1.18 mm | - | 0.10 | 9.4 |
|  | 600 micron | - | - | 35.0 |
|  | 300 micron | - | - | 87.4 |
|  | 150 micron | - | - | 98.4 |
| 2. | Fineness Modulus | 7.142 | 5.991 | 2.346 |
| 3. | Bulk Density (Kg./Litre) | 1.65 | 1.59 | 1.53 |
| 4. | Specific Gravity | 2.65 | 2.69 | 2.60 |
| 5. | Water absorption | 0.30 % | 0.10 % | 1.76% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING  LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./269/2021-22/                                                         Dated: 05.11.2021

#### TEST REPORTS

Tested for 	:      	M/s Welspun Enterprises Limited

2B-1470-3, Floor-1-2 Floor, Ahmed Bag,

Court Road, Saharanpur, U.P.- 247001

Ref.		:    		WKIJV/UPWS/AMETHI/2021-22/043   dated 16-08-2021

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grade, IS:269, W-32, M-08, Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with

admixture and M-30 grade with admixture

______________________________________________________________________________

#### Sub :  “Land Allocation for Survey, design Preparation of DPR, Construction, Commissioning and O & M for 10 years of Rural Water Supply Projects in the District of Amethi per Request for Proposal for Division-Ayodhya, under Jal Jeevan Mission” Regarding concrete Mix Design for M25 & M30 grade concrete

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 32 % 1.0 mm Expansion 90 Minutes 270 Minutes 26.4 N/mm2 36.7 N/mm2 46.8 N/mm2 1.37 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./269/2021-22/                                                         Dated: 05.11.2021

#### TEST REPORTS

Tested for 	:      	Welspun-Kaveri Infraprojects JV Private Limited

401-402, floor-4th, Ratan Square- 20A,

Vidhan Sabha Marg, Lucknow, Uttar Pradesh, 226001

Ref.		:    		WKIJV/UPWS/AMBEDKAR NAGAR/2021-22/42  dated 09-08-2021

Tax Invoice No. : 	 000621 IIT BHU dated 14.09.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  M-25 grade with admixture and M-30 grade with admixture

________________________________________________________________________

#### Sub :  “Land Allocation for Survey, design Preparation of DPR, Construction, Commissioning and O & M for 10 years of Rural Water Supply Projects in the District of Ambedkar Nagar  per Request for Proposal for Division-Ayodhya, under Jal Jeevan Mission” Regarding concrete Mix Design for M25 & M30 grade concrete

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 43-grade, IS:269, W-32, M-08, Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used BASF, Master Polyheed 8350.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./268/2021-22/                                                         Dated: 05.11.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion M-25 grade with admixture and M-30 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.60 : 1.47 : 1.79 : 0.7* 0.685 : 1 : 2.04 : 1.27 : 1.49 : 0.7* | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 2.22 : 1.30 : 1.59 : 0.7* 0.617 : 1 : 1.74 : 1.12 : 1.32 : 0.7* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture | M-30 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 |
| 2 | Slump range (initial) (mm) | 125-175 | 125-175 |
| 3 | Slump range (after two hour ) (mm) | 75-125 | 75-125 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 175.0 | 170.0 |
| 5 | Slump Achieved in the Laboratory(after two hour) (mm) | 95.0 | 95.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 29.0 37.5 | 32.5 43.0 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) BASF, Master Polyheed 8350 (Litre) | 165.0 330.0 858.0 485.0 591.0 2.31 | 166.5 370.0 821.0 481.0 588.0 2.59 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./268/2021-22/                                                         Dated: 05.11.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade with

admixture and M-30 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 85.5 | 100.0 | - |
|  | 10 mm | 0.30 | 86.2 | - |
|  | 4.75 mm | 0.05 | 11.6 | 7.6 |
|  | 2.36 mm | - | 3.05 | 21.8 |
|  | 1.18 mm | - | 0.10 | 51.6 |
|  | 600 micron | - | - | 86.0 |
|  | 300 micron | - | - | 98.4 |
|  | 150 micron | - | - | 99.4 |
| 2. | Fineness Modulus | 7.142 | 5.991 | 3.648 |
| 3. | Bulk Density (Kg./Litre) | 1.65 | 1.59 | 1.75 |
| 4. | Specific Gravity | 2.65 | 2.69 | 2.71 |
| 5. | Water absorption | 0.30 % | 0.10 % | 0.90% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING  LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./268/2021-22/                                                           Dated: 05.11.2021

#### TEST REPORTS

Tested for 	:      	Welspun-Kaveri Infraprojects JV Private Limited

401-402, floor-4th, Ratan Square- 20A,

Vidhan Sabha Marg, Lucknow, Uttar Pradesh, 226001

Ref.		:    		WKIJV/UPWS/AMBEDKAR NAGAR/2021-22/42  dated 09-08-2021

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grade, IS:269, W-32, M-08, Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with

admixture and M-30 grade with admixture

______________________________________________________________________________

#### Sub :  “Land Allocation for Survey, design Preparation of DPR, Construction, Commissioning and O & M for 10 years of Rural Water Supply Projects in the District of Ambedkar Nagar  per Request for Proposal for Division-Ayodhya, under Jal Jeevan Mission” Regarding concrete Mix Design for M25 & M30 grade concrete

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 32 % 1.0 mm Expansion 90 Minutes 270 Minutes 26.4 N/mm2 36.7 N/mm2 46.8 N/mm2 1.37 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./293/2021-22/ 	                                                  Dated: 29.10.2021

#### TEST REPORTS

Tested for 	:      	Executive Engineer

Construction Division-II

U.P. Jal Nigam, Azamgarh

(M/s L.C. Infra Project Ltd. and T.C.L. (JV))

Ref.		:    		1229 /dk;Z -17@204 dated: 20-09-2021

Tax Invoice No. : 	 000690 IIT BHU dated 30.09.2021

Details of Sample :      Concrete Mix Design.

Test Required       	 M-25 grade and M-30 grade

___________________________________________________________________________

Sub.: Regarding M25 & M30 Mix design for construction work at District Azamgarh under Jal Jeevan Mission Scheme.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes   (i) 20 mm. (ii) 10 mm

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech  Cement,  OPC 43-grade, IS : 269, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./293/2021-22/ 	                                                  Dated: 29.10.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade & M-30 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 1.86 : 1.23 : 1.85 0.70 : 1 : 1.48 : 1.10 : 1.49 | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 1.62 : 1.11 : 1.67 0.63 : 1 : 1.29 : 1.00 : 1.34 | By weight By volume |

| Sl. No. | Test | M-25 grade | M-30 grade |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 |
| 2 | Slump range (mm) | 50-100 | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 80.0 | 75.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 29.0 35.5 | 35.0 43.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 190.0 380.0 707.0 468.0 703.0 | 189.0 420.0 680.0 466.0 702.0 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./293/2021-22/ 	                                                  Dated: 29.10.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade &              M-30 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA—20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.8 | 100.0 | - |
|  | 10 mm | 0.35 | 87.55 | - |
|  | 4.75 mm | 0.05 | 3.00 | 2.6 |
|  | 2.36 mm | - | 0.40 | 8.2 |
|  | 1.18 mm | - | 0.10 | 19.4 |
|  | 600 micron | - | - | 59.6 |
|  | 300 micron | - | - | 92.4 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.128 | 6.090 | 2.812 |
| 3. | Bulk Density (Kg./Litre) | 1.74 | 1.56 | 1.76 |
| 4. | Specific Gravity | 2.83 | 2.84 | 2.68 |
| 5. | Water absorption | 0.10% | 0.10 % | 1.15% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./293/2021-22/ 	                                                  Dated: 29.10.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Executive Engineer

Construction Division-II

U.P. Jal Nigam, Azamgarh

(M/s L.C. Infra Project Ltd. and T.C.L. (JV))

Ref.		:    		1229 /dk;Z -17@204 dated: 20-09-2021

Details of Sample:	Cement: Ultra Tech  Cement,  OPC 43-grade, IS : 269, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade &                  M-30 grade

______________________________________________________________________________

Sub.: Regarding M25 & M30 Mix design for construction work at District Azamgarh under Jal Jeevan Mission Scheme.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 31 % 1.0 mm Expansion 95 Minutes 280 Minutes 26.8 N/mm2 36.8 N/mm2 47.1 N/mm2 1.40 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./265/2021-22/                                                       Dated: 25.10.2021

#### TEST REPORTS

Tested for 	:      	Project Manager

M/s L & T  Constructions

RWS Empanelment Tubewell Scheme

L-34 Ashok Vihar Colony Phase-1,Varanasi, U.P.-221007

Ref.		:    		Ref. No. L & T-WET/RWS/VNS-TWS/061  Dated :06-09-2021

Tax Invoice No. : 	 000611 IIT BHU dated 14.09.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  (i)     M-25 grade with admixture

(ii)   M-30 grade with admixture

(iii)   M-40 grade with admixture for 450 Kg. Cement

(iv)   M-40 grade with admixture for 425 Kg. Cement

________________________________________________________________________

Sub :  Concrete Mix design- Grades M25, M30 & M40

Projects: VARANASI TUBE WELL SCHEME-Package 1.

.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 43-grde, IS : 269, W-33, M-08, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Auramix 200 & Auracast 100.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./265/2021-22/                                                       Dated: 25.10.2021

#### (Page – 2(i))

#### TEST REPORTS

Recommended Mix Proportion (i)   M-25 grade with admixture, (ii)  M-30 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.28 : 1.49 : 1.83 : 0.5* 0.695 : 1 : 1.75 : 1.31 : 1.48 : 0.5* | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 1.95 : 1.34 : 1.63 : 0.5* 0.626 : 1 : 1.50 : 1.18 : 1.32 : 0.5* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture | M-30 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 |
| 3 | Slump range (after one hour ) (mm) | 100-150 | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 170.0 | 170.0 |
| 5 | Slump Achieved in the Laboratory(after one hour) (mm) | 125.0 | 120.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 25.0 36.0 | 31.0 41.5 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Auramix 200 (Litre) | 175.0 350.0 798.0 522.0 641.0 1.75 | 175.5 390.0 761.0 523.0 636.0 1.95 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./265/2021-22/                                                       Dated: 25.10.2021

#### (Page – 2(ii))

#### TEST REPORTS

Recommended Mix Proportion (i)  M-40 grade with admixture for 450 Kg. Cement  (ii)   M-40 grade with

admixture for 425 Kg. Cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-40 Grade for 450 Kg Cement | 0.38 : 1 : 1.66 : 1.14 : 1.38 : 0.4* 0.528 : 1 : 1.27 : 1.00 : 1.12 : 0.4* | By weight By volume |
| 2. | M-40 Grade for 425 Kg Cement | 0.38 : 1 : 2.02 : 1.13 : 1.38 : 0.75* 0.528 : 1 : 1.55 : 0.99 : 1.12 : 0.75* | By weight By volume |

| Sl. No. | Test | M-40 grade with admixture for 450 Kg. Cement | M-40 grade with admixture for 425 Kg. Cement |
| --- | --- | --- | --- |
| 1. | Target Strength (N/mm2) | 50.9 | 50.9 |
| 2. | Slump range (initial) (mm) | 150-200 | 150-200 |
| 3. | Slump range (after one hour ) (mm) | 100-150 | 100-150 |
| 4. | Slump Achieved in the Laboratory (initial) (mm) | 165.0 | 190.0 |
| 5. | Slump Achieved in the Laboratory(after one hour) (mm) | 115.0 | 135.0 |
| 6. | i) After 24 hours Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) iii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 25.0 41.0 51.5 | 28.5 46.5 54.5 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Auracast 100 (Litre) | 171.0 450.0 747.0 513.0 621.0 1.80 | 161.5 425.0 859.0 480.0 586.0 3.19 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./265/2021-22/                                                       Dated: 25.10.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design for (i)  M-25 grade with admixture,  (ii)   M-30 grade with admixture,  (iii)   M-40 grade with admixture for 450 Kg. Cement  (iv)   M-40 grade with admixture for 425 Kg. Cement

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm (Source: Hanumana, M.P.) | CA—10 mm (Source: Hanumana, M.P.) | FA (sand) (Source: Dehri-Bihar) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 90.55 | 100.0 | - |
|  | 10 mm | 0.45 | 89.4 | - |
|  | 4.75 mm | 0.05 | 6.25 | 2.6 |
|  | 2.36 mm | - | 0.30 | 8.4 |
|  | 1.18 mm | - | 0.05 | 30.2 |
|  | 600 micron | - | - | 66.4 |
|  | 300 micron | - | - | 94.0 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.090 | 6.040 | 3.006 |
| 3. | Bulk Density (Kg./Litre) | 1.72 | 1.58 | 1.81 |
| 4. | Specific Gravity | 2.85 | 2.86 | 2.71 |
| 5. | Water absorption | 0.15 % | 0.46 % | 0.82% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./265/2021-22/                                                       Dated: 25.10.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Project Manager

M/s L & T  Constructions

RWS Empanelment Tubewell Scheme

L-34 Ashok Vihar Colony Phase-1,Varanasi, U.P.-221007

Ref.		:    		Ref. No. L & T-WET/RWS/VNS-TWS/061  Dated :06-09-2021

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grde, IS : 269, W-33, M-08, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i)  M-25 grade with admixture,  (ii)  M-30 grade with admixture,  (iii)   M-40 grade with admixture for 450 Kg. Cement   (iv)   M-40 grade with admixture for 425 Kg. Cement

______________________________________________________________________________

Sub :  Concrete Mix design- Grades M25, M30 & M40

Projects: VARANASI TUBE WELL SCHEME-Package 1.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 30 % 1.0 mm Expansion 80 Minutes 270 Minutes 27.2 N/mm2 37.2 N/mm2 47.1 N/mm2 1.39 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./266/2021-22/                                                      Dated: 25.10.2021

#### TEST REPORTS

Tested for 	:      	M/s L & T  Construction

RWS Empanelment Tubewell Scheme

L-34 Ashok Vihar Colony Phase-1,Varanasi, U.P.-221007

Ref.		:    		 L & T-WET/RWS/VNS-TWS/062  Dated :02-09-2021

L & T-WET/RWS/VNS-TWS/060  Dated :02-09-2021

Tax Invoice No. : 	 000612 IIT BHU dated 14.09.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  (i)   M-40 grade with admixture for 425 Kg. Cement

________________________________________________________________________

Sub :  Concrete Mix design- Grade M40

Projects: VARANASI TUBE WELL SCHEME-Package 1.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 43-grde, IS : 269, W-29, M-07 Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Auracast 100.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./266/2021-22/                                                      Dated: 25.10.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion  M-40 grade with admixture for 425 Kg. cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-40 Grade | 0.38 : 1 : 2.07 : 1.09 : 1.32 : 0.75* 0.532 : 1 : 1.58 : 0.95 : 1.07 : 0.75* | By weight By volume |

| Sl. No. | Test | M-40 grade with admixture for 425 Kg. cement |
| --- | --- | --- |
| 1. | Target Strength (N/mm2) | 50.9 |
| 2. | Slump range (initial) (mm) | 150-200 |
| 3. | Slump range (after one hour ) (mm) | 100-150 |
| 4. | Slump Achieved in the Laboratory (initial) (mm) | 185.0 |
| 5. | Slump Achieved in the Laboratory(after one hour) (mm) | 140.0 |
| 6. | i) After 24 hours Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) iii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 30.0 47.0 53.0 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Auracast 100 (Litre) | 161.5 425.0 880.0 463.0 561.0 3.19 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./266/2021-22/                                                      Dated: 25.10.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design M-40 grade with admixture for

425 Kg. cement

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 85.75 | 100.0 | - |
|  | 10 mm | 0.45 | 86.05 | - |
|  | 4.75 mm | 0.05 | 14.85 | 5.2 |
|  | 2.36 mm | - | 0.40 | 18.8 |
|  | 1.18 mm | - | 0.05 | 49.6 |
|  | 600 micron | - | - | 82.8 |
|  | 300 micron | - | - | 97.6 |
|  | 150 micron | - | - | 99.2 |
| 2. | Fineness Modulus | 7.138 | 5.987 | 3.532 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.61 | 1.83 |
| 4. | Specific Gravity | 2.81 | 2.79 | 2.70 |
| 5. | Water absorption | 0.25 % | 0.35 % | 0.78% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./266/2021-22/                                                      Dated: 25.10.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	M/s L & T  Construction

RWS Empanelment Tubewell Scheme

L-34 Ashok Vihar Colony Phase-1,Varanasi, U.P.-221007

Ref.		:    		 L & T-WET/RWS/VNS-TWS/062  Dated :02-09-2021

L & T-WET/RWS/VNS-TWS/060  Dated :02-09-2021

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grde, IS : 269, W-29, M-07 Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-40 grade with admixture for 425 Kg. cement

______________________________________________________________________________

Sub :  Concrete Mix design- Grade M40

Projects: VARANASI TUBE WELL SCHEME-Package 1.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 30 % 1.0 mm Expansion 85 Minutes 275 Minutes 26.8 N/mm2 36.8 N/mm2 46.7 N/mm2 1.40 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./269/2021-22/                                                         Dated: 25.10.2021

#### TEST REPORTS

Tested for 	:      	M/s Welspun Enterprises Limited

2B-1470-3, Floor-1-2 Floor, Ahmed Bag,

Court Road, Saharanpur, U.P.- 247001

Ref.		:    		WKIJV/UPWS/AMETHI/2021-22/043 dated 16-08-2021

Tax Invoice No. : 	 000622 IIT BHU dated 14.09.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  M-25 grade with admixture and M-30 grade with admixture

________________________________________________________________________

#### Sub :  “Land Allocation for Survey, design Preparation of DPR, Construction, Commissioning and O & M for 10 years of Rural Water Supply Projects in the District of Amethi per Request for Proposal for Division-Ayodhya, under Jal Jeevan Mission” Regarding concrete Mix Design for M25 & M30 grade concrete

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 43-grade, IS:269, W-32, M-08, Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used BASF, Master Polyheed 8350.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./270/2021-22/                                                         Dated: 25.10.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion M-25 grade with admixture and M-30 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 1.99 : 1.30 : 2.42 : 0.8* 0.685 : 1 : 1.75 : 1.14 : 1.95 : 0.8* | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 1.69 : 1.16 : 2.15 : 0.8* 0.617 : 1 : 1.48 : 1.02 : 1.73 : 0.8* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture | M-30 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 |
| 3 | Slump range (after two hour ) (mm) | 75-125 | 75-125 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 195.0 | 200.0 |
| 5 | Slump Achieved in the Laboratory(after two hour) (mm) | 105.0 | 110.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 26.5 35.5 | 30.0 41.0 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) BASF, Master Polyheed 8350 (Litre) | 165.0 330.0 657.0 430.0 799.0 2.64 | 166.5 370.0 625.0 429.0 796.0 2.96 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./270/2021-22/                                                         Dated: 25.10.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade with

admixture and M-30 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 73.55 | 100.0 | - |
|  | 10 mm | 0.40 | 87.9 | - |
|  | 4.75 mm | 0.05 | 3.85 | 1.8 |
|  | 2.36 mm | - | 0.55 | 4.4 |
|  | 1.18 mm | - | 0.10 | 11.0 |
|  | 600 micron | - | - | 38.4 |
|  | 300 micron | - | - | 89.6 |
|  | 150 micron | - | - | 98.2 |
| 2. | Fineness Modulus | 7.260 | 6.077 | 2.434 |
| 3. | Bulk Density (Kg./Litre) | 1.70 | 1.56 | 1.56 |
| 4. | Specific Gravity | 2.68 | 2.58 | 2.61 |
| 5. | Water absorption | 1.09 % | 2.08 % | 1.89% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING  LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./270/2021-22/                                                         Dated: 25.10.2021

#### TEST REPORTS

Tested for 	:      	DGM –Project

M/s Welspun Enterprises Limited

2B-1470-3, Floor-1-2 Floor, Ahmed Bag,

Court Road, Saharanpur, U.P.- 247001

Ref.		:    		WKIJV/UPWS/Bhadohi/2021-22/20 dated 02-08-2021

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grade, IS:269, W-32, M-08, Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with

admixture and M-30 grade with admixture

______________________________________________________________________________

#### Sub :  Jal Jeevan Mission- “ Empanelment for Survey, design, Preparation of DPR, Construction, Commissioning and O & M for 10eyrs of various Rural Water support projects in the state of U.P. as per for Division-Varanasi, District Sant Ravidas Nagar”-concrete Mix Design for M25 & M30 grade concrete

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 32 % 1.0 mm Expansion 90 Minutes 270 Minutes 26.4 N/mm2 36.7 N/mm2 46.8 N/mm2 1.37 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./270/2021-22/                                                         Dated: 25.10.2021

#### TEST REPORTS

Tested for 	:      	DGM –Project

M/s Welspun Enterprises Limited

2B-1470-3, Floor-1-2 Floor, Ahmed Bag,

Court Road, Saharanpur, U.P.- 247001

Ref.		:    		WKIJV/UPWS/Bhadohi/2021-22/20 dated 02-08-2021

Tax Invoice No. : 	 000623 IIT BHU dated 14.09.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  M-25 grade with admixture and M-30 grade with admixture

________________________________________________________________________

#### Sub :  Jal Jeevan Mission- “ Empanelment for Survey, design, Preparation of DPR, Construction, Commissioning and O & M for 10eyrs of various Rural Water support projects in the state of U.P. as per for Division-Varanasi, District Sant Ravidas Nagar”-concrete Mix Design for M25 & M30 grade concrete

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 43-grade, IS:269, W-32, M-08, Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used BASF, Master Polyheed 8350.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./270/2021-22/                                                         Dated: 25.10.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion M-25 grade with admixture and M-30 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 1.99 : 1.30 : 2.42 : 0.8* 0.685 : 1 : 1.75 : 1.14 : 1.95 : 0.8* | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 1.69 : 1.16 : 2.15 : 0.8* 0.617 : 1 : 1.48 : 1.02 : 1.73 : 0.8* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture | M-30 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 |
| 3 | Slump range (after two hour ) (mm) | 75-125 | 75-125 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 195.0 | 200.0 |
| 5 | Slump Achieved in the Laboratory(after two hour) (mm) | 105.0 | 110.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 26.5 35.5 | 30.0 41.0 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) BASF, Master Polyheed 8350 (Litre) | 165.0 330.0 657.0 430.0 799.0 2.64 | 166.5 370.0 625.0 429.0 796.0 2.96 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./270/2021-22/                                                         Dated: 25.10.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade with

admixture and M-30 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 73.55 | 100.0 | - |
|  | 10 mm | 0.40 | 87.9 | - |
|  | 4.75 mm | 0.05 | 3.85 | 1.8 |
|  | 2.36 mm | - | 0.55 | 4.4 |
|  | 1.18 mm | - | 0.10 | 11.0 |
|  | 600 micron | - | - | 38.4 |
|  | 300 micron | - | - | 89.6 |
|  | 150 micron | - | - | 98.2 |
| 2. | Fineness Modulus | 7.260 | 6.077 | 2.434 |
| 3. | Bulk Density (Kg./Litre) | 1.70 | 1.56 | 1.56 |
| 4. | Specific Gravity | 2.68 | 2.58 | 2.61 |
| 5. | Water absorption | 1.09 % | 2.08 % | 1.89% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING  LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./270/2021-22/                                                         Dated: 25.10.2021

#### TEST REPORTS

Tested for 	:      	DGM –Project

M/s Welspun Enterprises Limited

2B-1470-3, Floor-1-2 Floor, Ahmed Bag,

Court Road, Saharanpur, U.P.- 247001

Ref.		:    		WKIJV/UPWS/Bhadohi/2021-22/20 dated 02-08-2021

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grade, IS:269, W-32, M-08, Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with

admixture and M-30 grade with admixture

______________________________________________________________________________

#### Sub :  Jal Jeevan Mission- “ Empanelment for Survey, design, Preparation of DPR, Construction, Commissioning and O & M for 10eyrs of various Rural Water support projects in the state of U.P. as per for Division-Varanasi, District Sant Ravidas Nagar”-concrete Mix Design for M25 & M30 grade concrete

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 32 % 1.0 mm Expansion 90 Minutes 270 Minutes 26.4 N/mm2 36.7 N/mm2 46.8 N/mm2 1.37 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./272/2021-22/                                                        Dated: 25.10.2021

#### TEST REPORTS

Tested for 	:      	Sr. Mgr. (FES & FQA)

Bhartiya Rail Bijlee Company Ltd.

NTPC, Nabinagar

(M/s  A  K  Verma)

Ref.			:    	BRBCL/FQA/2021/A.K. VERMA-Design Mix   Dated : 19-08-2021

Tax Invoice No. : 	 000630 IIT BHU dated 16.09.2021

Details of Sample :      	Concrete Mix Design.

Test Required       	 M-25 grade

___________________________________________________________________________

Work: Development of facilities at Dry ash Evacuation system and Maintenance Yard at BRBCL Nabinagar.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes   (i) 20 mm. (ii) 10 mm

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Double Bull Cement,  PPC IS : 1489, W-27, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./272/2021-22/                                                        Dated: 25.10.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M25 Grade | 0.50 : 1 : 1.80 : 1.05 : 1.96 0.655 : 1 : 1.35 : 0.89 : 1.54 | By weight By volume |

| Sl. No. | Test | M-25 grade |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (mm) | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 70.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 23.0 34.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 190.0 380.0 684.0 399.0 745.0 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./272/2021-22/                                                        Dated: 25.10.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA—20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 94.75 | 100.0 | - |
|  | 10 mm | 1.10 | 87.2 | - |
|  | 4.75 mm | 0.05 | 3.10 | 1.6 |
|  | 2.36 mm | - | 0.50 | 5.8 |
|  | 1.18 mm | - | 0.25 | 17.8 |
|  | 600 micron | - | 0.10 | 49.6 |
|  | 300 micron | - | - | 88.6 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.041 | 6.089 | 2.624 |
| 3. | Bulk Density (Kg./Litre) | 1.67 | 1.55 | 1.75 |
| 4. | Specific Gravity | 2.75 | 2.76 | 2.68 |
| 5. | Water absorption | 0.35% | 0.40 % | 0.69% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./272/2021-22/                                                        Dated: 25.10.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Sr. Mgr. (FES & FQA)

Bhartiya Rail Bijlee Company Ltd.

NTPC, Nabinagar

(M/s  A  K  Verma)

Ref.			:    	BRBCL/FQA/2021/A.K. VERMA-Design Mix   Dated : 19-08-2021

Details of Sample:	Cement: Double Bull Cement,  PPC IS : 1489, W-27, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade

______________________________________________________________________________

Work: Development of facilities at Dry ash Evacuation system and Maintenance Yard at BRBCL Nabinagar.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 35 % 1.5 mm Expansion 160 Minutes 265 Minutes 24.7 N/mm2 34.7 N/mm2 43.7 N/mm2 1.31 2.91 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./296(i)/2021-22/                                                      Dated: 25.10.2021

#### TEST REPORTS

Tested for 	:      	Director

M/s Abhay Fire Engineering & Services Pvt. Ltd.,

D-206-A, 2nd & 3rd Floor, Blossom Tower, Vibhuti Khand,

Gomti Nagar, Lucknow – 226010

Ref.		:    		AFSPL/VARANASI ZONE-UPRNN/2020-21/01  Dated:20-08-2021

Tax Invoice No. : 	 000697 IIT BHU dated 05.10.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  M-25 grade with admixture

________________________________________________________________________

Sub :  Construction of Drug ware House at Ballia (U.P.)

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Jaypee Cement, PPC, IS:1489, Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used Conplast SP 430 G8.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./296(i)/2021-22/                                                      Dated: 25.10.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-30 Grade | 0.50 : 1 : 2.15 : 1.24 : 1.86 : 0.6* 0.66 : 1 : 1.61 : 1.06 : 1.42 : 0.6* | By weight By volume |

| l. No. | Test | M-25 Grade with admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (mm) | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 135.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 23.5 34.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Conplast SP 430 G8 (Litre) | 180.0 360.0 774.0 446.0 670.0 2.16 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./296(i)/2021-22/                                                      Dated: 25.10.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade with

admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 85.85 | 100.0 | - |
|  | 10 mm | 0.35 | 87.4 | - |
|  | 4.75 mm | 0.05 | 2.75 | 2.8 |
|  | 2.36 mm | - | 0.40 | 15.0 |
|  | 1.18 mm | - | 0.10 | 49.2 |
|  | 600 micron | - | - | 85.6 |
|  | 300 micron | - | - | 98.2 |
|  | 150 micron | - | - | 99.4 |
| 2. | Fineness Modulus | 7.138 | 6.094 | 3.502 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.55 | 1.76 |
| 4. | Specific Gravity | 2.81 | 2.79 | 2.68 |
| 5. | Water absorption | 0.20 % | 0.20 % | 1.15% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING  LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./296(i)/2021-22/                                                      Dated: 25.10.2021

#### TEST REPORTS

Tested for 	:      	Director

M/s Abhay Fire Engineering & Services Pvt. Ltd.,

D-206-A, 2nd & 3rd Floor, Blossom Tower, Vibhuti Khand,

Gomti Nagar, Lucknow – 226010

Ref.		:    		AFSPL/VARANASI ZONE-UPRNN/2020-21/01  Dated:20-08-2021

Details of Sample:	Cement: Jaypee Cement, PPC, IS:1489, Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with admixture

______________________________________________________________________________

Sub :  Construction of Drug ware House at Ballia (U.P.)

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 36 % 1.5 mm Expansion 125 Minutes 375 Minutes 24.4 N/mm2 34.4 N/mm2 43.6 N/mm2 1.32 2.92 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./264/2021-22/                                                      Dated: 07.10.2021

#### TEST REPORTS

Tested for 	:      	M/s Shree Bala Ji Infratech

D-Plant, ATPP, Anpara,

Distt Sonebhadra-231225

Ref.		:    		 Nil     Dated : 03-09-2021

Tax Invoice No. : 	000606 IIT BHU  dated 10.09.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  M-30 grade with admixture

________________________________________________________________________

Sub :  Concrete Mix Design of M30 grade with PPC

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS:1489, W-35, M-09,Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used Fosroc Auramix 400.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./264/2021-22/                                                        Dated: 07.10.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-30 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-30 Grade | 0.45 : 1 : 1.99 : 1.26 : 1.54 : 0.7* 0.590 : 1 : 1.49 : 1.04 : 1.20 : 0.7* | By weight By volume |

| l. No. | Test | M-30 Grade with admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 39.9 |
| 2 | Slump range (mm) | 125-175 |
| 3 | Slump Achieved in the Laboratory (mm) | 170.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 31.0 43.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Fosroc Auramix 400 (Litre) | 175.5 390.0 776.0 491.0 601.0 2.73 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./264/2021-22/                                                      Dated: 07.10.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-30 grade with

admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 96.25 | 100.0 | - |
|  | 10 mm | 5.75 | 86.75 | - |
|  | 4.75 mm | 0.10 | 6.35 | 4.6 |
|  | 2.36 mm | - | 0.70 | 12.4 |
|  | 1.18 mm | - | 0.10 | 30.4 |
|  | 600 micron | - | - | 67.8 |
|  | 300 micron | - | - | 97.4 |
|  | 150 micron | - | - | 99.2 |
| 2. | Fineness Modulus | 6.979 | 6.061 | 3.118 |
| 3. | Bulk Density (Kg./Litre) | 1.68 | 1.59 | 1.75 |
| 4. | Specific Gravity | 2.81 | 2.83 | 2.65 |
| 5. | Water absorption | 0.35 % | 0.25 % | 1.65% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING  LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./264/2021-22/                                                      Dated: 07.10.2021

#### TEST REPORTS

Tested for 	:      	M/s Shree Bala Ji Infratech

D-Plant, ATPP, Anpara,

Distt Sonebhadra-231225

Ref.		:    		 Nil     Dated : 03-09-2021

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS:1489, W-35, M-09,Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-30 grade with admixture

______________________________________________________________________________

Sub :  Concrete Mix Design of M30 grade with PPC

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 125 Minutes 335 Minutes 24.8 N/mm2 36.1 N/mm2 44.8 N/mm2 1.31 2.91 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./202/2021-22/                                                         Dated: 05.10.2021

#### TEST REPORTS

Tested for 	:      	M/s Minocha Construction Company

4 First Floor, Ikon Tower, Hari Nagar,

Shamshabad Road, Agra (U.P.)-282001

Ref.		:    	MCC/HO/BHU/VNS/ASR/MSSR/21/72     Dated: 29.07.2021

Tax Invoice No. : 	000486 IIT BHU  dated 17.08.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  M-25 grade with admixture

________________________________________________________________________

Sub :  Construction of ASR/MDDR Building at L.B.S.I. Airport Varanasi

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was ACC SURAKSHA POWER+ Cement, PPC, IS:1489, W-26,M-06,Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used ) Sika Plastiment  2001 NS.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./202/2021-22/                                                         Dated: 05.10.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.06 : 1.36 : 1.91 : 0.6* 0.66 : 1 : 1.55 : 1.14 : 1.44 : 0.6* | By weight By volume |

| Sl. No. | Test | M-25 Grade with admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (mm) | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 135.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 22.0 34.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Sika Plastiment 2001 NS (Litre) | 180.0 360.0 742.0 490.0 688.0 2.16 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./202/2021-22/                                                         Dated: 05.10.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade with

admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 88.25 | 100.0 | - |
|  | 10 mm | 0.45 | 86.4 | - |
|  | 4.75 mm | 0.05 | 5.55 | 1.6 |
|  | 2.36 mm | - | 0.55 | 5.8 |
|  | 1.18 mm | - | 0.05 | 15.8 |
|  | 600 micron | - | - | 51.6 |
|  | 300 micron | - | - | 92.6 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.133 | 6.075 | 2.664 |
| 3. | Bulk Density (Kg./Litre) | 1.75 | 1.57 | 1.75 |
| 4. | Specific Gravity | 2.82 | 2.84 | 2.64 |
| 5. | Water absorption | 0.20 % | 0.30 % | 1.64% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING  LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./202/2021-22/                                                         Dated: 05.10.2021

#### TEST REPORTS

Tested for 	:      	M/s Minocha Construction Company

4 First Floor, Ikon Tower, Hari Nagar,

Shamshabad Road, Agra (U.P.)-282001

Ref.		:    	MCC/HO/BHU/VNS/ASR/MSSR/21/72     Dated: 29.07.2021

Details of Sample:	Cement: ACC SURAKSHA POWER+ Cement, PPC, IS:1489, W-26,M-06,Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with admixture

______________________________________________________________________________

Sub :  Construction of ASR/MDDR Building at L.B.S.I. Airport Varanasi

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 8 % 39 % 1.5 mm Expansion 140 Minutes 430 Minutes 23.9 N/mm2 34.0 N/mm2 43.6 N/mm2 1.32 2.92 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./137/2021-22/                                                            Dated: 21.09.2021

#### TEST REPORTS

Tested for 	:      	Executive Engineer/Con

E.C. Railway, Jhanjharpur

Office of the Dy. Chief Engineer/Con

Lichi Bagan,Muzaffarpur-1

(M/s Lord Vishnu Constructions Private Limited)

Ref.			:    	XEN/Con/JJP/SKI-NMA/Misc. Dated 03.07.2021

Tax Invoice No. : 	 000325 IIT BHU dated 28.07.2021

Details of Sample :      	Concrete Mix Design.

Test Required       	(i)  M-20 grade and M-25 grade

(ii) M-30 grade and M-35 grade

____________________________________________________________________________

Sub.: Design mix of M-20, M-25, M-30, & M-35 for the work “Construction of platform and FOB at Manigarchi, FOB at Lohna Road and Tamuria, RCC overhead tank with tube well at Jhansharpur,RPF Post at Jhanjharpur, blanketing for loop lines between Manigarchi- Nirmali and other miscellaneous works in connection with gauge conversion of Sakri-Nirmali section of Samastiur Division.”

.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489,  Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./137/2021-22/                                                            Dated: 21.09.2021

#### (Page – 2(i))

#### TEST REPORTS

Recommended Mix Proportion for (i) M-20 grade and M-25 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M20 Grade | 0.55 : 1 : 2.08 : 1.17 : 2.18 0.721 : 1 : 1.56 : 0.99 : 1.65 | By weight By volume |
| 2. | M25 Grade | 0.50 : 1 : 1.81 : 1.07 : 1.98 0.655 : 1 : 1.36 : 0.90 : 1.50 | By weight By volume |

| Sl. No. | Test | M-20 grade | M-25 grade |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 27.6 | 33.7 |
| 2 | Slump range (mm) | 50-100 | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 75.0 | 65.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 24.0 31.5 | 29.0 36.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 190.3 346.0 720.0 405.0 754.0 | 190.0 380.0 688.0 407.0 752.0 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./137/2021-22/                                                            Dated: 21.09.2021

#### (Page – 2(ii))

#### TEST REPORTS

Recommended Mix Proportion for (ii) M-30 grade and M-35 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M30 Grade | 0.45 : 1 : 1.54 : 0.95 : 1.75 0.590 : 1 : 1.15 : 0.80 : 1.33 | By weight By volume |
| 2. | M35 Grade | 0.40 : 1 : 1.31 : 0.84 : 1.56 0.524 : 1 : 0.98 : 0.71 : 1.18 | By weight By volume |

| Sl. No. | Test | M-30 grade | M-35 grade |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 39.9 | 45.4 |
| 2 | Slump range (mm) | 50-100 | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 60.0 | 60.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 32.0 43.0 | 34.5 48.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 191.25 425.0 655.0 404.0 744.0 | 190.0 475.0 622.0 399.0 741.0 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./137/2021-22/                                                            Dated: 21.09.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design for (i)  M-20 grade and

M-25 grade (ii) M-30 grade and M-35 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.1 | 100.0 | - |
|  | 10 mm | 0.25 | 86.8 | - |
|  | 4.75 mm | 0.05 | 2.10 | 2.8 |
|  | 2.36 mm | - | 0.35 | 7.8 |
|  | 1.18 mm | - | 0.10 | 23.2 |
|  | 600 micron | - | - | 57.2 |
|  | 300 micron | - | - | 88.2 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.136 | 6.107 | 2.786 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.55 | 1.75 |
| 4. | Specific Gravity | 2.83 | 2.84 | 2.64 |
| 5. | Water absorption | 0.15 % | 0.35 % | 1.63% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./137/2021-22/                                                            Dated: 21.09.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Executive Engineer/Con

E.C. Railway, Jhanjharpur

Office of the Dy. Chief Engineer/Con

Lichi Bagan,Muzaffarpur-1

(M/s Lord Vishnu Constructions Private Limited)

Ref.			:    	XEN/Con/JJP/SKI-NMA/Misc. Dated 03.07.2021

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489,  Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i)  M-20 grade and

M-25 grade (ii) M-30 grade and M-35 grade

______________________________________________________________________________

Sub.: Design mix of M-20, M-25, M-30, & M-35 for the work “Construction of platform and FOB at Manigarchi, FOB at Lohna Road and Tamuria, RCC overhead tank with tube well at Jhansharpur,RPF Post at Jhanjharpur, blanketing for loop lines between Manigarchi- Nirmali and other miscellaneous works in connection with gauge conversion of Sakri-Nirmali section of Samastiur Division.”

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.0 mm Expansion 130 Minutes 360 Minutes 24.8 N/mm2 34.4 N/mm2 44.5 N/mm2 1.31 2.91 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./200/2021-22/                                                        Dated: 21.09.2021

#### TEST REPORTS

Tested for 	:      	Project Manager

M/s L & T  Constructions

RWS Empanelment Tubewell Scheme

L-34 Ashok Vihar Colony Phase-1,Varanasi, U.P.-221007

(M/s OM SHARAN INFRASTRUCTURE)

Ref.		:    		Ref. No. L & T-WET/RWS/VNS-TWS/049  Dated :10-08-2021

L & T-WET/RWS/VNS-TWS/048  Dated :06-08-2021

L & T-WET/RWS/VNS-TWS/039  Dated :12-07-2021

Tax Invoice No. : 	 000484 IIT BHU dated 17.08.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  (i)   M-25 grade with admixture

(ii)  M-30 grade with admixture

(iv) M-40 grade with admixture

________________________________________________________________________

Sub :  Concrete Mix design- Grades M25, M30. & M40

Projects: RWS-Empanelment, Varanasi Tube Well Scheme.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 43-grde, IS : 269, W-29, M-07 Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Auramix 200 & Auracast 100.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./200/2021-22/                                                        Dated: 21.09.2021

#### (Page – 2(i))

#### TEST REPORTS

Recommended Mix Proportion (i)   M-25 grade with admixture, (ii)  M-30 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.33 : 1.44 : 1.76 : 0.5* 0.70 : 1 : 1.78 : 1.25 : 1.42 : 0.5* | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 2.00 : 1.29 : 1.58 : 0.5* 0.63 : 1 : 1.53 : 1.12 : 1.28 : 0.5* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture | M-30 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 |
| 3 | Slump range (after one hour ) (mm) | 100-150 | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 175.0 | 170.0 |
| 5 | Slump Achieved in the Laboratory(after one hour) (mm) | 145.0 | 140.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 23.5 34.5 | 28.0 41.0 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Auramix 200 (Litre) | 175.0 350.0 816.0 504.0 616.0 1.75 | 175.5 390.0 780.0 503.0 616.0 1.95 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./200/2021-22/                                                        Dated: 21.09.2021

#### (Page – 2(ii))

#### TEST REPORTS

Recommended Mix Proportion (iv)   M-40 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-40 Grade | 0.38 : 1 : 1.70 : 1.09 : 1.34 : 0.4* 0.532 : 1 : 1.30 : 0.95 : 1.08 : 0.4* | By weight By volume |

| Sl. No. | Test | M-40 grade with admixture |
| --- | --- | --- |
| 1. | Target Strength (N/mm2) | 50.9 |
| 2. | Slump range (initial) (mm) | 125-175 |
| 3. | Slump range (after one hour ) (mm) | 100-150 |
| 4. | Slump Achieved in the Laboratory (initial) (mm) | 170.0 |
| 5. | Slump Achieved in the Laboratory(after one hour) (mm) | 115.0 |
| 6. | i) After 24 hours Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) iii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 25.0 38.5 51.5 |
| 7. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Auracast 100 (Litre) | 171.0 450.0 765.0 491.0 603.0 1.80 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./200/2021-22/                                                        Dated: 21.09.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design for (i)   M-25 grade with

Admixture (ii)  M-30 grade with admixture   (iv) M-40 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 85.75 | 100.0 | - |
|  | 10 mm | 0.45 | 86.05 | - |
|  | 4.75 mm | 0.05 | 14.85 | 5.2 |
|  | 2.36 mm | - | 0.40 | 18.8 |
|  | 1.18 mm | - | 0.05 | 49.6 |
|  | 600 micron | - | - | 82.8 |
|  | 300 micron | - | - | 97.6 |
|  | 150 micron | - | - | 99.2 |
| 2. | Fineness Modulus | 7.138 | 5.987 | 3.532 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.61 | 1.83 |
| 4. | Specific Gravity | 2.81 | 2.79 | 2.70 |
| 5. | Water absorption | 0.25 % | 0.35 % | 0.78% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./200/2021-22/                                                        Dated: 21.09.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Project Manager

M/s L & T  Constructions

RWS Empanelment Tubewell Scheme

L-34 Ashok Vihar Colony Phase-1,Varanasi, U.P.-221007

(M/s OM SHARAN INFRASTRUCTURE)

Ref.		:    		Ref. No. L & T-WET/RWS/VNS-TWS/049  Dated :10-08-2021

L & T-WET/RWS/VNS-TWS/048  Dated :06-08-2021

L & T-WET/RWS/VNS-TWS/039  Dated :12-07-2021

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grde, IS : 269, W-29, M-07 Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i)   M-25 grade with

Admixture (ii)  M-30 grade with admixture   (iv) M-40 grade with admixture

______________________________________________________________________________

Sub :  Concrete Mix design- Grades M25, M30. & M40

Projects: RWS-Empanelment, Varanasi Tube Well Scheme.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 30 % 1.0 mm Expansion 85 Minutes 275 Minutes 26.8 N/mm2 36.8 N/mm2 46.7 N/mm2 1.40 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./197/2021-22/                                                      Dated: 09.09.2021

#### TEST REPORTS

Tested for 	:      	Executive Engineer

Provincial Division, P.W.D.

Chandauli

(M/s KPC PROJECTS LIMITED)

Ref.		:    		1644/1B     Dated : 16-07-2021

Tax Invoice No. : 	000439 IIT BHU  dated 09.08.2021

Details of Sample :      	Concrete Mix Design.

Test Required       	 (i) M-25 grade with admixture

(ii) M-30 grade with admixture

(iii) M-40 grade with admixture

__________________________________________________________________________

Sub: :  Construction of Medical College Chandauli by up gradation of District Hospital Chandauli under Central Funded Scheme

(Phase-3)

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement OPC 43-grade, IS : 269, W-20,M-05, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used SmartCare MaximoPlast PC 200       .                                .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./197/2021-22/                                                      Dated: 09.09.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for  (i) M-25 grade with admixture, (ii) M-30 grade with admixture , (iii) M-40 grade

with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.09 : 1.28 : 1.93 : 0.5* 0.705 : 1 : 1.67 : 1.17 : 1.57 : 0.5* | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 1.81 : 1.15 : 1.73 : 0.5* 0.635 : 1 : 1.45 : 1.04 : 1.41 : 0.5* | By weight By volume |
| 3. | M-40 Grade | 0.40 : 1 : 1.53 : 1.02 : 1.52 : 0.5* 0.564 : 1 : 1.23 : 0.92 : 1.24 : 0.5* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture | M-30 grade with admixture | M-340 grade with admixture |
| --- | --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 | 50.9 |
| 2 | Slump range (mm) | 125-175 | 125-175 | 125-175 |
| 3 | Slump Achieved in the Laboratory (mm) | 155.0 | 150.0 | 150.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 31.0 36.0 | 37.0 42.5 | 45.0 51.5 |
| 5 | Consumption of material per cubic meter of concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) SmartCare MaximoPlast PC 200 (Litre) | 180.0 360.0 752.0 461.0 695.0 1.80 | 180.0 400.0 724.0 460.0 692.0 2.00 | 180.0 450.0 689.0 459.0 684.0 2.25 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./197/2021-22/                                                      Dated: 09.09.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design for   (i) M-25 grade with admixture,

(ii) M-30 grade with admixture , (iii) M-40 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.2 | 100.0 | - |
|  | 10 mm | 0.30 | 85.35 | - |
|  | 4.75 mm | 0.05 | 0.75 | 2.0 |
|  | 2.36 mm | - | 0.35 | 7.0 |
|  | 1.18 mm | - | 0.05 | 15.8 |
|  | 600 micron | - | - | 47.6 |
|  | 300 micron | - | - | 90.8 |
|  | 150 micron | - | - | 99.2 |
| 2. | Fineness Modulus | 7.135 | 6.135 | 2.624 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.56 | 1.76 |
| 4. | Specific Gravity | 2.82 | 2.82 | 2.65 |
| 5. | Water absorption | 0.10 % | 0.20 % | 1.10% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./197/2021-22/                                                      Dated: 09.09.2021

#### TEST REPORTS

Tested for 	:      	Executive Engineer

Provincial Division, P.W.D.

Chandauli

(M/s KPC PROJECTS LIMITED)

Ref.		:    		1644/1B     Dated : 16-07-2021

Details of Sample:	Cement: Ultra Tech Cement OPC 43-grade, IS : 269, W-20, M-05, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design  (i) M-25 grade with admixture, (ii) M-30 grade with admixture , (iii) M-40 grade with admixture

______________________________________________________________________________

Sub: :  Construction of Medical College Chandauli by up gradation of District Hospital Chandauli under Central Funded Scheme

(Phase-3)

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 33 % 1.0mm Expansion 85 Minutes 265 Minutes 26.5 N/mm2 36.5 N/mm2 46.7 N/mm2 1.41 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./136/2021-22/                                                         Dated: 06.09.2021

#### TEST REPORTS

Tested for 	:      	Assistant Engineer-III

Office of the Executive Engineer and  Sr. Manager (Civil)-III

Bodhgaya Project Division, CPWD, Room No. 309

Central University of South Bihar, SH-7, Gaya Panchanpur Marg,

Po- Fatehpur Gaya-824236, Bihar

Ref.		:    		54(2)/AE-III/EE&SM(C)-II/BGPZ/2021/210   Dated : 08-07-2021

Tax Invoice No. : 	 000323 IIT BHU dated 280.07.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  (i)  M-25 grade with admixture with Ultra Tech cement

(ii)  M-30 grade with admixture with Ultra Tech cement

(iv)  M-25 grade with admixture with ACC cement

(iii)  M-30 grade with admixture with ACC cement

________________________________________________________________________

Sub :  Design Mix for  M-25 & M-30 grade for RCC work with admixture

Work: Construction of  New Building Project at Central University of South Bihar, Panchanpur Gaya on Engineering

Procurement and Construction (EPC) basis

Agency: M/s Kashyapi Infrastructure Pvt. Ltd.)

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was (i) Ultra Tech Cement, PPC, IS : 1489, Y-2021, (ii) ACC Cement, PPC, IS : 1489, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Conplast SP 430 G8.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./136/2021-22/                                                         Dated: 06.09.2021

#### (Page – 2(i))

#### TEST REPORTS

Recommended Mix Proportion for (i) M-25 grade with admixture with Ultra Tech cement (ii)  M-30 grade

with admixture with Ultra Tech cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 1.95 : 1.31 : 1.97 : 0.5* 0.655 : 1 : 1.46 : 1.11 : 1.49 : 0.5* | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 1.68 : 1.17 : 1.77 : 0.5* 0.590 : 1 : 1.26 : 0.99 : 1.34 : 0.5* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture with Ultra Tech cement | M-30 grade with admixture with Ultra Tech cement |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 |
| 2 | Slump range (mm) | 100-150 | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 145.0 | 135.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 23.0 34.5 | 28.0 41.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Conplast SP 430 G8 (Litre) | 180.0 360.0 702.0 472.0 709.0 1.80 | 180.0 400.0 672.0 468.0 708.0 2.00 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./136/2021-22/                                                         Dated: 06.09.2021

#### (Page – 2(ii))

#### TEST REPORTS

Recommended Mix Proportion for (iii) M-25 grade with admixture with ACC cement (iv)  M-30 grade with

admixture with ACC cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 1.95 : 1.31 : 1.97 : 0.5* 0.645 : 1 : 1.44 : 1.09 : 1.47 : 0.5* | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 1.68 : 1.17 : 1.77 : 0.5* 0.581 : 1 : 1.24 : 0.97 : 1.32 : 0.5* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture with ACC cement | M-30 grade with admixture with ACC cement |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 |
| 2 | Slump range (mm) | 100-150 | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 120.0 | 110.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 24.5 36.5 | 29.0 43.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Conplast SP 430 G8 (Litre) | 180.0 360.0 702.0 472.0 709.0 1.80 | 180.0 400.0 672.0 468.0 708.0 2.00 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./136/2021-22/                                                         Dated: 06.09.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design   (i)  M-25 grade with admixture

with Ultra Tech cement  (ii)  M-30 grade with admixture with Ultra Tech cement (iv)  M-25 grade with admixture

with ACC cement (iii)  M-30 grade with admixture with ACC cement

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.15 | 100.0 | - |
|  | 10 mm | 0.40 | 88.6 | - |
|  | 4.75 mm | 0.05 | 5.6 | 2.2 |
|  | 2.36 mm | - | 0.85 | 5.8 |
|  | 1.18 mm | - | 0.10 | 16.6 |
|  | 600 micron | - | - | 43.4 |
|  | 300 micron | - | - | 89.6 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.134 | 6.049 | 2.566 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.55 | 1.75 |
| 4. | Specific Gravity | 2.82 | 2.82 | 2.63 |
| 5. | Water absorption | 0.35 % | 0.42 % | 1.63% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./136/2021-22/                                                         Dated: 06.09.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Assistant Engineer-III

Office of the Executive Engineer and  Sr. Manager (Civil)-III

Bodhgaya Project Division, CPWD, Room No. 309

Central University of South Bihar, SH-7, Gaya Panchanpur Marg,

Po- Fatehpur Gaya-824236, Bihar

Ref.		:    		54(2)/AE-III/EE&SM(C)-II/BGPZ/2021/210   Dated : 08-07-2021

Details of Sample:	Cement: (i) Ultra Tech Cement, PPC, IS : 1489, Y-2021,

(ii) ACC Cement, PPC, IS : 1489, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design  (i)  M-25 grade with admixture with Ultra Tech cement  (ii)  M-30 grade with admixture with Ultra Tech cement                  (iv)  M-25 grade with admixture with ACC cement  (iii)  M-30 grade with admixture with ACC cement

______________________________________________________________________________

Sub :  Design Mix for  M-25 & M-30 grade for RCC work with admixture

Work: Construction of  New Building Project at Central University of South Bihar, Panchanpur Gaya on Engineering

Procurement and Construction (EPC) basis

Agency: M/s Kashyapi Infrastructure Pvt. Ltd.)

| Sl.No. | Tests | Results |  |
| --- | --- | --- | --- |
|  |  | Ultra Tech Cement, PPC, IS : 1489, Y-2021, | AAC Cement, PPC, IS : 1489, Y-2021, |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 125 Minutes 355 Minutes 24.3 N/mm2 34.8 N/mm2 44.4 N/mm2 1.31 2.91 | 8 % 41 % 1.5 mm Expansion 140 Minutes 375 Minutes 24.5 N/mm2 35.3 N/mm2 44.8 N/mm2 1.29 2.89 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./135/2021-22/                                                      Dated: 06.09.2021

#### TEST REPORTS

Tested for 	:      	M/s Winsome Developer Pvt. Ltd.

Beside Trade & Facilitation Centre

VDA Colony, Chandmari,

Bada Lalpur, Varanasi-221007

Ref.		:    	813   Dated 15.07.2021

Tax Invoice No. : 	000322  IIT (BHU) dated 28.07.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  M-25 grade with admixture

________________________________________________________________________

Sub :  Design mix of M-25 grade concrete for EMPIRE STATE BUILDING, Bada Lalpur, Varanasi

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489,W-28, M-07, Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used PERMA PLAST SUPER PS-34.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./135/2021-22/                                                      Dated: 06.09.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.23 : 1.26 : 1.90 : 0.8* 0.655 : 1 : 1.65 : 1.05 : 1.44 : 0.8* | By weight By volume |

| Sl. No. | Test | M-25 Grade with admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (mm) | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 125.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 23.5 34.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) PERMA PLAST SUPER PS-34 (Litre) | 180.0 360.0 803.0 454.0 684.0 2.88 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./135/2021-22/                                                      Dated: 06.09.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade with

Admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 90.25 | 100.0 | - |
|  | 10 mm | 0.75 | 88.25 | - |
|  | 4.75 mm | 0.05 | 10.75 | 5.0 |
|  | 2.36 mm | - | 1.75 | 9.0 |
|  | 1.18 mm | - | 0.05 | 39.0 |
|  | 600 micron | - | - | 74.2 |
|  | 300 micron | - | - | 88.6 |
|  | 150 micron | - | - | 97.8 |
| 2. | Fineness Modulus | 7.090 | 5.992 | 3.136 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.57 | 1.77 |
| 4. | Specific Gravity | 2.76 | 2.87 | 2.85 |
| 5. | Water absorption | 0.25 % | 0.10 % | 1.83% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING  LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./135/2021-22/                                                      Dated: 06.09.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	M/s Winsome Developer Pvt. Ltd.

Beside Trade & Facilitation Centre

VDA Colony, Chandmari,

Bada Lalpur, Varanasi-221007

Ref.		:    	813   Dated 15.07.2021

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489,  W-28, M-07, Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with admixture

______________________________________________________________________________

Sub :  Design mix of M-25 grade concrete for EMPIRE STATE BUILDING, Bada Lalpur, Varanasi

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 120 Minutes 350 Minutes 24.7 N/mm2 34.7 N/mm2 44.1 N/mm2 1.31 2.91 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./127/2021-22/                                                      Dated: 06.09.2021

#### TEST REPORTS

Tested for 	:      	Executive Engineer

Provincial Division

P.W.D., Azamgarh

(M/s VENSA INFRASTRUCTURE LIMITED)

Ref.		:    		763 / Òou Dated 18.06.2021

Tax Invoice No. : 	000301  IIT BHU  dated 20.07.2021

Details of Sample :      	Concrete Mix Design.

Test Required       	 (i) M-20 grade with admixture

(ii) M-25 grade with admixture

(iii) M-30 grade with admixture

__________________________________________________________________________

Sub.: Design Mix.

Work:  Construction of Atal Aawasiya Vidyalaya Gambheerwan Azamgarh

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate ( sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement PPC, IS:1489, W-23, M-06, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Sika Plast- 5202 NS.                                .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./127/2021-22/                                                      Dated: 06.09.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for (i) M-20 grade with admixture (ii) M-25 grade with admixture   (iii) M-30 grade

with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-20 grade | 0.55 : 1 : 2.48 : 1.22 : 2.25 : 0.5* 0.721 : 1 : 1.85 : 1.02 : 1.69 : 0.5* | By weight By volume |
| 2. | M-25 grade | 0.50 : 1 : 2.19 : 1.12 : 2.07 : 0.5* 0.655 : 1 : 1.63 : 0.93 : 1.56 : 0.5* | By weight By volume |
| 3. | M-30 grade | 0.45 : 1 : 1.88 : 1.00 : 1.85 : 0.5* 0.590 : 1 : 1.40 : 0.83 : 1.39 : 0.5* | By weight By volume |

| Sl. No. | Test | M-20 grade with admixture | M-25 grade with admixture | M-30 grade with admixture |
| --- | --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 27.6 | 33.7 | 39.9 |
| 2 | Slump range (mm) | 75-125 | 75-125 | 75-125 |
| 3 | Slump Achieved in the Laboratory (mm) | 100.0 | 100.0 | 95.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 20.0 29.5 | 24.5 35.5 | 31.0 42.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) )Sika Plast- 5202 NS (Litre) | 176.0 320.0 794.0 390.0 720.0 1.60 | 175.0 350.0 766.0 392.0 725.0 1.75 | 175.5 390.0 733.0 390.0 722.0 1.95 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./127/2021-22/                                                      Dated: 06.09.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design   (i) M-20 grade with admixture (ii)

M-25 grade with admixture   (iii) M-30 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (Crushed sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 88.4 | 100.0 | - |
|  | 10 mm | 0.25 | 89.4 | - |
|  | 4.75 mm | 0.05 | 0.90 | 5.0 |
|  | 2.36 mm | - | 0.20 | 21.6 |
|  | 1.18 mm | - | 0.10 | 43.8 |
|  | 600 micron | - | - | 66.4 |
|  | 300 micron | - | - | 93.0 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.113 | 6.094 | 3.288 |
| 3. | Bulk Density (Kg./Litre) | 1.74 | 1.57 | 1.76 |
| 4. | Specific Gravity | 2.73 | 2.70 | 2.69 |
| 5. | Water absorption | 0.40 % | 0.45 % | 0.88% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./127/2021-22/                                                      Dated: 06.09.2021

#### TEST REPORTS

Tested for 	:      	Executive Engineer

Provincial Division

P.W.D., Azamgarh

(M/s VENSA INFRASTRUCTURE LIMITED)

Ref.		:    		763 / Òou Dated 18.06.2021

Details of Sample:	Cement: Ultra Tech Cement PPC, IS:1489, W-23, M-06, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design  Design (i) M-20 grade with admixture (ii) M-25 grade with admixture   (iii) M-30 grade with admixture

______________________________________________________________________________

Sub.: Design Mix.

Work:  Construction of Atal Aawasiya Vidyalaya Gambheerwan Azamgarh

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 125 Minutes 350 Minutes 24.4 N/mm2 34.4 N/mm2 44.0 N/mm2 1.30 2.90 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./111/2021-22/                                                           Dated: 06.09.2021

#### TEST REPORTS

Tested for 	:      	Executive Engineer

Provincial Division, P.W.D.

Chandauli

(M/s KPC PROJECTS LIMITED)

Ref.		:    		1127/1B     Dated : 02-06-2021

Tax Invoice No. : 	000264  IIT BHU  dated 06.07.2021

Details of Sample :      	Concrete Mix Design.

Test Required       	 (i) M-25 grade with admixture with crushed sand

(ii) M-30 grade with admixture with crushed sand

(iii) M-40 grade with admixture with crushed sand

__________________________________________________________________________

Sub: :  Construction of Medical College Chandauli by up gradation of District Hospital Chandauli under Central Funded Scheme

(Phase-3)

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (crushed sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement OPC 43-grade, IS : 269, W-20,M-05, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used SmartCare MaximoPlast PC 200       .                                .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./111/2021-22/                                                           Dated: 06.09.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for   (i) M-25 grade with admixture with crushed sand,  (ii) M-30 grade with admixture with

crushed sand (iii) M-40 grade with admixture with crushed sand

| Grade of Concrete |  | Water : Cement : Fly Ash :Crushed : CA-(10) : CA- (20) : Admixture Sand | Remark |
| --- | --- | --- | --- |
| 1. | M-25 grade | 0.60 : 1 : 0.20 : 2.69 : 1.40 : 2.10 : 0.6* 0.846 : 1 : 0.29 : 2.30 : 1.27 : 1.71 : 0.6* | By weight By volume |
| 2. | M-30 grade | 0.54 : 1 : 0.20 : 2.36 : 1.22 : 1.83 : 0.6* 0.761 : 1 : 0.29 : 2.02 : 1.10 : 1.49 : 0.6* | By weight By volume |
| 3. | M-40 grade | 0.46 : 1 : 0.21 : 1.88 : 1.01 : 1.52 : 0.6* 0.649 : 1 : 0.31 : 1.61 : 0.91 : 1.24 : 0.6* | By weight By volume |

| Sl. No. | Test | M-25 grade with crushed sand | M-30 grade with crushed sand | M-40 grade with crushed sand |
| --- | --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 | 50.9 |
| 2 | Slump range (mm) | 125-175 | 125-175 | 125-175 |
| 3 | Slump Achieved in the Laboratory (mm) | 150.0 | 140.0 | 145.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 26.0 36.5 | 30.5 42.5 | 38.5 52.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Fly Ash (Kg) iv) Crushed Sand (Kg) v) C.A. (10 mm) (Kg) vi) C.A.(20 mm) (Kg.) vii) ) SmartCare MaximoPlast PC 200 (Litre) | 186.0 310.0 62.0 834.0 434.0 651.0 1.86 | 186.3 345.0 69.0 814.0 421.0 631.0 2.07 | 186.3 405.0 85.0 761.0 409.0 616.0 2.43 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./111/2021-22/                                                           Dated: 06.09.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design for   (i) M-25 grade with admixture

with crushed sand,  (ii) M-30 grade with admixture with crushed sand (iii) M-40 grade with admixture with crushed sand

| S.No | Test | R E S U L T |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (Crushed sand) | Fly Ash |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |  |
|  | 40 mm | 100.0 | 100.0 | - | - |
|  | 20 mm | 86.2 | 100.0 | - | - |
|  | 10 mm | 0.30 | 85.35 | - | - |
|  | 4.75 mm | 0.05 | 0.75 | 6.0 | - |
|  | 2.36 mm | - | 0.35 | 37.2 | - |
|  | 1.18 mm | - | 0.05 | 64.2 | - |
|  | 600 micron | - | - | 81.8 | - |
|  | 300 micron | - | - | 91.2 | - |
|  | 150 micron | - | - | 96.4 | - |
| 2. | Fineness Modulus | 7.135 | 6.135 | 3.768 | - |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.56 | 1.65 | 0.97 gm/cc |
| 4. | Specific Gravity | 2.82 | 2.82 | 2.89 | 1.86 |
| 5. | Water absorption | 0.10 % | 0.20 % | 1.10% | - |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./111/2021-22/                                                           Dated: 06.09.2021

#### TEST REPORTS

Tested for 	:      	Executive Engineer

Provincial Division, P.W.D.

Chandauli

(M/s KPC PROJECTS LIMITED)

Ref.		:    		1127/1B     Dated : 02-06-2021

Details of Sample:	Cement: Ultra Tech Cement OPC 43-grade, IS : 269, W-20, M-05, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design  Design (i) M-25 grade with admixture with crushed sand,  (ii) M-30 grade with admixture with crushed sand (iii) M-40 grade with admixture with crushed sand

______________________________________________________________________________

Sub: :  Construction of Medical College Chandauli by up gradation of District Hospital Chandauli under Central Funded Scheme

(Phase-3)

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 33 % 1.0mm Expansion 85 Minutes 265 Minutes 26.5 N/mm2 36.5 N/mm2 46.7 N/mm2 1.41 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./641(i)/2021-22/ 	                                                  Dated: 02.09.2021

#### TEST REPORTS

Tested for 	:      	Executive Engineer

Chandauli Project Division

CPWD,  Chandauli

(M/s Kamal and Associates Pvt. Ltd. )

Ref.		:    	 23(2) / p0ifj0Eka / 2020 /107    dated : 22-01-2020

Tax Invoice No. : 	 001253 IIT (BHU) dated 04.03.2020

Details of Sample :      Concrete Mix Design.

Test Required       	 (i) M-25 grade with admixture with Ultra Tech Cement for self compacting

(ii) M-25 grade with admixture with ACC Cement for self compacting

________________________________________________________________________

Sub.: Design Mix of M-25 Grade Concrete.

Work: Construction of Various Infrastructure at GC, CRPF, Chandauli (UP) (SH: C/o 683 Nos. Residential Qtrs. i.e. Type-II

640 Nos., Type-III 26 Nos., Type-IV 12 Nos., and Type-V 5 Nos., including Internal Electrical Installation and

development work son EPC basis.

Agreement No. 04/EE/CP/2019-20

Agency: M/s Kamal and Associates Pvt. Ltd.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489, Y-2021 and ACC Cement, PPC, IS:1489, Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used Sika Plastiment – 100.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./641(i)/2021-22/ 	                                                  Dated: 02.09.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for  (i) M-25 grade with admixture with Ultra Tech Cement for self

compacting (ii) M-25 grade with admixture with ACC Cement for self compacting

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade with Ultra Tech | 0.50 : 1 : 2.03 : 1.37 : 1.13 : 1.1* 0.655 : 1 : 1.52 : 1.14 : 0.85 : 1.1* | By weight By volume |
| 2. | M-25 Grade with Ultra ACC | 0.50 : 1 : 2.04 : 1.38 : 1.13 : 1.15* 0.675 : 1 : 1.57 : 1.19 : 0.88 : 1.15* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture with Ultra Tech Cement | M-25 grade with admixture with ACC Cement |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 33.7 |
| 2 | An average slump flow value (mm) Achieved in the Laboratory | 650 | 660 |
| 3 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 25.5 35.5 | 25.0 35.0 |
| 4 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Sika Plastiment – 100 (Litre) | 200.0 400.0 812.0 548.0 452.0 4.40 | 200.0 400.0 816.0 552.0 452.0 4.60 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./641(i)/2021-22/ 	                                                  Dated: 02.09.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design (i) M-25 grade with

admixture with Ultra Tech Cement for self compacting (ii) M-25 grade with admixture with ACC

Cement for self compacting

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 89.05 | 100.0 | - |
|  | 10 mm | 0.20 | 85.75 | - |
|  | 4.75 mm | 0.05 | 2.65 | 2.2 |
|  | 2.36 mm | - | 0.20 | 5.4 |
|  | 1.18 mm | - | 0.10 | 15.0 |
|  | 600 micron | - | - | 51.2 |
|  | 300 micron | - | - | 93.2 |
|  | 150 micron | - | - | 99.2 |
| 2. | Fineness Modulus | 7.107 | 6.113 | 2.662 |
| 3. | Bulk Density (Kg./Litre) | 1.74 | 1.57 | 1.75 |
| 4. | Specific Gravity | 2.85 | 2.81 | 2.71 |
| 5. | Water absorption | 0.30 % | 0.25 % | 1.49% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING  LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./641(i)/2021-22/ 	                                                  Dated: 02.09.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Executive Engineer

Chandauli Project Division

CPWD,  Chandauli

(M/s Kamal and Associates Pvt. Ltd. )

Ref.		:    	 23(2) / p0ifj0Eka / 2020 /107    dated : 22-01-2020

Details of Sample:	Cement: (i) Ultra Tech Cement, PPC, IS : 1489,   Y-2021.

(ii) ACC Cement, PPC, IS : 1489,   Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design (i) M-25 grade with admixture with Ultra Tech Cement for self compacting (ii) M-25 grade with admixture with ACC Cement for self compacting

______________________________________________________________________________

Sub.: Design Mix of M-25 Grade Concrete.

Work: Construction of Various Infrastructure at GC, CRPF, Chandauli (UP) (SH: C/o 683 Nos. Residential Qtrs. i.e. Type-II

640 Nos., Type-III 26 Nos., Type-IV 12 Nos., and Type-V 5 Nos., including Internal Electrical Installation and

development work son EPC basis.

Agreement No. 04/EE/CP/2019-20

Agency: M/s Kamal and Associates Pvt. Ltd.

| Sl.No. | Tests | Results |  |
| --- | --- | --- | --- |
|  |  | Ultra Tech Cement, PPC, IS : 1489, Y-2021 | ACC Cement, PPC, IS : 1489,Y-2021 |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 60 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 130 Minutes 340 Minutes 25.2 N/mm2 34.7 N/mm2 44.4 N/mm2 1.31 2.91 | 8 % 41 % 1.5 mm Expansion 140 Minutes 355 Minutes 25.1 N/mm2 35.1 N/mm2 45.2 N/mm2 1.35 2.95 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./132/2021-22/                                                              Dated: 02.09.2021

#### TEST REPORTS

Tested for 	:      	Executive Engineer and Sr. Manager (Civil)

B.H.U. Project Varanasi-I

CPWD, Varanasi

(M/s Suri Constructions)

Ref.		:    	dk0vfHk0@ ch0,Pk0;w0ifj0ea0-1/395     dated 29-07-2020

Tax Invoice No. : 	000388  IIT (BHU) dated 03.12.2020

Details of Sample :      Concrete Mix Design.

Test Required       	  M-25 grade with plasticizer

________________________________________________________________________

Sub :  Design of cement concrete Mix M-25 with plasticizer.

Work: C/o 6th floor to 10th floor (Vertical extension) of International Boys Hostel Building including Internal E.I.,

Fire alarm System & Fire Fighting System at BHU, Varanasi

Agency: M/s Suri Constructions

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489, Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used Sika Plastiment  2001 NS.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./132/2021-22/                                                              Dated: 02.09.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade with plasticizer

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 2.00 : 1.27 : 1.90 : 0.6* 0.65 : 1 : 1.49 : 1.06 : 1.42 : 0.6* | By weight By volume |

| Sl. No. | Test | M-25 Grade with plasticizer |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (mm) | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 125.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 21.5 35.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Sika Plastiment 2001 NS (Litre) | 180.0 360.0 720.0 457.0 684.0 2.16 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./132/2021-22/                                                              Dated: 02.09.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade with

plasticizer

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.05 | 100.0 | - |
|  | 10 mm | 0.25 | 86.55 | - |
|  | 4.75 mm | 0.05 | 0.90 | 1.8 |
|  | 2.36 mm | - | 0.40 | 7.4 |
|  | 1.18 mm | - | 0.05 | 18.5 |
|  | 600 micron | - | - | 60.2 |
|  | 300 micron | - | - | 89.4 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.137 | 6.121 | 2.763 |
| 3. | Bulk Density (Kg./Litre) | 1.74 | 1.56 | 1.75 |
| 4. | Specific Gravity | 2.75 | 2.82 | 2.63 |
| 5. | Water absorption | 0.15 % | 0.21 % | 1.33% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING  LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./132/2021-22/                                                              Dated: 02.09.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Executive Engineer and Sr. Manager (Civil)

B.H.U. Project Varanasi-I

CPWD, Varanasi

(M/s Suri Constructions)

Ref.		:    	dk0vfHk0@ ch0,Pk0;w0ifj0ea0-1/395     dated 29-07-2020

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489,  W-22, M-03, Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with

plasticizer

______________________________________________________________________________

Sub :  Design of cement concrete Mix M-25 with plasticizer.

Work: C/o 6th floor to 10th floor (Vertical extension) of International Boys Hostel Building including Internal E.I.,

Fire alarm System & Fire Fighting System at BHU, Varanasi

Agency: M/s Suri Constructions

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 130 Minutes 350 Minutes 24.0 N/mm2 34.1 N/mm2 43.7 N/mm2 1.30 2.90 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./154/2021-22/                                                          Dated: 25.08.2021

#### TEST REPORTS

Tested for 	:      	Executive Engineer and Sr. Manager

Bodhgaya Project Zone-1

CPWD, Bodhgaya-824234

(M/s Ramacivil India Construction Private Limited)

Ref.		:    		54(2)/BGPD/CPWD/2021/208 (fg)  Dated :08-07-2021

Tax Invoice No. : 	 000354 IIT BHU dated 29.07.2021

Details of Sample :      Concrete Mix Design.

Test Required       	  (i)   M-10 grade with admixture

(ii)  M-25 grade with admixture

(iii)   M-30 grade with admixture

(iv)  M-35 grade with admixture

__________________________________________________________________________

Sub :  Concrete Mix design.

Work: Construction of Permanent campus of Indian Institute of Management (IIM) Bodhgaya, Bodhgaya, Bihar Phase-I SH- Construction of Academic block, MDP Education, MDP Residence, Auditorium, Sports Complex, Hostel, Mess, Professor’s Residence, Assistant Professor’s Residence, Staff Housing, Director Bungalow, servant quarter, medical centre, Utility block including Civil, electrical & Mechanical and Horticulture Works/ services/External Development works, road, Pathway, sculpture signage, internal and external electrical installation, fire Fighting, Fire Alarm, Lift, Substation, DG set, HVAC, LAN Networking, IBMS, AUDIOVISUAL SYSEM, Solar Photovoltaic Panel, STP, WTP etc on EPC Basis.

Agency : M/s Ramacivil India Construction Private Limited

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (crushed sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement PPC, IS:1489, W-20, M-05, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Sika Plast- 5061 NS.                                .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./154/2021-22/                                                          Dated: 25.08.2021

#### (Page – 2(i)

#### TEST REPORTS

Recommended Mix Proportion (i)   M-10 grade with admixture, (ii)  M-25 grade with admixture

| Grade of Concrete |  | Water : Cement : Crushed : CA-(10) : CA- (20) : Admixture Sand | Remark |
| --- | --- | --- | --- |
| 1. | M-10 Grade | 0.70 : 1 : 3.65 : 1.55 : 2.88 : 0.6* 0.91 : 1 : 2.56 : 1.27 : 2.15 : 0.6* | By weight By volume |
| 2. | M-25 Grade | 0.50 : 1 : 2.26 : 1.13 : 2.10 : 0.6* 0.65 : 1 : 1.59 : 0.92 : 1.57 : 0.6* | By weight By volume |

| Sl. No. | Test | M-10 grade with admixture | M-25 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 15.5 | 33.7 |
| 2 | Slump range (mm) | 75-125 | 75-125 |
| 3 | Slump Achieved in the Laboratory (mm) | 100.0 | 115.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 12.5 18.5 | 23.0 34.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Crushed Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Sika Plast- 5061 NS (Litre) | 180.6 258.0 942.0 400.0 743.0 1.55 | 180.0 360.0 814.0 407.0 756.0 2.16 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./154/2021-22/                                                          Dated: 25.08.2021

#### (Page – 2(ii)

#### TEST REPORTS

Recommended Mix Proportion (iii)   M-30 grade with admixture, (iv)  M-35 grade with admixture

| Grade of Concrete |  | Water : Cement : Crushed : CA-(10) : CA- (20) : Admixture Sand | Remark |
| --- | --- | --- | --- |
| 1. | M-30 Grade | 0.45 : 1 : 1.95 : 1.01 : 1.89 : 0.6* 0.585 : 1 : 1.37 : 0.83 : 1.49 : 0.6* | By weight By volume |
| 2. | M-35 Grade | 0.40 : 1 : 1.64 : 0.89 : 1.66 : 0.6* 0.52 : 1 : 1.15 : 0.73 : 1.24 : 0.6* | By weight By volume |

| Sl. No. | Test | M-30 grade with admixture | M-35 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 39.9 | 45.4 |
| 2 | Slump range (mm) | 75-125 | 75-125 |
| 3 | Slump Achieved in the Laboratory (mm) | 110.0 | 90.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 30.0 43.0 | 34.5 47.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Crushed Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Sika Plast- 5061 NS (Litre) | 180.0 400.0 780.0 404.0 756.0 2.40 | 180.0 450.0 738.0 401.0 747.0 2.70 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./154/2021-22/                                                          Dated: 25.08.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design   (i)   M-10 grade with admixture

(ii)  M-25 grade with admixture  (iii)   M-30 grade with admixture   (iv)  M-35 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (Crushed sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 94.9 | 100.0 | - |
|  | 10 mm | 1.30 | 91.25 | - |
|  | 4.75 mm | 0.05 | 14.75 | 13.6 |
|  | 2.36 mm | - | 2.40 | 35.4 |
|  | 1.18 mm | - | 0.10 | 48.6 |
|  | 600 micron | - | - | 62.8 |
|  | 300 micron | - | - | 79.4 |
|  | 150 micron | - | - | 98.0 |
| 2. | Fineness Modulus | 7.038 | 5.915 | 3.378 |
| 3. | Bulk Density (Kg./Litre) | 1.74 | 1.59 | 1.85 |
| 4. | Specific Gravity | 2.85 | 2.81 | 2.97 |
| 5. | Water absorption | 0.10 % | 0.55 % | 0.46% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./154/2021-22/                                                          Dated: 25.08.2021

#### TEST REPORTS

Tested for 	:      	Executive Engineer and Sr. Manager

Bodhgaya Project Zone-1

CPWD, Bodhgaya-824234

(M/s Ramacivil India Construction Private Limited)

Ref.		:    		54(2)/BGPD/CPWD/2021/208 (fg)  Dated :08-07-2021

Details of Sample:	Cement: Ultra Tech Cement PPC, IS:1489, W-20, M-05, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design  Design i)   M-10 grade with admixture (ii)  M-25 grade with admixture  (iii)   M-30 grade with admixture   (iv)  M-35 grade with admixture

______________________________________________________________________________

Sub :  Concrete Mix design.

Work: Construction of Permanent campus of Indian Institute of Management (IIM) Bodhgaya, Bodhgaya, Bihar Phase-I SH- Construction of Academic block, MDP Education, MDP Residence, Auditorium, Sports Complex, Hostel, Mess, Professor’s Residence, Assistant Professor’s Residence, Staff Housing, Director Bungalow, servant quarter, medical centre, Utility block including Civil, electrical & Mechanical and Horticulture Works/ services/External Development works, road, Pathway, sculpture signage, internal and external electrical installation, fire Fighting, Fire Alarm, Lift, Substation, DG set, HVAC, LAN Networking, IBMS, AUDIOVISUAL SYSEM, Solar Photovoltaic Panel, STP, WTP etc on EPC Basis.

Agency : M/s Ramacivil India Construction Private Limited

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 125 Minutes 360 Minutes 24.1 N/mm2 34.4 N/mm2 44.0 N/mm2 1.30 2.90 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./141(ii)/2021-22/ 	                                                  Dated: 23.08.2021

#### TEST REPORTS

Tested for 	:      	Head-Buxar Project Site,

Larsen and Toubro  Limited,

2x660 MW,  Buxar Thermal Power Project

Ref.		:    	L & T POWER/SJVN/BUXAR/EPC/21-22/01 Dated 15-04-2021

Tax Invoice No. : 	000362  IIT BHU dated 29.07.2021

Details of Sample :      	Concrete Mix Design with MP Birla Concrecem OPC Cement, Conplast SP 430 ES2 and Chhatarpur source of aggregate for Slump 100-150 mm.

Test Required       	 (i) M-20 grade (ii) M-25 grade (iii) M-30 grade  (iv) M-35 grade (v) M-40 grade

__________________________________________________________________________

Work: Concrete Mix Design

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was M.P. Birla Concrecem Cement, OPC 43-grade, IS : 269, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Conplast SP 430 ES2.                                .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./141(ii)/2021-22/ 	                                                  Dated: 23.08.2021

#### (Page – 2(i)

#### TEST REPORTS

Recommended Mix Proportion (i) M-20 grade (ii) M-25 grade (iii) M-30 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-20 grade | 0.55 : 1 : 2.32 : 1.38 : 2.06 : 0.8* 0.809 : 1 : 1.95 : 1.28 : 1.79 : 0.8* | By weight By volume |
| 2. | M-25 grade | 0.48 : 1 : 1.96 : 1.21 : 1.81 : 0.8* 0.706 : 1 : 1.65 : 1.13 : 1.57 : 0.8* | By weight By volume |
| 2. | M-30 grade | 0.45 : 1 : 1.77 : 1.14 : 1.71 : 0.8* 0.662 : 1 : 1.49 : 1.06 : 1.49 : 0.8* | By weight By volume |

| Sl. No. | Test | M-20 grade | M-25 grade | M-30 grade |
| --- | --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 27.6 | 33.7 | 39.9 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 | 150-200 |
| 3 | Slump range (after one hour ) (mm) | 100-150 | 100-150 | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 200.0 | 200.0 | 200.0 |
| 5 | Slump Achieved in the Laboratory(after one hour) (mm) | 150.0 | 145.0 | 145.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 23.0 29.0 | 27.5 35.5 | 31.0 41.0 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Conplast SP 430 ES2 (Litre) | 181.5 330.0 766.0 455.0 680.0 2.64 | 180.0 375.0 735.0 454.0 679.0 3.0 | 180.0 400.0 708.0 456.0 684.0 3.2 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./141(ii)/2021-22/ 	                                                  Dated: 23.08.2021

#### (Page – 2(ii)

#### TEST REPORTS

Recommended Mix Proportion (iv) M-35 grade (v) M-40 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-35 grade | 0.40 : 1 : 1.55 : 1.04 : 1.56 : 0.8* 0.588 : 1 : 1.30 : 0.97 : 1.36 : 0.8* | By weight By volume |
| 2. | M-40 grade | 0.38 : 1 : 1.51 : 1.05 : 1.58 : 1.0* 0.559 : 1 : 1.27 : 0.98 : 1.37 : 1.0* | By weight By volume |

| Sl. No. | Test | M-35 grade | M-40 grade |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 45.4 | 50.9 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 |
| 3 | Slump range (after one hour ) (mm) | 100-150 | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 195.0 | 190.0 |
| 5 | Slump Achieved in the Laboratory(after one hour) (mm) | 140.0 | 135.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 32.5 46.5 | 40.0 51.5 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Conplast SP 430 ES2 (Litre) | 176.0 440.0 682.0 458.0 686.0 3.52 | 169.1 445.0 672.0 467.0 703.0 4.45 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./141(ii)/2021-22/ 	                                                  Dated: 23.08.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design (i) M-20 grade (ii) M-25 grade

(iii)  M-30 grade  (iv) M-35 grade (v) M-40 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm (Source: Chhatarpur) | CA—10 mm Source: Chhatarpur) | FA (sand) (Source: Sone River ) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 99.5 | 100.0 | - |
|  | 10 mm | 3.25 | 90.15 | - |
|  | 4.75 mm | 0.05 | 9.75 | 1.8 |
|  | 2.36 mm | - | 0.40 | 6.4 |
|  | 1.18 mm | - | 0.05 | 24.6 |
|  | 600 micron | - | - | 54.8 |
|  | 300 micron | - | - | 90.6 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 6.972 | 5.997 | 2.770 |
| 3. | Bulk Density (Kg./Litre) | 1.69 | 1.58 | 1.75 |
| 4. | Specific Gravity | 2.70 | 2.78 | 2.67 |
| 5. | Water absorption | 0.50 % | 0.55 % | 1.09% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./141(ii)/2021-22/ 	                                                  Dated: 23.08.2021

#### TEST REPORTS

Tested for 	:      	Head-Buxar Project Site,

Larsen and Toubro  Limited,

2x660 MW,  Buxar Thermal Power Project

Ref.		:    	L & T POWER/SJVN/BUXAR/EPC/21-22/01 Dated 15-04-2021

Details of Sample:	Cement: M.P. Birla Concrecem Cement, OPC 43-grade, IS : 269, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design  Design (i) M-20 grade                 (ii) M-25 grade (iii)  M-30 grade  (iv) M-35 grade (v) M-40 grade

______________________________________________________________________________

Work: Concrete Mix Design

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 26 % 1.0 mm Expansion 115 Minutes 315 Minutes 26.8 N/mm2 36.7 N/mm2 46.5 N/mm2 1.47 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./108/2021-22/                                                      Dated: 17.08.2021

#### TEST REPORTS

Tested for 	:      	Assistant Engineer (Civil),

B.H.U. Project Subdivision-3,

CPWD, Varanasi

(M/s Kashyapi Infrastructure Pvt. Ltd.)

Ref.		:    		23(75)/ch0,Pk0;w0 Ikfj0ok0-3/2021/39  Dated : 08-06-2021

Tax Invoice No. : 	 000249 IIT BHU dated 30.06.2021

Details of Sample :      Concrete Mix Design.

Test Required       	 M-30 grade with plasticizer and M-30 grade without plasticizer

________________________________________________________________________

Sub :  Design of cement concrete Mix M-30 with plasticizer and without plasticizer.

Work: Construction of International Girls Hostel (G+10) including Internal Electrical Installations, Fire Fighting System, Fire alarm System, Lifts, DG Set, Sub-Station, CCTV & LAN Networking and external Development at BHU, Varanasi

Agency: M/s Kashyapi Infrastructure Pvt. Ltd.)

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489, Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used Conplast SP 430 G8.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./108/2021-22/                                                      Dated: 17.08.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-30 grade with plasticizer and M-30 grade without plasticizer

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-30 Grade | 0.45 : 1 : 1.71 : 1.15 : 1.71 : 0.5* 0.59 : 1 : 1.28 : 0.97 : 1.29 : 0.5* | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 1.48 : 0.99 : 1.49 : - 0.59 : 1 : 1.11 : 0.83 : 1.12 : - | By weight By volume |

| Sl. No. | Test | M-30 Grade with plasticizer | M-30 Grade without plasticizer |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 39.9 | 39.9 |
| 2 | Slump range (mm) | 100-150 | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 135.0 | 125.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 8 days Cube Compressive Strength achieved in the Laboratory (N/mm2 | 30.5 45.0 | 29.5 44.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Conplast SP 430 G8 (Litre) | 180.0 400.0 684.0 460.0 684.0 2.0 | 198.0 440.0 651.0 436.0 656.0 - |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./108/2021-22/                                                      Dated: 17.08.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-30 grade with

plasticizer and M-30 grade without plasticizer

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 89.4 | 100.0 | - |
|  | 10 mm | 0.30 | 86.6 | - |
|  | 4.75 mm | 0.05 | 5.45 | 2.6 |
|  | 2.36 mm | - | 0.60 | 7.8 |
|  | 1.18 mm | - | 0.10 | 18.8 |
|  | 600 micron | - | - | 61.4 |
|  | 300 micron | - | - | 91.2 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.103 | 6.073 | 2.808 |
| 3. | Bulk Density (Kg./Litre) | 1.74 | 1.56 | 1.75 |
| 4. | Specific Gravity | 2.78 | 2.81 | 2.60 |
| 5. | Water absorption | 0.55 % | 0.51 % | 1.75% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./108/2021-22/                                                      Dated: 17.08.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Assistant Engineer (Civil),

B.H.U. Project Subdivision-3,

CPWD, Varanasi

(M/s Kashyapi Infrastructure Pvt. Ltd.)

Ref.		:    		23(75)/ch0,Pk0;w0 Ikfj0ok0-3/2021/39  Dated : 08-06-2021

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489,  Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-30 grade with

plasticizer and M-30 grade without plasticizer

______________________________________________________________________________

Sub :  Design of cement concrete Mix M-30 with plasticizer and without plasticizer.

Work: Construction of International Girls Hostel (G+10) including Internal Electrical Installations, Fire Fighting System, Fire alarm System, Lifts, DG Set, Sub-Station, CCTV & LAN Networking and external Development at BHU, Varanasi

Agency: M/s Kashyapi Infrastructure Pvt. Ltd.)

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 130 Minutes 345 Minutes 24.9 N/mm2 35.7 N/mm2 44.8 N/mm2 1.31 2.91 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./93/2021-22/                                                      Dated: 17.08.2021

#### TEST REPORTS

Tested for 	:      	Upper Project Manager

U.P.R.N.N. Ltd.

Rajkiya Allopathic Medical College

Unit Baharaich, Camp Office Plot No. 4, Sector-7,

Gomti Nagar Extension, Lucknow-226010

( M/s ISC INFRA DEVELOPERS)

Ref.			:    	293/  vi0ifj0iz0 cgjkbp jkt0ikyh0eglh/jk0fu0fu0/21 Dated : 02-06-2021

Tax Invoice No. : 	 000193 IIT BHU dated 21.06.2021

Details of Sample :      	Concrete Mix Design.

Test Required       	 M-25 grade

___________________________________________________________________________

Sub.: Mix design M-25 grade for Construction of Government Polytechnic Mahasi Baharaich

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes   (i) 20 mm. (ii) 10 mm

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was M.P. Birla Multicem  Cement,  PPC IS : 1489, W-8, M-05, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./93/2021-22/                                                      Dated: 17.08.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M25 Grade | 0.50 : 1 : 1.73 : 1.04 : 1.57 0.655 : 1 : 1.27 : 0.90 : 1.27 | By weight By volume |

| Sl. No. | Test | M-25 grade |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (mm) | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 140.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2 | 24.5 35.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 200.0 400.0 692.0 416.0 628.0 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./93/2021-22/                                                      Dated: 17.08.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA—20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 88.55 | 100.0 | - |
|  | 10 mm | 0.50 | 86.25 | - |
|  | 4.75 mm | 0.05 | 0.55 | 4.6 |
|  | 2.36 mm | - | 0.30 | 14.6 |
|  | 1.18 mm | - | 0.10 | 32.6 |
|  | 600 micron | - | - | 61.6 |
|  | 300 micron | - | - | 94.4 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.109 | 6.128 | 3.068 |
| 3. | Bulk Density (Kg./Litre) | 1.62 | 1.52 | 1.78 |
| 4. | Specific Gravity | 2.68 | 2.65 | 2.65 |
| 5. | Water absorption | 0.10% | 0.20 % | 1.89% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./93/2021-22/                                                      Dated: 17.08.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Upper Project Manager

U.P.R.N.N. Ltd.

Rajkiya Allopathic Medical College

Unit Baharaich, Camp Office Plot No. 4, Sector-7,

Gomti Nagar Extension, Lucknow-226010

( M/s ISC INFRA DEVELOPERS)

Ref.			:    	293/  vi0ifj0iz0 cgjkbp jkt0ikyh0eglh/jk0fu0fu0/21 Dated : 02-06-2021

Details of Sample:	Cement: M.P. Birla Multicem  Cement,  PPC IS : 1489, W-8, M-05, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade

______________________________________________________________________________

Sub.: Mix design M-25 grade for Construction of Government Polytechnic Mahasi Baharaich

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 36 % 1.5 mm Expansion 135 Minutes 375 Minutes 26.0 N/mm2 37.1 N/mm2 43.6 N/mm2 1.31 2.91 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./423/2021-22/                                                      Dated: 17.08.2021

#### TEST REPORTS

Tested for 	:      	Upper Project Manager

U.P.R.N.N. Ltd. , Unit-21,

Plot No. 4, Sector-7, Gomti Nagar Extension,

Lucknow-226010

Ref.			:    	138/ e0Yks0ikfdZx/y0Ã0-21v/jk0fu0fu0/ 2021  Dated : 03-02-2021

Tax Invoice No. : 	 000975 IIT BHU dated 31.03.2021

Details of Sample :      	Concrete Mix Design.

Test Required       	 M-25 grade

___________________________________________________________________________

Sub.: Mix design M-25 grade for Construction of Multi Level Parking at Chief Engineer Office, Public Works

Department, Lucknow

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes   (i) 20 mm. (ii) 10 mm

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Mycem Cement,  PPC IS : 1489, W-23, M-06, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./423/2021-22/                                                      Dated: 17.08.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M25 Grade | 0.50 : 1 : 1.77 : 1.03 : 1.53 0.64 : 1 : 1.26 : 0.86 : 1.19 | By weight By volume |

| Sl. No. | Test | M-25 grade |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (mm) | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 120.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 24.5 34.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 200.0 400.0 708.0 412.0 612.0 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./423/2021-22/                                                      Dated: 17.08.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA—20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 91.6 | 100.0 | - |
|  | 10 mm | 0.55 | 86.4 | - |
|  | 4.75 mm | 0.05 | 7.4 | 6.4 |
|  | 2.36 mm | - | 0.75 | 17.0 |
|  | 1.18 mm | - | 0.10 | 29.8 |
|  | 600 micron | - | - | 79.6 |
|  | 300 micron | - | - | 90.8 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.078 | 6.054 | 3.226 |
| 3. | Bulk Density (Kg./Litre) | 1.64 | 1.53 | 1.80 |
| 4. | Specific Gravity | 2.68 | 2.65 | 2.65 |
| 5. | Water absorption | 0.10% | 0.20 % | 1.80% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./423/2021-22/                                                      Dated: 17.08.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Upper Project Manager

U.P.R.N.N. Ltd. , Unit-21,

Plot No. 4, Sector-7, Gomti Nagar Extension,

Lucknow-226010

Ref.			:    	138/ e0Yks0ikfdZx/y0Ã0-21v/jk0fu0fu0/ 2021  Dated : 03-02-2021

Details of Sample:	Cement: Mycem Cement,  PPC IS : 1489, W-23, M-06, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade

______________________________________________________________________________

Sub.: Mix design M-25 grade for Construction of Multi Level Parking at Chief Engineer Office, Public Works

Department, Lucknow

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 135 Minutes 365 Minutes 24.1 N/mm2 34.5 N/mm2 43.6 N/mm2 1.28 2.88 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./141(i)/2021-22/ 	                                                  Dated: 17.08.2021

#### TEST REPORTS

Tested for 	:      	Head-Buxar Project Site,

Larsen and Toubro  Limited,

2x660 MW,  Buxar Thermal Power Project

Ref.		:    	L & T POWER/SJVN/BUXAR/EPC/21-22/01 Dated 15-04-2021

Tax Invoice No. : 	000362  IIT BHU dated 29.07.2021

Details of Sample :      	Concrete Mix Design with MP Birla Concrecem OPC Cement, CICO Plast Super C711 and Chhatarpur source of aggregate for Slump 100-150 mm.

Test Required       	 (i) M-20 grade (ii) M-25 grade (iii) M-30 grade  (iv) M-35 grade (v) M-40 grade

__________________________________________________________________________

Work: Concrete Mix Design

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was M.P. Birla Concrecem Cement, OPC 43-grade, IS : 269, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used CICO Plast Super C711.                                .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./141(i)/2021-22/ 	                                                  Dated: 17.08.2021

#### (Page – 2(i)

#### TEST REPORTS

Recommended Mix Proportion (i) M-20 grade (ii) M-25 grade (iii) M-30 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-20 grade | 0.55 : 1 : 2.32 : 1.38 : 2.06 : 0.8* 0.809 : 1 : 1.95 : 1.28 : 1.79 : 0.8* | By weight By volume |
| 2. | M-25 grade | 0.48 : 1 : 1.96 : 1.21 : 1.81 : 0.8* 0.706 : 1 : 1.65 : 1.13 : 1.57 : 0.8* | By weight By volume |
| 2. | M-30 grade | 0.45 : 1 : 1.77 : 1.14 : 1.71 : 0.8* 0.662 : 1 : 1.49 : 1.06 : 1.49 : 0.8* | By weight By volume |

| Sl. No. | Test | M-20 grade | M-25 grade | M-30 grade |
| --- | --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 27.6 | 33.7 | 39.9 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 | 150-200 |
| 3 | Slump range (after one hour ) (mm) | 100-150 | 100-150 | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 200.0 | 200.0 | 200.0 |
| 5 | Slump Achieved in the Laboratory(after one hour) (mm) | 145.0 | 145.0 | 140.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2 ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2)) | 21.5 28.0 | 25.0 35.0 | 28.0 41.5 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) CICO Plast Super C711 (Litre) | 181.5 330.0 766.0 455.0 680.0 2.64 | 180.0 375.0 735.0 454.0 679.0 3.0 | 180.0 400.0 708.0 456.0 684.0 3.2 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./141(i)/2021-22/ 	                                                  Dated: 17.08.2021

#### (Page – 2(ii)

#### TEST REPORTS

Recommended Mix Proportion (iv) M-35 grade (v) M-40 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-35 grade | 0.40 : 1 : 1.55 : 1.04 : 1.56 : 0.8* 0.588 : 1 : 1.30 : 0.97 : 1.36 : 0.8* | By weight By volume |
| 2. | M-40 grade | 0.38 : 1 : 1.51 : 1.05 : 1.58 : 1.0* 0.559 : 1 : 1.27 : 0.98 : 1.37 : 1.0* | By weight By volume |

| Sl. No. | Test | M-35 grade | M-40 grade |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 45.4 | 50.9 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 |
| 3 | Slump range (after one hour ) (mm) | 100-150 | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 195.0 | 195.0 |
| 5 | Slump Achieved in the Laboratory(after one hour) (mm) | 140.0 | 135.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 31.5 46.0 | 39.5 51.0 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) CICO Plast Super C711 (Litre) | 176.0 440.0 682.0 458.0 686.0 3.52 | 169.1 445.0 672.0 467.0 703.0 4.45 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./141(i)/2021-22/ 	                                                  Dated: 17.08.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design (i) M-20 grade (ii) M-25 grade

(iii)  M-30 grade  (iv) M-35 grade (v) M-40 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm (Source: Chhatarpur) | CA—10 mm Source: Chhatarpur) | FA (sand) (Source: Sone River ) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 99.5 | 100.0 | - |
|  | 10 mm | 3.25 | 90.15 | - |
|  | 4.75 mm | 0.05 | 9.75 | 1.8 |
|  | 2.36 mm | - | 0.40 | 6.4 |
|  | 1.18 mm | - | 0.05 | 24.6 |
|  | 600 micron | - | - | 54.8 |
|  | 300 micron | - | - | 90.6 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 6.972 | 5.997 | 2.770 |
| 3. | Bulk Density (Kg./Litre) | 1.69 | 1.58 | 1.75 |
| 4. | Specific Gravity | 2.70 | 2.78 | 2.67 |
| 5. | Water absorption | 0.50 % | 0.55 % | 1.09% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./141(i)/2021-22/ 	                                                  Dated: 17.08.2021

#### TEST REPORTS

Tested for 	:      	Head-Buxar Project Site,

Larsen and Toubro  Limited,

2x660 MW,  Buxar Thermal Power Project

Ref.		:    	L & T POWER/SJVN/BUXAR/EPC/21-22/01 Dated 15-04-2021

Details of Sample:	Cement: M.P. Birla Concrecem Cement, OPC 43-grade, IS : 269, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design  Design (i) M-20 grade                 (ii) M-25 grade (iii)  M-30 grade  (iv) M-35 grade (v) M-40 grade

______________________________________________________________________________

Work: Concrete Mix Design

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 26 % 1.0 mm Expansion 115 Minutes 315 Minutes 26.8 N/mm2 36.7 N/mm2 46.5 N/mm2 1.47 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./78/2021-22/                                                      Dated: 12.08.2021

#### TEST REPORTS

Tested for 	:      	M/s JPG Engineers Pvt. Ltd.

722, 7th Floor, wave Silver Tower,

Sector-18, Noida, U.P.-201301

Ref.		:    	Nil    Dated: 24-03-2021

Tax Invoice No. : 	000121  IIT BHU dated 07.06.2021

Details of Sample :      Concrete Mix Design.

Test Required       	 M-25 grade with admixture

___________________________________________________________________________

Sub: Design Mix Grade M25

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes   (i) 20 mm. (ii) 10 mm

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was KESO RAM Birla  Shakti Cement, PPC, IS:1489,W-15, M-04, Y-2021

The properties are given in Appendix – ‘B’

4.	Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used Sika Plastiment  2001 NS.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./78/2021-22/                                                      Dated: 12.08.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 grade | 0.50 : 1 : 1.99 : 1.30 : 1.95 : 0.6* 0.645 : 1 : 1.47 : 1.07 : 1.45 : 0.6* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (mm) | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 135.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 25.0 35.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Sika Plastiment 2001 NS (Litre) | 180.0 360.0 716.0 468.0 702.0 2.16 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./78/2021-22/                                                      Dated: 12.08.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA—20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.75 | 100.0 | - |
|  | 10 mm | 0.45 | 86.05 | - |
|  | 4.75 mm | 0.05 | 1.25 | 1.6 |
|  | 2.36 mm | - | 0.85 | 5.8 |
|  | 1.18 mm | - | 0.10 | 13.8 |
|  | 600 micron | - | - | 45.6 |
|  | 300 micron | - | - | 91.6 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 7.128 | 6.118 | 2.572 |
| 3. | Bulk Density (Kg./Litre) | 1.74 | 1.57 | 1.75 |
| 4. | Specific Gravity | 2.86 | 2.83 | 2.62 |
| 5. | Water absorption | 0.10% | 0.26 % | 1.64% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./78/2021-22/                                                      Dated: 12.08.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	M/s JPG Engineers Pvt. Ltd.

722, 7th Floor, wave Silver Tower,

Sector-18, Noida, U.P.-201301

Ref.		:    	Nil    Dated: 24-03-2021

Details of Sample:	Cement: KESO RAM Birla  Shakti Cement, PPC, IS:1489,W-15, M-04, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with

admixture

______________________________________________________________________________

Sub: Design Mix Grade M25

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 130 Minutes 355 Minutes 22.4 N/mm2 32.5 N/mm2 44.0 N/mm2 1.29 2.89 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./208(v)/2021-22/                                                      Dated: 10.08.2021

#### TEST REPORTS

Tested for 	:      	Head-Buxar Project Site

Larsen and Toubro  Limited

2x660 MW, Buxar Thermal Power Project

Ref.		:    	 L & T POWER/SJVN/BUXAR/EPC/19-20/07  Dated : 28-12-2020

L & T POWER/SJVN/BUXAR/EPC/19-20/11A  Dated : 22-02-2021

Details of Sample :      Concrete Mix Design.

Test Required       	 (i) M-25 grade for piling with Ultra Tech cements OPC with Conplast SP 430 ES2   for

slump  150-200

(ii) M-25 grade for piling with Ultra Tech cements OPC with CICO Plast Super C711 for

slump  150-200

__________________________________________________________________________

Sub: Design mix.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement OPC 43-grade, IS : 269, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used  Conplast SP 430 ES2 and  CICO Plast Super C711.                                .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./208(v)/2021-22/                                                      Dated: 10.08.2021

#### (Page – 2(i)

#### TEST REPORTS

Recommended Mix Proportion (i) M-25 grade for piling with Ultra Tech cements OPC with Conplast SP 430 ES2

for slump  150-200

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade For piling | 0.45 : 1 : 1.87 : 1.21 : 1.49 : 0.8* 0.648 : 1 : 1.50 : 1.16 : 1.31 : 0.8* | By weight By volume |

| Sl. No. | Test | M-25 grade for piling with Ultra Tech cements OPC with Conplast SP 430 ES2 for slump 150-200 |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (initial) (mm) | 200-250 |
| 3 | Slump range (after 1 hour) (mm) | 150-200 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 235.0 |
| 5 | Slump Achieved in the Laboratory(after 1 hour) (mm) | 165.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 32.0 36.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Conplast SP 430 ES2 (Litre) | 180.0 400.0 748.0 484.0 596.0 3.20 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./208(v)/2021-22/                                                      Dated: 10.08.2021

#### (Page – 2(ii)

#### TEST REPORTS

Recommended Mix Proportion (ii) M-25 grade for piling with Ultra Tech cements OPC with CICO Plast Super C711

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade For piling | 0.45 : 1 : 1.87 : 1.21 : 1.49 : 0.8* 0.648 : 1 : 1.50 : 1.16 : 1.31 : 0.8* | By weight By volume |

| Sl. No. | Test | M-25 grade for piling with Ultra Tech cements OPC with CICO Plast Super C711 |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (initial) (mm) | 200-250 |
| 3 | Slump range (after 1 hour) (mm) | 150-200 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 245.0 |
| 5 | Slump Achieved in the Laboratory(after 1 hour) (mm) | 150.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 32.0 36.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) CICO Plast Super C711 (Litre) | 180.0 400.0 748.0 484.0 596.0 3.20 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./208(v)/2021-22/                                                      Dated: 10.08.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design (i) M-25 grade for piling with Ultra

Tech cements OPC with Conplast SP 430 ES2   for  slump  150-200  (ii) M-25 grade for piling with Ultra Tech cements

OPC with CICO Plast Super C711 for slump  150-200

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm (Source: REWA, M.P.) | CA—10 mm (Source: REWA, M.P.) | FA (sand) (Source: Sone River ) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 85.55 | 100.0 | - |
|  | 10 mm | 0.40 | 93.6 | - |
|  | 4.75 mm | 0.05 | 1.40 | 2.4 |
|  | 2.36 mm | - | 0.40 | 8.4 |
|  | 1.18 mm | - | 0.05 | 28.4 |
|  | 600 micron | - | - | 60.4 |
|  | 300 micron | - | - | 91.0 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.140 | 6.046 | 2.896 |
| 3. | Bulk Density (Kg./Litre) | 1.64 | 1.50 | 1.79 |
| 4. | Specific Gravity | 2.69 | 2.67 | 2.69 |
| 5. | Water absorption | 0.60 % | 0.77 % | 1.16% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

### Ref. No. Civil/Struct./208(v)/2021-22/                                                      Dated: 10.08.2021

#### TEST REPORTS

Tested for 	:      	Head-Buxar Project Site

Larsen and Toubro  Limited

2x660 MW, Buxar Thermal Power Project

Ref.		:    	 L & T POWER/SJVN/BUXAR/EPC/19-20/07  Dated : 28-12-2020

L & T POWER/SJVN/BUXAR/EPC/19-20/11A  Dated : 22-02-2021

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grade, IS : 269 , Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design  (i) M-25 grade for piling with Ultra

Tech cements OPC with Conplast SP 430 ES2   for  slump  150-200  (ii) M-25 grade for piling with Ultra Tech cements OPC with CICO Plast Super C711 for slump  150-200

______________________________________________________________________________

Sub: Design mix.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 30 % 1.0 mm Expansion 110 Minutes 320 Minutes 32.4 N/mm2 40.9 N/mm2 47.6 N/mm2 1.44 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./208(iv)/2021-22/ 	                                                  Dated: 27.07.2021

#### TEST REPORTS

Tested for 	:      	Head-Buxar Project Site

Larsen and Toubro  Limited

2x660 MW, Buxar Thermal Power Project

Ref.		:    	 L & T POWER/SJVN/BUXAR/EPC/19-20/07  Dated : 28-12-2020

L & T POWER/SJVN/BUXAR/EPC/19-20/11A  Dated : 22-02-2021

Details of Sample :      	Concrete Mix Design with CICO Plast Super C711.

Test Required       	 (i) M-20 grade with admixture with PPC Cement

(ii) M-25 grade with admixture with PPC Cement

(iii) M-30 grade with admixture with PPC Cement

(iv) M-30 grade (PQC) with admixture with PPC Cement

__________________________________________________________________________

Sub: Design mix.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Double Bull Cement, PPC, IS : 1489, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used CICO Plast Super C711.                                .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./208(iv)/2021-22/ 	                                                  Dated: 27.07.2021

#### (Page – 2(i)

#### TEST REPORTS

Recommended Mix Proportion (i) M-20 grade with admixture with PPC Cement  (ii) M-25 grade with admixture with

PPC Cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-20 grade | 0.55 : 1 : 2.26 : 1.35 : 2.03 : 0.6* 0.704 : 1 : 1.62 : 1.15 : 1.58 : 0.6* | By weight By volume |
| 2. | M-25 grade | 0.48 : 1 : 1.90 : 1.19 : 1.78 : 0.6* 0.614 : 1 : 1.36 : 1.02 : 1.39 : 0.6* | By weight By volume |

| Sl. No. | Test | M-20 grade | M-25 grade |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 27.6 | 33.7 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 |
| 3 | Slump range (after one hour ) (mm) | 100-150 | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 200.0 | 200.0 |
| 5 | Slump Achieved in the Laboratory(after one hour) (mm) | 140.0 | 140.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 21.0 28.5 | 23.5 34.5 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) CICO Plast Super C711 (Litre) | 181.5 330.0 746.0 446.0 670.0 1.98 | 180.0 375.0 713.0 446.0 668.0 2.25 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./208(iv)/2021-22/ 	                                                  Dated: 27.07.2021

#### (Page – 2(ii)

#### TEST REPORTS

Recommended Mix Proportion for  (iii) M-30 grade with admixture with PPC Cement  (iv) M-30 grade (PQC) with admixture with PPC Cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-30 grade | 0.45 : 1 : 1.72 : 1.11 : 1.67 : 0.6* 0.576 : 1 : 1.23 : 0.95 : 1.30 : 0.6* | By weight By volume |
| 2. | M-30 grade (PQC) | 0.45 : 1 : 1.75 : 1.14 : 1.70 : 0.6* 0.576 : 1 : 1.25 : 0.97 : 1.33 : 0.6* | By weight By volume |

| Sl. No. | Test | M-30 grade | M-30 grade (PQC) |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 39.9 | 39.9 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 |
| 3 | Slump range (after one hour ) (mm) | 100-150 | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 200.0 | 195.0 |
| 5 | Slump Achieved in the Laboratory(after one hour) (mm) | 135.0 | 135.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 28.0 41.0 | 26.0 41.0 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) CICO Plast Super C711 (Litre) | 180.0 400.0 688.0 444.0 668.0 2.40 | 177.75 395.0 690.0 450.0 672.0 2.37 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./208(iv)/2021-22/ 	                                                  Dated: 27.07.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design (i) M-20 grade with admixture with

PPC Cement (ii) M-25 grade with admixture with PPC Cement  (iii) M-30 grade with admixture with PPC Cement  (iv) M-30

grade (PQC) with admixture with PPC Cement

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm (Source: REWA, M.P.) | CA—10 mm (Source: REWA, M.P.) | FA (sand) (Source: Sone River ) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 85.55 | 100.0 | - |
|  | 10 mm | 0.40 | 93.6 | - |
|  | 4.75 mm | 0.05 | 1.40 | 2.4 |
|  | 2.36 mm | - | 0.40 | 8.4 |
|  | 1.18 mm | - | 0.05 | 28.4 |
|  | 600 micron | - | - | 60.4 |
|  | 300 micron | - | - | 91.0 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.140 | 6.046 | 2.896 |
| 3. | Bulk Density (Kg./Litre) | 1.64 | 1.50 | 1.79 |
| 4. | Specific Gravity | 2.69 | 2.67 | 2.69 |
| 5. | Water absorption | 0.60 % | 0.77 % | 1.16% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./208(iv)/2021-22/ 	                                                  Dated: 27.07.2021

#### TEST REPORTS

Tested for 	:      	Head-Buxar Project Site

Larsen and Toubro  Limited

2x660 MW, Buxar Thermal Power Project

Ref.		:    	 L & T POWER/SJVN/BUXAR/EPC/19-20/07  Dated : 28-12-2020

L & T POWER/SJVN/BUXAR/EPC/19-20/11A  Dated : 22-02-2021

Details of Sample:	Cement: Double Bull Cement, PPC, IS : 1489, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design  (i) M-20 grade with admixture with

PPC Cement (ii) M-25 grade with admixture with PPC Cement  (iii) M-30 grade with admixture with PPC Cement  (iv) M-30 grade (PQC) with admixture with PPC Cement

______________________________________________________________________________

Sub: Design mix.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 5 % 36 % 1.5 mm Expansion 150 Minutes 415 Minutes 24.7 N/mm2 34.5 N/mm2 41.7 N/mm2 1.30 2.90 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./208(iii)/2021-22/ 	                                                  Dated: 27.07.2021

#### TEST REPORTS

Tested for 	:      	Head-Buxar Project Site

Larsen and Toubro  Limited

2x660 MW, Buxar Thermal Power Project

Ref.		:    	 L & T POWER/SJVN/BUXAR/EPC/19-20/07  Dated : 28-12-2020

L & T POWER/SJVN/BUXAR/EPC/19-20/11A  Dated : 22-02-2021

Details of Sample :      	Concrete Mix Design with Conplast SP 430 ES2 .

Test Required       	 (i) M-20 grade with admixture with PPC Cement

(ii) M-25 grade with admixture with PPC Cement

(iii) M-30 grade with admixture with PPC Cement

(iv) M-30 grade (PQC) with admixture with PPC Cement

__________________________________________________________________________

Sub: Design mix.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Double Bull Cement, PPC, IS : 1489, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used Conplast SP 430 ES2.                                .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./208(iii)/2021-22/ 	                                                  Dated: 27.07.2021

#### (Page – 2(i)

#### TEST REPORTS

Recommended Mix Proportion (i) M-20 grade with admixture with PPC Cement  (ii) M-25 grade with admixture with

PPC Cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-20 grade | 0.55 : 1 : 2.26 : 1.35 : 2.03 : 0.6* 0.704 : 1 : 1.62 : 1.15 : 1.58 : 0.6* | By weight By volume |
| 2. | M-25 grade | 0.48 : 1 : 1.90 : 1.19 : 1.78 : 0.6* 0.614 : 1 : 1.36 : 1.02 : 1.39 : 0.6* | By weight By volume |

| Sl. No. | Test | M-20 grade | M-25 grade |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 27.6 | 33.7 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 |
| 3 | Slump range (after one hour ) (mm) | 100-150 | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 200.0 | 195.0 |
| 5 | Slump Achieved in the Laboratory(after one hour) (mm) | 145.0 | 140.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 21.5 28.5 | 24.5 34.5 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Conplast SP 430 ES2 (Litre) | 181.5 330.0 746.0 446.0 670.0 1.98 | 180.0 375.0 713.0 446.0 668.0 2.25 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./208(iii)/2021-22/ 	                                                  Dated: 27.07.2021

#### (Page – 2(ii)

#### TEST REPORTS

Recommended Mix Proportion for  (iii) M-30 grade with admixture with PPC Cement  (iv) M-30 grade (PQC) with admixture with PPC Cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-30 grade | 0.45 : 1 : 1.72 : 1.11 : 1.67 : 0.6* 0.576 : 1 : 1.23 : 0.95 : 1.30 : 0.6* | By weight By volume |
| 2. | M-30 grade (PQC) | 0.45 : 1 : 1.75 : 1.14 : 1.70 : 0.6* 0.576 : 1 : 1.25 : 0.97 : 1.33 : 0.6* | By weight By volume |

| Sl. No. | Test | M-30 grade | M-30 grade (PQC) |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 39.9 | 39.9 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 |
| 3 | Slump range (after one hour ) (mm) | 100-150 | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 200.0 | 195.0 |
| 5 | Slump Achieved in the Laboratory(after one hour) (mm) | 140.0 | 145.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 28.0 41.0 | 26.5 41.0 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Conplast SP 430 ES2 (Litre) | 180.0 400.0 688.0 444.0 668.0 2.40 | 177.75 395.0 690.0 450.0 672.0 2.37 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./208(iii)/2021-22/ 	                                                  Dated: 27.07.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix Design (i) M-20 grade with admixture with

PPC Cement (ii) M-25 grade with admixture with PPC Cement  (iii) M-30 grade with admixture with PPC Cement  (iv) M-30

grade (PQC) with admixture with PPC Cement

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm (Source: REWA, M.P.) | CA—10 mm (Source: REWA, M.P.) | FA (sand) (Source: Sone River ) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 85.55 | 100.0 | - |
|  | 10 mm | 0.40 | 93.6 | - |
|  | 4.75 mm | 0.05 | 1.40 | 2.4 |
|  | 2.36 mm | - | 0.40 | 8.4 |
|  | 1.18 mm | - | 0.05 | 28.4 |
|  | 600 micron | - | - | 60.4 |
|  | 300 micron | - | - | 91.0 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.140 | 6.046 | 2.896 |
| 3. | Bulk Density (Kg./Litre) | 1.64 | 1.50 | 1.79 |
| 4. | Specific Gravity | 2.69 | 2.67 | 2.69 |
| 5. | Water absorption | 0.60 % | 0.77 % | 1.16% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./208(iii)/2021-22/ 	                                                  Dated: 27.07.2021

#### TEST REPORTS

Tested for 	:      	Head-Buxar Project Site

Larsen and Toubro  Limited

2x660 MW, Buxar Thermal Power Project

Ref.		:    	 L & T POWER/SJVN/BUXAR/EPC/19-20/07  Dated : 28-12-2020

L & T POWER/SJVN/BUXAR/EPC/19-20/11A  Dated : 22-02-2021

Details of Sample:	Cement: Double Bull Cement, PPC, IS : 1489, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design  (i) M-20 grade with admixture with

PPC Cement (ii) M-25 grade with admixture with PPC Cement  (iii) M-30 grade with admixture with PPC Cement  (iv) M-30 grade (PQC) with admixture with PPC Cement

______________________________________________________________________________

Sub: Design mix.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 5 % 36 % 1.5 mm Expansion 150 Minutes 415 Minutes 24.7 N/mm2 34.5 N/mm2 41.7 N/mm2 1.30 2.90 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./208(ii)/2021-22/ 	                                                  Dated: 27.07.2021

#### TEST REPORTS

Tested for 	:      	Head-Buxar Project Site

Larsen and Toubro  Limited

2x660 MW, Buxar Thermal Power Project

Ref.		:    	 L & T POWER/SJVN/BUXAR/EPC/19-20/07  Dated : 28-12-2020

L & T POWER/SJVN/BUXAR/EPC/19-20/11A  Dated : 22-02-2021

Details of Sample :      	Concrete Mix Design with CICO Plast Super C711.

Test Required       	 (i) M-20 grade with admixture with OPC Cement

(ii) M-25 grade with admixture with OPC Cement

(iii) M-30 grade with admixture with OPC Cement

(iv) M-35 grade with admixture with OPC Cement

__________________________________________________________________________

Sub: Design mix.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 43-grade, IS:269,  Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used CICO Plast Super C711.                                .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./208(ii)/2021-22/ 	                                                  Dated: 27.07.2021

#### (Page – 2(i)

#### TEST REPORTS

Recommended Mix Proportion for  (i) M-20 grade with admixture with OPC Cement (ii) M-25 grade with

admixture with  OPC Cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-20 grade | 0.55 : 1 : 2.39 : 1.43 : 2.14 : 0.6* 0.792 : 1 : 1.92 : 1.37 : 1.88 : 0.6* | By weight By volume |
| 2. | M-25 grade | 0.48 : 1 : 2.00 : 1.25 : 1.87 : 0.6* 0.691 : 1 : 1.61 : 1.20 : 1.64 : 0.6* | By weight By volume |

| Sl. No. | Test | M-20 grade | M-25 grade |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 27.6 | 33.7 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 |
| 3 | Slump range (after one hour ) (mm) | 100-150 | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 200.0 | 200.0 |
| 5 | Slump Achieved in the Laboratory(after one hour) (mm) | 150.0 | 140.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 25.0 31.0 | 31.0 35.5 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) CICO Plast Super C711 (Litre) | 176.0 320.0 765.0 458.0 685.0 1.92 | 175.2 365.0 730.0 456.0 683.0 2.19 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./208(ii)/2021-22/ 	                                                  Dated: 27.07.2021

#### (Page – 2(ii)

#### TEST REPORTS

Recommended Mix Proportion for (iii)  M-30 grade with admixture with OPC Cement   (iv)  M-35 grade  with

admixture with  OPC Cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-30 grade | 0.45 : 1 : 1.81 : 1.17 : 1.76 : 0.6* 0.648 : 1 : 1.46 : 1.12 : 1.55 : 0.6* | By weight By volume |
| 2. | M-35 grade | 0.40 : 1 : 1.52 : 1.03 : 1.55 : 0.6* 0.576 : 1 : 1.22 : 0.99 : 1.36 : 0.6* | By weight By volume |

| Sl. No. | Test | M-30 grade | M-35 grade |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 39.9 | 45.4 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 |
| 3 | Slump range (after one hour ) (mm) | 100-150 | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 200.0 | 195.0 |
| 5 | Slump Achieved in the Laboratory(after one hour) (mm) | 135.0 | 135.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 35.5 41.0 | 38.0 46.0 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) CICO Plast Super C711 (Litre) | 175.5 390.0 706.0 456.0 686.0 2.34 | 176.0 440.0 669.0 453.0 682.0 2.64 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./208(ii)/2021-22/ 	                                                  Dated: 27.07.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design  (i) M-20 grade with admixture

with OPC Cement (ii) M-25 grade with admixture with OPC Cement (iii) M-30 grade with admixture with OPC

Cement (iv) M-35 grade with admixture with OPC Cement

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm (Source: REWA, M.P.) | CA—10 mm (Source: REWA, M.P.) | FA (sand) (Source: Sone River ) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 85.55 | 100.0 | - |
|  | 10 mm | 0.40 | 93.6 | - |
|  | 4.75 mm | 0.05 | 1.40 | 2.4 |
|  | 2.36 mm | - | 0.40 | 8.4 |
|  | 1.18 mm | - | 0.05 | 28.4 |
|  | 600 micron | - | - | 60.4 |
|  | 300 micron | - | - | 91.0 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.140 | 6.046 | 2.896 |
| 3. | Bulk Density (Kg./Litre) | 1.64 | 1.50 | 1.79 |
| 4. | Specific Gravity | 2.69 | 2.67 | 2.69 |
| 5. | Water absorption | 0.60 % | 0.77 % | 1.16% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./208(ii)/2021-22/ 	                                                  Dated: 27.07.2021

#### TEST REPORTS

Tested for 	:      	Head-Buxar Project Site

Larsen and Toubro  Limited

2x660 MW, Buxar Thermal Power Project

Ref.		:    	 L & T POWER/SJVN/BUXAR/EPC/19-20/07  Dated : 28-12-2020

L & T POWER/SJVN/BUXAR/EPC/19-20/11A  Dated : 22-02-2021

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grade, IS : 269 ,W-03, M-01, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design  (i) M-20 grade with admixture with  OPC Cement (ii) M-25 grade with admixture with OPC Cement (iii) M-30 grade with admixture with OPC Cement (iv) M-35 grade with admixture with OPC Cement

______________________________________________________________________________

Sub: Design mix.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 30 % 1.0 mm Expansion 110 Minutes 320 Minutes 32.4 N/mm2 40.9 N/mm2 47.6 N/mm2 1.44 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./208(i)/2021-22/ 	                                                  Dated: 27.07.2021

Tested for 	:      	Head-Buxar Project Site

Larsen and Toubro  Limited

2x660 MW, Buxar Thermal Power Project

Ref.		:    	 L & T POWER/SJVN/BUXAR/EPC/19-20/07  Dated : 28-12-2020

L & T POWER/SJVN/BUXAR/EPC/19-20/11A  Dated : 22-02-2021

Details of Sample :      Concrete Mix Design.

Test Required       	 (i) M-40 grade with conplast SP430ES2 admixture with Ultra Tech cements OPC for

slump 100-150

(ii) M-40 grade with CICO Plast Super C711 admixture with Ultra Tech cements OPC for

slump  100-150

__________________________________________________________________________

Sub: Design mix.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 43-grade, IS:269,  Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used Conplast SP 430 ES2 and CICO Plast Super C711.

.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./208(i)/2021-22/ 	                                                  Dated: 27.07.2021

#### (Page – 2(i)

#### TEST REPORTS

Recommended Mix Proportion for  (i) M-40 grade with conplast SP430ES2 admixture with Ultra Tech cements

OPC for slump 100-150

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-40 Grade with SP430ES2 | 0.375 : 1 : 1.48 : 1.05 : 1.58 : 0.9* 0.540 : 1 : 1.19 : 1.01 : 1.39 : 0.9* | By weight By volume |

| Sl. No. | Test | M-40 grade with Ultra Tech OPC Cement and SP430ES2 admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 50.9 |
| 2 | Slump range (initial) (mm) | 150-200 |
| 3 | Slump range (after 1 hour) (mm) | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 200.0 |
| 5 | Slump Achieved in the Laboratory(after 1 hour) (mm) | 150.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 40.0 51.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Conplast SP 430 ES2 (Litre) | 166.5 444.0 657.0 466.0 702.0 3.996 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./208(i)/2021-22/ 	                                                  Dated: 27.07.2021

#### (Page – 2(ii)

#### TEST REPORTS

Recommended Mix Proportion for (ii) M-40 grade with CICO Plast Super C711 admixture with Ultra Tech cements

OPC for slump 100-150

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-40 Grade with CICO Plast Super | 0.375 : 1 : 1.48 : 1.05 : 1.58 : 0.9* 0.540 : 1 : 1.19 : 1.01 : 1.39 : 0.9* | By weight By volume |

| Sl. No. | Test | M-40 grade with Ultra Tech OPC Cement and CICO Plast Super admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 50.9 |
| 2 | Slump range (initial) (mm) | 150-200 |
| 3 | Slump range (after 1 hour) (mm) | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 200.0 |
| 5 | Slump Achieved in the Laboratory(after 1 hour) (mm) | 140.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 39.5 51.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) CICO Plast Super C711 (Litre) | 166.5 444.0 657.0 466.0 702.0 3.996 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./208(i)/2021-22/ 	                                                  Dated: 27.07.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design   (i) M-40 grade with conplast

SP430ES2 admixture with Ultra Tech cements OPC for  slump 100-150 (ii) M-40 grade with CICO Plast Super

C711 admixture with Ultra Tech cements OPC for  slump  100-150

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm (Source: REWA, M.P.) | CA—10 mm (Source: REWA, M.P.) | FA (sand) (Source: Sone River ) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 85.55 | 100.0 | - |
|  | 10 mm | 0.40 | 93.6 | - |
|  | 4.75 mm | 0.05 | 1.40 | 2.4 |
|  | 2.36 mm | - | 0.40 | 8.4 |
|  | 1.18 mm | - | 0.05 | 28.4 |
|  | 600 micron | - | - | 60.4 |
|  | 300 micron | - | - | 91.0 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.140 | 6.046 | 2.896 |
| 3. | Bulk Density (Kg./Litre) | 1.64 | 1.50 | 1.79 |
| 4. | Specific Gravity | 2.69 | 2.67 | 2.69 |
| 5. | Water absorption | 0.60 % | 0.77 % | 1.16% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./208(i)/2021-22/ 	                                                  Dated: 27.07.2021

#### TEST REPORTS

Tested for 	:      	Head-Buxar Project Site

Larsen and Toubro  Limited

2x660 MW, Buxar Thermal Power Project

Ref.		:    	 L & T POWER/SJVN/BUXAR/EPC/19-20/07  Dated : 28-12-2020

L & T POWER/SJVN/BUXAR/EPC/19-20/11A  Dated : 22-02-2021

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grade, IS : 269 , Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design  Design   (i) M-40 grade with conplast SP430ES2 admixture with Ultra Tech cements OPC for  slump 100-150 (ii)   M-40 grade with CICO Plast Super C711 admixture with Ultra Tech cements OPC for  slump  100-150

______________________________________________________________________________

Sub: Design mix.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 30 % 1.0 mm Expansion 110 Minutes 320 Minutes 32.4 N/mm2 40.9 N/mm2 47.6 N/mm2 1.44 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./109(i)/2021-22/ 	                                                         Dated: 07.07.2021

#### TEST REPORTS

Tested for 	:      	M/s NCC Limited

H/o Rajesh Kumar Pandey, Plot No. 411,

Sarswati  Vihar  Colony, Bharouna Chowraha (Under Baroudha),

Mirzapur-231001

Ref.		:    	NCC/W&E/Ahugi & Mahadev/2021-22/01  dated 05.04.2021

Tax Invoice No. : 	 000261 IIT BHU  dated 06.07.2021

Details of Sample :      	Concrete Mix Design.

Test Required       	 M-25 grade with admixture and M-30 grade with admixture

___________________________________________________________________________

Sub.: Design & Construction of Ahugi Kala & Mahadev group of villages Water Supply Scheme, District Mirzapur (Based

on Surface Water). Conducting Design mix of RCC Concrete Works – reg.

Ref. No: 1) Ahugi kala group of villages agreement no. 8/ED/2020-21  Dt. 18.06.2020

2) Mahadev group of villages agreement no.  7/ED/2020-21   Dt. 18.06.2020

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes   (i) 20 mm. (ii) 10 mm

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was KJS Cement, OPC 43-grade, IS : 269,W-14, M-04, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7.   Admixture used Sikament  5206 NS.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./109(i)/2021-22/ 	                                                         Dated: 07.07.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade with admixture and M-30grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 grade | 0.50 : 1 : 2.23 : 1.09 : 2.03 : 0.5* 0.705 : 1 : 1.79 : 0.98 : 1.65 : 0.5* | By weight By volume |
| 2. | M-30 grade | 0.45 : 1 : 1.91 : 0.98 : 1.82 : 0.5* 0.635 : 1 : 1.53 : 0.88 : 1.48 : 0.5* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture | M-30 grade with admixture |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 | 39.9 |
| 2 | Slump range (mm) | 50-100 | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 70.0 | 80.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 25.5 36.5 | 34.5 43.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Sikament 5206 NS (Litre) | 175.0 350.0 780.0 382.0 711.0 1.75 | 175.5 390.0 745.0 382.0 710.0 1.95 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./109(i)/2021-22/ 	                                                         Dated: 07.07.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade with admixture and M-30 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA—20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 97.1 | 100.0 | - |
|  | 10 mm | 0.30 | 91.25 | - |
|  | 4.75 mm | 0.05 | 0.90 | 5.2 |
|  | 2.36 mm | - | 0.35 | 23.2 |
|  | 1.18 mm | - | 0.05 | 56.8 |
|  | 600 micron | - | - | 92.2 |
|  | 300 micron | - | - | 98.2 |
|  | 150 micron | - | - | 98.8 |
| 2. | Fineness Modulus | 7.026 | 6.075 | 3.744 |
| 3. | Bulk Density (Kg./Litre) | 1.74 | 1.57 | 1.76 |
| 4. | Specific Gravity | 2.64 | 2.64 | 2.71 |
| 5. | Water absorption | 0.70% | 0.55 % | 0.90% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./109(i)/2021-22/ 	                                                         Dated: 07.07.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	M/s NCC Limited

H/o Rajesh Kumar Pandey, Plot No. 411,

Sarswati  Vihar  Colony, Bharouna Chowraha (Under Baroudha),

Mirzapur-231001

Ref.		:    	NCC/W&E/Ahugi & Mahadev/2021-22/01  dated 05.04.2021

Details of Sample:	Cement: KJS Cement, OPC 43-grade, IS : 269,  W-14, M-04, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with

admixture and M-30 grade with admixture

______________________________________________________________________________

Sub.: Design & Construction of Ahugi Kala & Mahadev group of villages Water Supply Scheme, District Mirzapur (Based

on Surface Water). Conducting Design mix of RCC Concrete Works – reg.

Ref. No: 1) Ahugi kala group of villages agreement no. 8/ED/2020-21  Dt. 18.06.2020

2) Mahadev group of villages agreement no.  7/ED/2020-21   Dt. 18.06.2020

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 32 % 1.0 mm Expansion 85 Minutes 375 Minutes 26.7 N/mm2 37.2 N/mm2 46.5 N/mm2 1.41 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./83/2021-22/ 	                                                         Dated: 22.06.2021

#### TEST REPORTS

Tested for 	:      	Executive Engineer

Scarcity Division, UP Jal Nigam,

Bathua Gandhighat, Nakahara Road,

Mirzapur-231001

(M/s GANNON DUNKERLEY & CO LIMITED)

Ref.		:    	813/AMRUT-GANNON/37   Dated : 19-03-2021

Tax Invoice No. : 	 000144 IIT BHU  dated 09.06.2021

Details of Sample :      	Concrete Mix Design.

Test Required       	 M-30 grade with admixture

___________________________________________________________________________

Project: Mirzapur water Supply Reorganization Scheme’s Package-2

Sub.: Design mix of Concrete for Construction.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes   (i) 20 mm. (ii) 10 mm

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 43-grade, IS : 269,  Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2009.

5.   Saturated surface dry aggregates were used as per IS: 10262-2009.

6.   Degree of control - Good.

7. Admixture used Conplast SP 430 G8.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./83/2021-22/ 	                                                         Dated: 22.06.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-30grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-30 grade | 0.45 : 1 : 1.73 : 1.12 : 2.02 : 0.5* 0.635 : 1 : 1.39 : 1.01 : 1.64 : 0.5* | By weight By volume |

| Sl. No. | Test | M-30 grade with admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 39.9 |
| 2 | Slump range (mm) | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 70.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 32.0 42.0 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Conplast SP 430 G8 (Litre) | 175.5 390.0 675.0 437.0 788.0 1.95 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./83/2021-22/ 	                                                         Dated: 22.06.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-30 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA—20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.15 | 100.0 | - |
|  | 10 mm | 0.20 | 85.80 | - |
|  | 4.75 mm | 0.05 | 0.35 | 2.8 |
|  | 2.36 mm | - | 0.20 | 5.8 |
|  | 1.18 mm | - | 0.10 | 16.6 |
|  | 600 micron | - | - | 56.4 |
|  | 300 micron | - | - | 88.8 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.136 | 6.136 | 2.694 |
| 3. | Bulk Density (Kg./Litre) | 1.74 | 1.56 | 1.75 |
| 4. | Specific Gravity | 2.84 | 2.81 | 2.65 |
| 5. | Water absorption | 0.20% | 0.35 % | 1.45% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./83/2021-22/ 	                                                         Dated: 22.06.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Executive Engineer

Scarcity Division, UP Jal Nigam,

Bathua Gandhighat, Nakahara Road,

Mirzapur-231001

(M/s GANNON DUNKERLEY & CO LIMITED)

Ref.		:    	813/AMRUT-GANNON/37   Dated : 19-03-2021

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grade, IS : 269,  Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-30 grade with

admixture

______________________________________________________________________________

Project: Mirzapur water Supply Reorganization Scheme’s Package-2

Sub.: Design mix of Concrete for Construction.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 32 % 1.0 mm Expansion 75 Minutes 340 Minutes 26.5 N/mm2 37.6 N/mm2 47.7 N/mm2 1.41 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./18/2021-22/ 	                                                           Dated: 21.06.2021

#### TEST REPORTS

Tested for 	:      	Executive Engineer and Sr. Manager (Civil)

B.H.U. Project Varanasi-3

CPWD, Varanasi

(M/s Tempcon Engineers)

Ref.		:    	23(150) /ok0izks0ea0-3 / 2021 /189 Dated : 27-03-2021

Tax Invoice No. : 	 000043 IIT BHU  dated 09.04.2021

Details of Sample :      Concrete Mix Design.

Test Required       	 M-30 grade without plasticizer and M-30 grade with plasticizer

__________________________________________________________________________

Sub.: Design of cement concrete mix M-30 with plasticizer and without plasticizer.

Work: C/o Phase-II for Vaidik Vigyan Kendra (G+2) (foundation for (G+3) including Electrical Installation, Fire Fighting System, Fire Alarm System, Air Conditioning System and CCTV & LAN Networking System at BHU, Varanasi

Agency: M/s Tempcon Engineers

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, PPC, IS : 1489, W-12,  Y-2021.

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2009.

5.   Saturated surface dry aggregates were used as per IS: 10262-2009.

6.   Degree of control - Good.

7. Admixture used Conplast SP 430 G8.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./18/2021-22/ 	                                                           Dated: 21.06.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-30 grade without plasticizer and M-30 grade with plasticizer

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-30 Grade | 0.45 : 1 : 1.47 : 1.02 : 1.53 : - 0.590 : 1 : 1.10 : 0.86 : 1.16 : - | By weight By volume |
| 2. | M-30 Grade | 0.45 : 1 : 1.69 : 1.18 : 1.77 : 0.5* 0.590 : 1 : 1.27 : 1.00 : 1.34 : 0.5* | By weight By volume |

| Sl. No. | Test | M-30 grade without plasticizer | M-30 grade with plasticizer |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 39.9 | 39.9 |
| 2 | Slump range (mm) | 100-150 | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 130.0 | 135.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 27.5 40.5 | 28.0 41.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) Conplast SP 430 G8 (Litre) | 198.0 440.0 647.0 449.0 673.0 - | 180.0 400.0 676.0 472.0 708.0 2.00 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./18/2021-22/ 	                                                           Dated: 21.06.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-30 grade without

plasticizer and M-30 grade with plasticizer

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.75 | 100.0 | - |
|  | 10 mm | 0.15 | 86.2 | - |
|  | 4.75 mm | 0.05 | 0.65 | 2.2 |
|  | 2.36 mm | - | 0.40 | 5.8 |
|  | 1.18 mm | - | 0.10 | 16.4 |
|  | 600 micron | - | - | 56.4 |
|  | 300 micron | - | - | 88.6 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.131 | 6.127 | 2.684 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.55 | 1.75 |
| 4. | Specific Gravity | 2.85 | 2.80 | 2.65 |
| 5. | Water absorption | 0.25 % | 0.55 % | 1.65% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./18/2021-22/ 	                                                           Dated: 21.06.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	Executive Engineer and Sr. Manager (Civil)

B.H.U. Project Varanasi-3

CPWD, Varanasi

(M/s Tempcon Engineers)

Ref.		:    	23(150) /ok0izks0ea0-3 / 2021 /189 Dated : 27-03-2021

Details of Sample:	Cement: Ultra Tech Cement, PPC, IS : 1489, W-12,  Y-2021.

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-30 grade without

plasticizer and M-30 grade with plasticizer

______________________________________________________________________________

Sub.: Design of cement concrete mix M-30 with plasticizer and without plasticizer.

Work: C/o Phase-II for Vaidik Vigyan Kendra (G+2) (foundation for (G+3) including Electrical Installation, Fire Fighting System, Fire Alarm System, Air Conditioning System and CCTV & LAN Networking System at BHU, Varanasi

Agency: M/s Tempcon Engineers

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 36 % 1.5 mm Expansion 115 Minutes 345 Minutes 25.3 N/mm2 34.4 N/mm2 45.2 N/mm2 1.31 2.91 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./75(i)/2021-22/ 	                                                          Dated: 21.06.2021

#### TEST REPORTS

Tested for 	:      	DGM (FQA)

Kanti Bijlee Utpadan Nigam Limited,

Muzafrpur Thermal Power Station

P.O. Kanti Thermal, Dist. Muzaffarpur – 843130, Bihar

(M/s Aneja Constructions (India) Limited)

Ref.		:    	KBUNL/FQA/ANEJA/5500036679/2021/02  Dated : 22-03-2020

Details of Sample :      Concrete Mix Design.

Test Required       	 M-20 grade and M-25 grade

___________________________________________________________________________

Sub.: Concrete Mix design.

Work: Balance Civil Work of Lagoon-I and Over flow Lagoon package

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes   (i) 20 mm. (ii) 10 mm

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Infracem Cement, PPC IS : 1489, W-04,  Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2009.

5.   Saturated surface dry aggregates were used as per IS: 10262-2009.

6.   Degree of control - Good.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./75(i)/2021-22/ 	                                                          Dated: 21.06.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-20 grade & M-25 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M-20 Grade | 0.55 : 1 : 2.05 : 1.18 : 2.19 0.715 : 1 : 1.51 : 0.98 : 1.64 | By weight By volume |
| 2. | M-25 Grade | 0.50 : 1 : 1.78 : 1.06 : 1.98 0.65 : 1 : 1.31 : 0.88 : 1.48 | By weight By volume |

| Sl. No. | Test | M-20 grade | M-25 grade |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 27.6 | 33.7 |
| 2 | Slump range (mm) | 50-100 | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 80.0 | 75.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 21.5 31.5 | 27.5 35.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 189.75 345.0 707.0 407.0 755.0 | 190.0 380.0 676.0 403.0 752.0 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./75(i)/2021-22/ 	                                                          Dated: 21.06.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-20 grade &                     M-25 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA—20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.75 | 100.0 | - |
|  | 10 mm | 0.35 | 89.2 | - |
|  | 4.75 mm | 0.05 | 0.65 | 1.8 |
|  | 2.36 mm | - | 0.45 | 5.8 |
|  | 1.18 mm | - | 0.10 | 17.2 |
|  | 600 micron | - | - | 49.0 |
|  | 300 micron | - | - | 89.8 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.129 | 6.096 | 2.626 |
| 3. | Bulk Density (Kg./Litre) | 1.74 | 1.57 | 1.76 |
| 4. | Specific Gravity | 2.84 | 2.74 | 2.66 |
| 5. | Water absorption | 0.30% | 0.45 % | 1.50% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./75(i)/2021-22/ 	                                                          Dated: 21.06.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	DGM (FQA)

Kanti Bijlee Utpadan Nigam Limited,

Muzafrpur Thermal Power Station

P.O. Kanti Thermal, Dist. Muzaffarpur – 843130, Bihar

(M/s Aneja Constructions (India) Limited)

Ref.		:    	KBUNL/FQA/ANEJA/5500036679/2021/02  Dated : 22-03-2020

Details of Sample:	Cement: Infracem Cement, PPC IS : 1489, W-04,  Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-20 grade &

M-25 grade

______________________________________________________________________________

Sub.: Concrete Mix design.

Work: Balance Civil Work of Lagoon-I and Over flow Lagoon package

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 36 % 1.5 mm Expansion 120 Minutes 350 Minutes 25.1 N/mm2 34.7 N/mm2 44.5 N/mm2 1.30 2.91 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./76(i)/2021-22/ 	                                                         Dated: 21.06.2021

#### TEST REPORTS

Tested for 	:      	DGM (FQA)

Kanti Bijlee Utpadan Nigam Limited,

Muzafrpur Thermal Power Station

P.O. Kanti Thermal, Dist. Muzaffarpur – 843130, Bihar

(M/s Kumar Piyush Construction Private Limited)

Ref.		:    	KBUNL/FQA/KPCPL/5500037640/2021  Dated : 06-04-2021

Tax Invoice No. : 	 000122 IIT BHU  dated 07.06.2021

Details of Sample :      	Concrete Mix Design.

Test Required       	 M-25 grade with admixture

___________________________________________________________________________

Sub.: Concrete Mix design.

Work: Construction of Pedestal and Culverts and Thrust block for Ash Slurry Disposal Package

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes   (i) 20 mm. (ii) 10 mm

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Infracem Cement, PPC IS : 1489, W-04,  Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2009.

5.   Saturated surface dry aggregates were used as per IS: 10262-2009.

6.   Degree of control - Good.

7. Admixture used Sikament 2004 NS.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./76(i)/2021-22/ 	                                                         Dated: 21.06.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade with admixture

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-25 grade | 0.50 : 1 : 2.00 : 1.31 : 1.96 : 0.5* 0.65 : 1 : 1.49 : 1.11 : 1.47 : 0.5* | By weight By volume |

| Sl. No. | Test | M-25 grade with admixture |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (initial) (mm) | 100-150 |
| 3 | Slump range (after 1 hour) (mm) | 50-100 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 130.0 |
| 5 | Slump Achieved in the Laboratory(after 1 hour ) (mm) | 70.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 26.5 35.5 |
| 7 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Sikament 2004 NS (Litre) | 180.0 360.0 720.0 472.0 706.0 1.80 |

*  Admixture used percentage by weight of cement.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./76(i)/2021-22/ 	                                                         Dated: 21.06.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade with admixture

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA—20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 87.05 | 100.0 | - |
|  | 10 mm | 0.20 | 85.85 | - |
|  | 4.75 mm | 0.05 | 1.20 | 1.6 |
|  | 2.36 mm | - | 0.35 | 4.2 |
|  | 1.18 mm | - | 0.10 | 14.8 |
|  | 600 micron | - | - | 44.6 |
|  | 300 micron | - | - | 83.8 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.127 | 6.125 | 2.480 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.53 | 1.75 |
| 4. | Specific Gravity | 2.83 | 2.81 | 2.68 |
| 5. | Water absorption | 0.25% | 0.50 % | 1.55% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./76(i)/2021-22/ 	                                                         Dated: 21.06.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	DGM (FQA)

Kanti Bijlee Utpadan Nigam Limited,

Muzafrpur Thermal Power Station

P.O. Kanti Thermal, Dist. Muzaffarpur – 843130, Bihar

(M/s Kumar Piyush Construction Private Limited)

Ref.		:    	KBUNL/FQA/KPCPL/5500037640/2021  Dated : 06-04-2021

Details of Sample:	Cement: Infracem Cement, PPC IS : 1489, W-04,  Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade with

admixture

______________________________________________________________________________

Sub.: Concrete Mix design.

Work: Construction of Pedestal and Culverts and Thrust block for Ash Slurry Disposal Package

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 36 % 1.5 mm Expansion 115 Minutes 345 Minutes 25.3 N/mm2 34.9 N/mm2 44.8 N/mm2 1.30 2.91 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./208/2021-22/ 	                                                  Dated: 07.06.2021

#### TEST REPORTS

Tested for 	:      	Head-Buxar Project Site

Larsen and Toubro  Limited

2x660 MW, Buxar Thermal Power Project

Ref.		:    	 L & T POWER/SJVN/BUXAR/EPC/19-20/07  Dated : 28-12-2020

L & T POWER/SJVN/BUXAR/EPC/19-20/11A  Dated : 22-02-2021

Details of Sample :      	Concrete Mix Design.

Test Required       	 (i) M-20 grade with admixture with OPC Cement

(ii) M-25 grade with admixture with OPC Cement

(iii) M-30 grade with admixture with OPC Cement

(iv) M-35 grade with admixture with OPC Cement

(v) M-40 grade with admixture with OPC Cement

__________________________________________________________________________

Sub: Design mix.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes  (i) 20 mm,   (ii) 10 mm.

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech Cement, OPC 43-grade, IS:269, W-03 M-01, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2019.

5.   Saturated surface dry aggregates were used as per IS: 10262-2019.

6.   Degree of control - Good.

7. Admixture used Conplast SP 430 ES2.                                .

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./208/2021-22/ 	                                                  Dated: 07.06.2021

#### (Page – 2(i)

#### TEST REPORTS

Recommended Mix Proportion for  (i) M-20 grade with admixture with OPC Cement (ii) M-25 grade with

admixture with  OPC Cement  (iii)  M-30 grade with admixture with OPC Cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-20 grade | 0.55 : 1 : 2.39 : 1.43 : 2.14 : 0.6* 0.792 : 1 : 1.92 : 1.37 : 1.88 : 0.6* | By weight By volume |
| 2. | M-25 grade | 0.48 : 1 : 2.00 : 1.25 : 1.87 : 0.6* 0.691 : 1 : 1.61 : 1.20 : 1.64 : 0.6* | By weight By volume |
| 2. | M-30 grade | 0.45 : 1 : 1.81 : 1.17 : 1.76 : 0.6* 0.648 : 1 : 1.46 : 1.12 : 1.55 : 0.6* | By weight By volume |

| Sl. No. | Test | M-20 grade | M-25 grade | M-30 grade |
| --- | --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 27.6 | 33.7 | 39.9 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 | 150-200 |
| 3 | Slump range (after one hour ) (mm) | 100-150 | 100-150 | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 215.0 | 210.0 | 200.0 |
| 5 | Slump Achieved in the Laboratory(after one hour) (mm) | 120.0 | 115.0 | 110.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 24.5 32.5 | 31.0 36.0 | 37.0 42.0 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Conplast SP 430 ES2 (Litre) | 176.0 320.0 765.0 458.0 685.0 1.92 | 175.2 365.0 730.0 456.0 683.0 2.19 | 175.5 390.0 706.0 456.0 686.0 2.34 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./208/2021-22/ 	                                                  Dated: 07.06.2021

#### (Page – 2(ii)

#### TEST REPORTS

Recommended Mix Proportion for  (iv)  M-35 grade  with admixture with  OPC Cement   (v) M-40 grade with

admixture with OPC Cement

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) : Admixture | Remark |
| --- | --- | --- | --- |
| 1. | M-35 grade | 0.40 : 1 : 1.52 : 1.03 : 1.55 : 0.6* 0.576 : 1 : 1.22 : 0.99 : 1.36 : 0.6* | By weight By volume |
| 2. | M-40 grade | 0.375 : 1 : 1.36 : 0.97 : 1.45 : 0.6* 0.540 : 1 : 1.09 : 1.93 : 1.27 : 0.6* | By weight By volume |

| Sl. No. | Test | M-35 grade | M-40 grade |
| --- | --- | --- | --- |
| 1 | Target Strength (N/mm2) | 45.4 | 50.9 |
| 2 | Slump range (initial) (mm) | 150-200 | 150-200 |
| 3 | Slump range (after one hour ) (mm) | 100-150 | 100-150 |
| 4 | Slump Achieved in the Laboratory (initial) (mm) | 190.0 | 195.0 |
| 5 | Slump Achieved in the Laboratory(after one hour) (mm) | 100.0 | 110.0 |
| 6 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 41.0 46.0 | 47.5 54.5 |
| 5. | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) vi) ) Conplast SP 430 ES2 (Litre) | 176.0 440.0 669.0 453.0 682.0 2.64 | 176.25 470.0 639.0 456.0 682.0 2.82 |

*  Admixture used percentage by weight of cement.

Senior Technical 	             Teacher In-charge                     Professor-In-Charge

Superintendent                                        Testing                	     Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./208/2021-22/ 	                                                  Dated: 07.06.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design  (i) M-20 grade with admixture

with OPC Cement (ii) M-25 grade with admixture with OPC Cement (iii) M-30 grade with admixture with OPC

Cement (iv) M-35 grade with admixture with OPC Cement  (v) M-40 grade with admixture with OPC Cement

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA– 20 mm (Source: REWA, M.P.) | CA—10 mm (Source: REWA, M.P.) | FA (sand) (Source: Sone River ) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 85.55 | 100.0 | - |
|  | 10 mm | 0.40 | 93.6 | - |
|  | 4.75 mm | 0.05 | 1.40 | 2.4 |
|  | 2.36 mm | - | 0.40 | 8.4 |
|  | 1.18 mm | - | 0.05 | 28.4 |
|  | 600 micron | - | - | 60.4 |
|  | 300 micron | - | - | 91.0 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.140 | 6.046 | 2.896 |
| 3. | Bulk Density (Kg./Litre) | 1.64 | 1.50 | 1.79 |
| 4. | Specific Gravity | 2.69 | 2.67 | 2.69 |
| 5. | Water absorption | 0.60 % | 0.77 % | 1.16% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./208/2021-22/ 	                                                  Dated: 07.06.2021

#### TEST REPORTS

Tested for 	:      	Head-Buxar Project Site

Larsen and Toubro  Limited

2x660 MW, Buxar Thermal Power Project

Ref.		:    	 L & T POWER/SJVN/BUXAR/EPC/19-20/07  Dated : 28-12-2020

L & T POWER/SJVN/BUXAR/EPC/19-20/11A  Dated : 22-02-2021

Details of Sample:	Cement: Ultra Tech Cement, OPC 43-grade, IS : 269 ,W-03, M-01, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design  (i) M-20 grade with admixture with  OPC Cement (ii) M-25 grade with admixture with OPC Cement (iii) M-30 grade with admixture with OPC Cement (iv) M-35 grade with admixture with OPC Cement  (v) M-40 grade with admixture with OPC Cement

______________________________________________________________________________

Sub: Design mix.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 3 % 30 % 1.0 mm Expansion 110 Minutes 320 Minutes 32.4 N/mm2 40.9 N/mm2 47.6 N/mm2 1.44 3.10 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./404/2021-22/ 	                                                     Dated: 04.06.2021

#### TEST REPORTS

Tested for 	:      	SO (Civil)

Office of the Staff Officer (Civil)

Northern Coalfields Limited

Khadia Project

( M/s Pollucon Technologies Limited)

Ref.		:    		NCL/KHD/Civil 2021/921 dated 18.02.2021

Tax Invoice No.:	000938 IIT(BHU) dated 26.03.2021

Details of Sample :      Concrete Mix Design.

Test Required       	 M-30 grade ___________________________________________________________________________

Sub :  Design Mix M30 concrete for the work “Planning, design, Engineering, Construction, Fabrication Erection, Supply installation, testing, Trial Run and commissioning of sewage Treatment Plant (1.5 MLD) consisting of all civil, structural, electrical and mechanical works and all other accessories and facilities required to  make it complete in all rsp4ct o turnkey basis including operation and maintenance of plant for four years.

Name of Contractor: M/s Pollucon Technologies Limited

Ref.: LOA No. & date : GM (C) / SGR/LOA-102/ETN-90/19/2591 dated 25.03.2020

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes   (i) 20 mm. (ii) 10 mm

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Ultra Tech  Cement, PPC IS : 1489,  Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2009.

5.   Saturated surface dry aggregates were used as per IS: 10262-2009.

6.   Degree of control - Good.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./404/2021-22/ 	                                                     Dated: 04.06.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-30 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M-30 Grade | 0.45 : 1 : 1.56 : 0.96 : 1.79 0.590 : 1 : 1.17 : 0.81 : 1.35 | By weight By volume |

| Sl. No. | Test | M-30 grade |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 39.9 |
| 2 | Slump range (mm) | 50-100 |
| 3 | Slump Achieved in the Laboratory (mm) | 80.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 28.0 44.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 191.25 425.0 663.0 408.0 761.0 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./404/2021-22/ 	                                                     Dated: 04.06.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-30 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA—20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 86.75 | 100.0 | - |
|  | 10 mm | 0.15 | 85.25 | - |
|  | 4.75 mm | 0.05 | 5.20 | 1.8 |
|  | 2.36 mm | - | 0.45 | 6.4 |
|  | 1.18 mm | - | 0.10 | 18.4 |
|  | 600 micron | - | - | 53.2 |
|  | 300 micron | - | - | 92.4 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.131 | 6.090 | 2.712 |
| 3. | Bulk Density (Kg./Litre) | 1.74 | 1.56 | 1.75 |
| 4. | Specific Gravity | 2.91 | 2.88 | 2.68 |
| 5. | Water absorption | 0.10% | 0.15 % | 1.77% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./404/2021-22/ 	                                                     Dated: 04.06.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	SO (Civil)

Office of the Staff Officer (Civil)

Northern Coalfields Limited

Khadia Project

( M/s Pollucon Technologies Limited)

Ref.		:    		NCL/KHD/Civil 2021/921 dated 18.02.2021

Details of Sample:	Cement: Ultra Tech  Cement, PPC IS : 1489,  Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade

______________________________________________________________________________

Sub :  Design Mix M30 concrete for the work “Planning, design, Engineering, Construction, Fabrication Erection, Supply installation, testing, Trial Run and commissioning of sewage Treatment Plant (1.5 MLD) consisting of all civil, structural, electrical and mechanical works and all other accessories and facilities required to  make it complete in all rsp4ct o turnkey basis including operation and maintenance of plant for four years.

Name of Contractor: M/s Pollucon Technologies Limited

Ref.: LOA No. & date : GM (C) / SGR/LOA-102/ETN-90/19/2591 dated 25.03.2020

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 7 % 37 % 1.5 mm Expansion 140 Minutes 365 Minutes 22.8 N/mm2 30.7 N/mm2 41.9 N/mm2 1.31 2.91 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./379/2021-22/ 	                                                  Dated: 04.06.2021

#### TEST REPORTS

Tested for 	:      	M/s SKV Enterprises

AT+PO – Duddhi,

Distt.-Sonebhadra, U.P. – 231208

Ref.		:    		Nil   Dated : 05-02-2021

Tax Invoice No. : 	 000903  IIT (BHU) dated 26.03.2021

Details of Sample :      Concrete Mix Design.

Test Required       	 M-25 grade

___________________________________________________________________________

Sub.: Concrete Mix Design M-25 grade for construction of Tribal School at Sewakunj Ashram Chopaki NTPC Rihand.

#### REPORT

1.	The above mix-designs are done with reference to the letter mentioned above.

2.	Coarse aggregate was supplied in two sizes   (i) 20 mm. (ii) 10 mm

The properties including Fine Aggregate (sand) are given in Appendix –‘A’.

- The Cement supplied was Double Bull Cement, PPC IS : 1489, W-14, M-04, Y-2021

The properties are given in Appendix – ‘B’

4.  Trial Mixes were cast and tested as per IS: 10262-2009.

5.   Saturated surface dry aggregates were used as per IS: 10262-2009.

6.   Degree of control - Good.

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

Cont’d …. 2

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./379/2021-22/ 	                                                  Dated: 04.06.2021

#### (Page – 2)

#### TEST REPORTS

Recommended Mix Proportion for M-25 grade

| Grade of Concrete |  | Water : Cement : Sand : CA-(10) : CA- (20) | Remark |
| --- | --- | --- | --- |
| 1. | M-25 Grade | 0.50 : 1 : 1.81 : 1.23 : 1.52 0.65 : 1 : 1.33 : 1.03 : 1.14 | By weight By volume |

| Sl. No. | Test | M-25 grade |
| --- | --- | --- |
| 1 | Target Strength (N/mm2) | 33.7 |
| 2 | Slump range (mm) | 100-150 |
| 3 | Slump Achieved in the Laboratory (mm) | 130.0 |
| 4 | i) 7 days Cube Compressive Strength achieved in the Laboratory (N/mm2) ii) 28 days Cube Compressive Strength achieved in the Laboratory (N/mm2) | 24.0 34.5 |
| 5 | Consumption of material per cubic meter of Concrete. i) Water (Liter) ii) Cement (Kg) iii) Sand (Kg) iv) C.A. (10 mm) (Kg) v) C.A.(20 mm) (Kg.) | 200.0 400.0 724.0 492.0 608.0 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./379/2021-22/ 	                                                  Dated: 04.06.2021

Appendix – ‘A’

#### TEST REPORTS

Properties of Coarse Aggregate and Fine Aggregate for Concrete Mix-Design M-25 grade

| S.No | Test | R E S U L T |  |  |
| --- | --- | --- | --- | --- |
|  |  | CA—20 mm | CA—10 mm | FA (sand) |
| 1. | Sieve Analysis I.S. Sieve No. | % Passing | % Passing | Cumulative % Retained |
|  | 40 mm | 100.0 | 100.0 | - |
|  | 20 mm | 87.75 | 100.0 | - |
|  | 10 mm | 0.25 | 86.4 | - |
|  | 4.75 mm | 0.05 | 5.95 | 5.8 |
|  | 2.36 mm | - | 0.40 | 21.8 |
|  | 1.18 mm | - | 0.10 | 52.0 |
|  | 600 micron | - | - | 82.2 |
|  | 300 micron | - | - | 96.6 |
|  | 150 micron | - | - | 99.0 |
| 2. | Fineness Modulus | 7.120 | 6.072 | 3.574 |
| 3. | Bulk Density (Kg./Litre) | 1.73 | 1.56 | 1.77 |
| 4. | Specific Gravity | 2.86 | 2.84 | 2.72 |
| 5. | Water absorption | 0.35% | 0.46 % | 0.87% |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory

STRUCTURAL ENGINEERING LABORATORY

Department of Civil Engineering

##### Indian Institute of Technology

##### (Banaras Hindu University)

## Varanasi – 221005

### Websites : www.Itbhu.ac.in

Ref. No. Civil/Struct./379/2021-22/ 	                                                  Dated: 04.06.2021

### Appendix ‘B’

#### TEST REPORTS

Tested for 	:      	M/s SKV Enterprises

AT+PO – Duddhi,

Distt.-Sonebhadra, U.P. – 231208

Ref.		:    		Nil   Dated : 05-02-2021

Details of Sample:	Cement: Double Bull Cement, PPC IS : 1489, W-14, M-04, Y-2021

Test Required  :     	Physical Properties of Cement for Concrete Mix-Design M-25 grade

______________________________________________________________________________

Sub.: Concrete Mix Design M-25 grade for construction of Tribal School at Sewakunj Ashram Chopaki NTPC Rihand.

| Sl.No. | Tests | Results |
| --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. | Fineness by dry Sieving (90 micron) Consistency of Standard Cement Paste Soundness Test Setting Time: a) Initial Setting Time b) Final Setting Time Compressive Strength Cement, Standard Sand Ratio = 1:3 a) 3 days Strength b) 7 days Strength c) 28 days Strength Unit Weight in gm/cc Specific Gravity | 6 % 33 % 1.5 mm Expansion 160 Minutes 450 Minutes 21.1 N/mm2 25.6 N/mm2 40.8 N/mm2 1.30 2.90 |

Senior Technical 	 	    Teacher In-charge                Professor-In-Charge

Superintendent                                         Testing                	               Laboratory